3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210031

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1477 | rev | - | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to ENDC Integrity protection test cases 7.1.3.2.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| s |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-12-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The function f_NR_Security_Get is trying to read from the variable vc_NR_Global which is not yet inicialized.This cause a run-time error. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Move calling the function f_NR_Security_Get after f_ENDC_NR_Init where vc_NR_Global is already initialized |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testcase will fail a conformant UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.2.1.ENDC, 7.1.3.2.2.ENDC, 7.1.3.2.3.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | fl_ENDC_PDCP_Integrity |
| --- | --- |
| Reason for change | The function f_NR_Security_Get is reading from variable vc_NR_Global which is not yet inicialized.This cause a run-time error. |
| Summary of change | Move calling the function f_NR_Security_Get after f_ENDC_NR_Init where vc_NR_Global is already initialized |
| TTCN module | PDCP_ENDC_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_ENDC_PDCP_Integrity(IntegrityProtAlgorithm p_IntegrityProAlgorithm) runs on ENDC_NR_PTC { var ENDC_SRB_DRB_Configuration_Type v_SecondaryConfig:= NR_PDCP_SRB1_2; //var template (value) FreqBandList v_MRDC_FreqList; // Anritsu TTCN WA for FreqBandList var template (value) FreqBandListWrapper v_MRDC_FreqList; // Anritsu TTCN WA for FreqBandList var octetstring v_OctetData; var integer v_EUTRABandInd; var integer v_NRBandInd; var template (value) NR_RadioBearerList_Type v_RadioBearerList; var template (value) CellGroupConfig v_SCGConfig; var NR_SecurityParams_Type v_Security_Params := f_NR_Security_Get(); var IntegrityProtAlgorithm v_IntegrityAlg:= v_Security_Params.AS_Integrity.Algorithm; var boolean v_DontWaitForComplete:=false; if (pc_srb3){ v_SecondaryConfig := NR_PDCP_SRB1_2_SRB3; } f_ENDC_NR_Init(); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); f_NR_AS_IntegrityAlgorithm_Set(p_IntegrityProAlgorithm); // Get NR capabilities sent during EUTRA preamble // @sic R5-195339 sic@ f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_SCG); //Configure DRB2 as normal f_NR_RRC_PSCellChange_SRB_NR_PDCP(nr_Cell1, v_SecondaryConfig); f_NR_TestBody_Set(true); f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); … }

After Change:

function fl_ENDC_PDCP_Integrity(IntegrityProtAlgorithm p_IntegrityProAlgorithm) runs on ENDC_NR_PTC { var ENDC_SRB_DRB_Configuration_Type v_SecondaryConfig:= NR_PDCP_SRB1_2; //var template (value) FreqBandList v_MRDC_FreqList; // Anritsu TTCN WA for FreqBandList var template (value) FreqBandListWrapper v_MRDC_FreqList; // Anritsu TTCN WA for FreqBandList var octetstring v_OctetData; var integer v_EUTRABandInd; var integer v_NRBandInd; var template (value) NR_RadioBearerList_Type v_RadioBearerList; var template (value) CellGroupConfig v_SCGConfig; var NR_SecurityParams_Type v_Security_Params; var IntegrityProtAlgorithm v_IntegrityAlg; var boolean v_DontWaitForComplete:=false; if (pc_srb3){ v_SecondaryConfig := NR_PDCP_SRB1_2_SRB3; } f_ENDC_NR_Init(); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); v_Security_Params := f_NR_Security_Get(); var IntegrityProtAlgorithm v_IntegrityAlg:= v_Security_Params.AS_Integrity.Algorithm; f_NR_AS_IntegrityAlgorithm_Set(p_IntegrityProAlgorithm); // Get NR capabilities sent during EUTRA preamble // @sic R5-195339 sic@ f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_SCG); //Configure DRB2 as normal f_NR_RRC_PSCellChange_SRB_NR_PDCP(nr_Cell1, v_SecondaryConfig); f_NR_TestBody_Set(true); f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); … }