3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210058

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1498 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC RRC test case 8.1.4.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Security parameters will be refreshed during the execution of f_NR5GC_MappedContext_IntraN1TransparentContainer(), so the parameters should be read after this function has completed |  |  |  |  |  |  |  |  |
| Summary of change: |  | Get security parameters after the call to f_NR5GC_MappedContext_IntraN1TransparentContainer() |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail these test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_BuildIntraN1TransparentContainer() |
| --- | --- |
| Reason for change | Security parameters will be refreshed during the execution of f_NR5GC_MappedContext_IntraN1TransparentContainer(), so the parameters should be read after this function has completed |
| Summary of change | Get security parameters after the call to f_NR5GC_MappedContext_IntraN1TransparentContainer() |
| TTCN module | RRC_IntraNR_Handover_NR5GC |
| MCC160 Comment |  |

Before Change

function fl_BuildIntraN1TransparentContainer () runs on NR5GC_PTC return octetstring { //@sic R5-202641 sic@ var NG_NAS_SecurityParams_Type v_SecurityParams := f_NR5GC_Security_Get(); var NasCountInfo_Type v_NASCountInfo := f_SS_NG_NAS_CountGet(NASCTRL); var O4_Type v_NASCountDL := f_NasSecurity_IncrementCOUNT(v_NASCountInfo.DL); // @sic R5s201076 sic@ var template (value) IntraN1TransparentContainer v_NASContainer; var MessageAuthenticationCode v_CalculatedMac; var bitstring v_NASContainerString; // Update Security parameters @sic R5s201076 sic@ f_NR5GC_MappedContext_IntraN1TransparentContainer(v_NASCountDL); v_NASContainer := cs_IntraN1TransparentContainer (omit, // set to omit in order to calculate its own value v_SecurityParams.NAS_Ciphering.Algorithm, v_SecurityParams.NAS_Integrity.Algorithm, '1'B, '0'B, v_SecurityParams.KSIamf, substr(v_NASCountDL, 3, 1)); v_NASContainerString := encvalue(valueof(v_NASContainer)); v_CalculatedMac := fx_NG_NasIntegrityAlgorithm(bit2oct(v_NASContainerString), v_SecurityParams.NAS_Integrity.Algorithm, v_SecurityParams.NAS_Integrity.K_NAS, 'FFFFFFFF'O, // according to 33.501 cl. 6.9.2.3.3 '00001'B, tsc_DirectionDL); v_NASContainer.mac := v_CalculatedMac; v_NASContainerString := encvalue(v_NASContainer); return bit2oct(v_NASContainerString); }

After Change

function fl_BuildIntraN1TransparentContainer () runs on NR5GC_PTC return octetstring { //@sic R5-202641 sic@ var NG_NAS_SecurityParams_Type v_SecurityParams; var NasCountInfo_Type v_NASCountInfo := f_SS_NG_NAS_CountGet(NASCTRL); var O4_Type v_NASCountDL := f_NasSecurity_IncrementCOUNT(v_NASCountInfo.DL); // @sic R5s201076 sic@ var template (value) IntraN1TransparentContainer v_NASContainer; var MessageAuthenticationCode v_CalculatedMac; var bitstring v_NASContainerString; // Update Security parameters @sic R5s201076 sic@ f_NR5GC_MappedContext_IntraN1TransparentContainer(v_NASCountDL); v_SecurityParams := f_NR5GC_Security_Get(); v_NASContainer := cs_IntraN1TransparentContainer (omit, // set to omit in order to calculate its own value v_SecurityParams.NAS_Ciphering.Algorithm, v_SecurityParams.NAS_Integrity.Algorithm, '1'B, '0'B, v_SecurityParams.KSIamf, substr(v_NASCountDL, 3, 1)); v_NASContainerString := encvalue(valueof(v_NASContainer)); v_CalculatedMac := fx_NG_NasIntegrityAlgorithm(bit2oct(v_NASContainerString), v_SecurityParams.NAS_Integrity.Algorithm, v_SecurityParams.NAS_Integrity.K_NAS, 'FFFFFFFF'O, // according to 33.501 cl. 6.9.2.3.3 '00001'B, tsc_DirectionDL); v_NASContainer.mac := v_CalculatedMac; v_NASContainerString := encvalue(v_NASContainer); return bit2oct(v_NASContainerString); }