3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210077

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1511 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC IRAT test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | From 25.401: To allow the network to release the N1 NAS signalling connection, the UE: a) shall start the timer T3540 if the UE receives any of the 5GMM cause values #7, #11, #12, #13, #15, #27, #31, #62, #72, #73, #74, #75, #76; b) shall start the timer T3540 for a UE in 3GPP access if: 1) the UE receives a REGISTRATION ACCEPT message which does not include a Pending NSSAI IE or UE radio capability ID deletion indication IE; 2) the UE has set the Follow-on request indicator to "No follow-on request pending" in the REGISTRATION REQUEST message; Upon expiry of T3540, - in cases a), b), f) and g), the UE shall locally release the established N1 NAS signalling connection; - in cases c) and d) the UE shall locally release the established N1 NAS signalling connection and the UE shall initiate the registration procedure as described in subclause 5.5.1.3.5 or 5.6.1.5; or - in case e), the UE shall locally release the established N1 NAS signalling connection and perform a new registration procedure as specified in subclause 5.5.1.3.2. This means if the UE has sent REGISTRATION REQUEST with “no follow-onm request pending” and no further downlink signaling is received for 10s (=T3540) the UE will locally release the NAS signaling connection. In f_NR5GC_IRAT_S1ToN1_Steps14, TTCN will wait always 10s for a PDU SESSION MODFICATION REQUEST (regardless of follow-on request setting). A UE which is not reaqesting PDU session modification will locally release the NAS connection at a time which can clash with the expiry of this timer. This issue affects e.g. test cases 6.2.3.5 and 8.1.1.3.4 where TTCN is going on to send RRCReconfiguration or RRCRelease (with specific content) following this 10s delay. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reduce t_Wait to 8s, so the TTCN can send further signalling before the UE has locally released its NAS signalling connection due to T3540 expiry. Note : An associated prose CR on 38.508-1 will be raised at RAN5#90 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail some IRAT test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.5.NR5GC, 8.1.1.3.4.NR5GC and others |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.508-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_NR5GC_IRAT_S1ToN1_Steps14() |
| --- | --- |
| Reason for change | From 25.401: To allow the network to release the N1 NAS signalling connection, the UE: a) shall start the timer T3540 if the UE receives any of the 5GMM cause values #7, #11, #12, #13, #15, #27, #31, #62, #72, #73, #74, #75, #76; b) shall start the timer T3540 for a UE in 3GPP access if: 1) the UE receives a REGISTRATION ACCEPT message which does not include a Pending NSSAI IE or UE radio capability ID deletion indication IE; 2) the UE has set the Follow-on request indicator to "No follow-on request pending" in the REGISTRATION REQUEST message; Upon expiry of T3540, - in cases a), b), f) and g), the UE shall locally release the established N1 NAS signalling connection; - in cases c) and d) the UE shall locally release the established N1 NAS signalling connection and the UE shall initiate the registration procedure as described in subclause 5.5.1.3.5 or 5.6.1.5; or - in case e), the UE shall locally release the established N1 NAS signalling connection and perform a new registration procedure as specified in subclause 5.5.1.3.2. This means if the UE has sent REGISTRATION REQUEST with “no follow-on request pending” and no further downlink signaling is received for 10s (=T3540) the UE will locally release the NAS signaling connection. In f_NR5GC_IRAT_S1ToN1_Steps14(), TTCN will wait always 10s for a PDU SESSION MODFICATION REQUEST (regardless of follow-on request setting). A UE which is not reauesting PDU session modification will locally release the NAS connection at a time which can clash with the expiry of this timer. This issue affects e.g. test cases 6.2.3.5 and 8.1.1.3.4 where TTCN is going on to send RRCReconfiguration or RRCRelease (with specific content) following this 10s delay. |
| Summary of change | Reduce t_Wait to 8s, so the TTCN can send further signalling before the UE has locally released its NAS signalling connection due to T3540 expiry. Note : An associated prose CR on 38.508-1 will be raised at RAN5#90 |
| TTCN module |  |
| MCC160 Comment |  |

Before Change

function f_NR5GC_IRAT_S1ToN1_Steps14(NR_CellId_Type p_NR_CellId, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, boolean p_FirstTimeS1ToN1 := true, InterworkWithoutN26 p_IwkN26 := NOT_SUPPORTED) runs on NR5GC_PTC return NR_RadioBearerId_Type { var EUTRA_NR_PduSessionInfoList_Type v_PduSessionInfoList; //@sic R5s201060 sic@ var NR_SRB_COMMON_IND v_ReceivedAsp; var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB1; var boolean v_FailWhenK_LessN := false; var template NG_Request_Type v_RequestType := cr_NG_Request_Type('001'B); timer t_Wait := 10.0; // @sic R5-204399 sic@ ….. }

After Change

function f_NR5GC_IRAT_S1ToN1_Steps14(NR_CellId_Type p_NR_CellId, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, boolean p_FirstTimeS1ToN1 := true, InterworkWithoutN26 p_IwkN26 := NOT_SUPPORTED) runs on NR5GC_PTC return NR_RadioBearerId_Type { var EUTRA_NR_PduSessionInfoList_Type v_PduSessionInfoList; //@sic R5s201060 sic@ var NR_SRB_COMMON_IND v_ReceivedAsp; var NR_RadioBearerId_Type v_SRB := tsc_NR_RbId_SRB1; var boolean v_FailWhenK_LessN := false; var template NG_Request_Type v_RequestType := cr_NG_Request_Type('001'B); timer t_Wait := 8.0; // @sic R5-204399 sic@ ….. }