3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210083

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1517 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for ENDC RRC test case 8.2.2.3.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - At step 3 of the test case, SS shall send a reconfiguration message on SRB3. SRB3 is configured using KgNB as per the reconfiguration message at step 1 but the keys are not yet applied on the SS. It is required to send KgNB keys from EUTRA PTC to NR PTC and apply the keys for SRB3. - Template cs_NR_RlcBearerConfig_AM is used to configure SS side SRB. This will end up using template to configure IE “sn_FieldLength” with value snSize18. For Signalling bearers sn_FieldLength shall be set as snSize12. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Send KgNB information to NR PTC before step 3 and configure security for SRB3 - New template for SRB3 RlcBearer configuration |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.3.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1 Change 1	3

2.2 Change 2	5

2.3 Change 3	8

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## 2.1 Change 1

| Function name | fl_TC_8_2_2_3_1_ENDC_EUTRA_TestBody |
| --- | --- |
| Reason for change | At step 3 of the test case, SS shall send a reconfiguration message on SRB3. SRB3 is configured using KgNB as per the reconfiguration message at step 1 but the keys are not yet applied on the SS. It is required to send KgNB keys from EUTRA PTC to NR PTC and apply the keys for SRB3. |
| Summary of change | At step3, function f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys() is called with SKCounter value 1 to send security keys for SRB3 to NR PTC |
| TTCN module | RRCReconfig_ENDC_EUTRA.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_8_2_2_3_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC

{// @sic R5-202647, R5-206340, R5s201348 sic@

var template (omit) octetstring v_NR_SCGConfig := omit;

var template (omit) octetstring v_NR_RBConfig := omit;

var template (value) RRCConnectionReconfiguration_v890_IEs v_RRCConnectionReconfiguration_v890_IEs;

var EUTRA38_SecurityParamsExtd_Type v_Security_Params := f_EUTRA38_Security_Get();

// @siclog "Steps 1-2" siclog@

f_EUTRA38_ReconfigureSRBs_NR_PDCPSplit(eutra_Cell1, true, (v_Security_Params.SK_Counter + 1), false);

// @siclog "Step 3" siclog@

// The SS transmits on SRB3/SCG an NR RRCReconfiguration message to reconfigure SCG DRB.

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger);

f_EUTRA_NR_WaitForCoOrd_Trigger(NR);

// @siclog "Step 5" siclog@

// The SS transmits over split SRB1 SCG path/NR Cell 1 an RRCConnectionReconfiguration message to release SRB3.

v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig");

v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB");

v_RRCConnectionReconfiguration_v890_IEs := f_EUTRA38_508_BuildRRCConnReconfig_NR_Config_Setup (v_NR_SCGConfig, v_NR_RBConfig);

SRB.send(cas_SRB1_RrcNasPduList_REQ(eutra_Cell1,

cs_TimingInfo_Now,

cs_RRCConnectionReconfiguration_Common(tsc_RRC_TI_Def,-,-,-,-,v_RRCConnectionReconfiguration_v890_IEs),

omit));

// @siclog "Step 6" siclog@

SRB.receive(car_SRB_RrcPdu_IND_Common(eutra_Cell1,tsc_SRB1, cr_RRCConnectionReconfigurationComplete_r15, -, crs_RlcBearerRouting_Eutra(eutra_Cell1)));

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 6");

// @siclog "Step 7" siclog@

// The SS transmits an RRCConnectionReconfiguration message to release SRB1 over SCG path/NR Cell 1

// Keep split SRB configured at SS side to ensure UE is not using the SCG path

v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig");

v_RRCConnectionReconfiguration_v890_IEs := f_EUTRA38_508_BuildRRCConnReconfig_NR_Config_Setup (v_NR_SCGConfig);

SRB.send(cas_SRB1_RrcNasPduList_REQ(eutra_Cell1,

cs_TimingInfo_Now,

cs_RRCConnectionReconfiguration_Common(tsc_RRC_TI_Def,-,-,-,-,v_RRCConnectionReconfiguration_v890_IEs),

omit));

// @siclog "Step 8" siclog@

// Check: Does the UE send RRCConnectionReconfigurationComplete containing NR RRCReconfigurationComplete message on SRB1 over the MCG path?

SRB.receive(car_SRB_RrcPdu_IND_Common(eutra_Cell1, tsc_SRB1, cr_RRCConnectionReconfigurationComplete_r15, -, crs_RlcBearerRouting_Eutra(eutra_Cell1)));

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 8");

}

After Change

function fl_TC_8_2_2_3_1_ENDC_EUTRA_TestBody() runs on EUTRA_5GS_PTC

{// @sic R5-202647, R5-206340, R5s201348 sic@

var template (omit) octetstring v_NR_SCGConfig := omit;

var template (omit) octetstring v_NR_RBConfig := omit;

var template (value) RRCConnectionReconfiguration_v890_IEs v_RRCConnectionReconfiguration_v890_IEs;

var EUTRA38_SecurityParamsExtd_Type v_Security_Params := f_EUTRA38_Security_Get();

// @siclog "Steps 1-2" siclog@

f_EUTRA38_ReconfigureSRBs_NR_PDCPSplit(eutra_Cell1, true, (v_Security_Params.SK_Counter + 1), false);

// @siclog "Step 3" siclog@

// The SS transmits on SRB3/SCG an NR RRCReconfiguration message to reconfigure SCG DRB.

f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger);

// Initiate SgNB Keys

f_EUTRA38_ENDC_InitiateAndSendS_gNBKeys(NR,1);

f_EUTRA_NR_WaitForCoOrd_Trigger(NR);

// @siclog "Step 5" siclog@

// The SS transmits over split SRB1 SCG path/NR Cell 1 an RRCConnectionReconfiguration message to release SRB3.

v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig");

v_NR_RBConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "RB");

v_RRCConnectionReconfiguration_v890_IEs := f_EUTRA38_508_BuildRRCConnReconfig_NR_Config_Setup (v_NR_SCGConfig, v_NR_RBConfig);

SRB.send(cas_SRB1_RrcNasPduList_REQ(eutra_Cell1,

cs_TimingInfo_Now,

cs_RRCConnectionReconfiguration_Common(tsc_RRC_TI_Def,-,-,-,-,v_RRCConnectionReconfiguration_v890_IEs),

omit));

// @siclog "Step 6" siclog@

SRB.receive(car_SRB_RrcPdu_IND_Common(eutra_Cell1,tsc_SRB1, cr_RRCConnectionReconfigurationComplete_r15, -, crs_RlcBearerRouting_Eutra(eutra_Cell1)));

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 6");

// @siclog "Step 7" siclog@

// The SS transmits an RRCConnectionReconfiguration message to release SRB1 over SCG path/NR Cell 1

// Keep split SRB configured at SS side to ensure UE is not using the SCG path

v_NR_SCGConfig := f_EUTRA_NR_WaitForCoOrd_Data(NR, "Reconfig");

v_RRCConnectionReconfiguration_v890_IEs := f_EUTRA38_508_BuildRRCConnReconfig_NR_Config_Setup (v_NR_SCGConfig);

SRB.send(cas_SRB1_RrcNasPduList_REQ(eutra_Cell1,

cs_TimingInfo_Now,

cs_RRCConnectionReconfiguration_Common(tsc_RRC_TI_Def,-,-,-,-,v_RRCConnectionReconfiguration_v890_IEs),

omit));

// @siclog "Step 8" siclog@

// Check: Does the UE send RRCConnectionReconfigurationComplete containing NR RRCReconfigurationComplete message on SRB1 over the MCG path?

SRB.receive(car_SRB_RrcPdu_IND_Common(eutra_Cell1, tsc_SRB1, cr_RRCConnectionReconfigurationComplete_r15, -, crs_RlcBearerRouting_Eutra(eutra_Cell1)));

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 8");

}

## 2.2 Change 2

| Function name | fl_TC_8_2_2_3_1_ENDC_NR_TestBody |
| --- | --- |
| Reason for change | - Template cs_NR_RlcBearerConfig_AM is used to configure SS side SRB. This will end up using template to configure IE “sn_FieldLength” with value snSize18. For Signalling bearers sn_FieldLength shall be set as snSize12. - At step 3 of the test case, SS shall send a reconfiguration message on SRB3. SRB3 is configured using KgNB as per the reconfiguration message at step 1 but the keys are not yet applied on the SS. It is required to send KgNB keys from EUTRA PTC to NR PTC and apply the keys for SRB3. |
| Summary of change | - New template cs_NR_RlcBearerConfig_AM_SRB is used to configure SS at step #1 - At step 3, function f_ENDC_GetKeysActivateSecurity() Is called to wait for KgNB Keys from EUTRA and apply the Keys on SRB3. |
| TTCN module | RRCReconfig_ENDC_NR.ttcn |
| MCC160 Comment |  |

Before Change

function fl_TC_8_2_2_3_1_ENDC_NR_TestBody() runs on ENDC_NR_PTC

{// @sic R5-202647, R5-206340, R5s201348 sic@

var template (value) CellGroupConfig v_SCGConfig;

var template (value) RadioBearerConfig v_RadioBearerConfig1;

var template (value) RadioBearerConfig v_RadioBearerConfig2;

var template (omit) octetstring v_EncodedSCG_Config := omit;

var template (value) SRB_ToAddModList v_SRB_ToAddModList;

var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerToAddModList;

var template (value) NR_RadioBearerList_Type v_RadioBearerList;

var boolean v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1);

var template (value) UL_AM_RLC v_ParametersTX := f_NR_UL_AM_RLC(v_IsFR1);

var template (value) DL_AM_RLC v_ParametersRX := f_NR_DL_AM_RLC(v_IsFR1);

// @siclog "Step 1" siclog@

f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_OctetData (bit2oct(f_NR_ConvertCiphAlg2Bitstring(f_NR_AS_CipheringAlgorithm_Get())), "NR Ciphering Alg"));

f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_OctetData (bit2oct(f_NR_ConvertIntAlg2Bitstring(f_NR_AS_IntegrityAlgorithm_Get())), "NR Integrity Alg"));

//Set the NR ENDC Configuration for SRB1

v_SRB_ToAddModList := {

cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB1, cs_NR_PDCP_Config_SplitSRB(-, f_NR_LogicalChannelId_SRB(tsc_SRB1))),

cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB2)

};

v_RadioBearerConfig1 := cs_38508_RadioBearerConfigDef({cs_NR_SRB_ToAddMod(tsc_SRB3)}, -, -, cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig(f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()), secondary));

v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(v_SRB_ToAddModList, -, -, cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig(f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()), master));

v_RLC_BearerToAddModList := {

cs_RLC_BearerConfig_SRB_ReestablishRLC(tsc_SRB1),

cs_38508_RLC_BearerConfig_SRB(tsc_SRB3),

cs_38508_RLC_BearerConfig_DRB(tsc_NR_DRB2,

cs_38508_RLC_Config_AM_DRB(v_ParametersTX, v_ParametersRX),

cs_38508_LogicalChannelConfig_DRB,

true_)

};

v_SCGConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_SCG, v_RLC_BearerToAddModList);

//SS : configure SRB1 with RLCAckProhibit

v_RadioBearerList := {

cs_NR_SS_RadioBearer_Config(tsc_NR_RbId_SRB1, cs_NR_PDCP_Configuration_Proxy(crs_RlcBearerRouting_Eutra(eutra_Cell1)), cs_NR_RlcBearerConfig_AM(v_IsFR1 ,-, f_NR_LogicalChannelId_SRB(tsc_SRB1))),

cs_NR_SS_SRB3_Config

};

f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_RadioBearerList, cs_TimingInfo_Now); //Configure SRB1 in NR with PDCP set to proxy + SRB3 on NR

f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig1, v_SCGConfig, -, -, false);  // Now send octetstrings to EUTRA

f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig2, omit);  // Now send octetstrings to EUTRA

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA);

// @siclog "Step 3" siclog@

// SS transmits an NR RRCReconfiguration message to reconfigure SCG DRB on SRB3/NR Cell 1

v_SCGConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_SCG, -, -, cs_NR_MAC_CellGroupConfig(-, -, cs_NR_BSR_ConfigDef(sf5, sf40)));

v_EncodedSCG_Config := bit2oct(encvalue(v_SCGConfig));

SRB.send(cas_NR_SRB3_RrcPdu_REQ(nr_Cell1, -, cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, -, v_EncodedSCG_Config)));

// @siclog "Step 4" siclog@

// Check: Does the UE transmit an NR RRCReconfigurationComplete message on SRB3/NR Cell 1?

SRB.receive(car_NR_SRB3_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");

f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger);

// @siclog "Step 5" siclog@

v_SCGConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_SCG, -, { f_NR_LogicalChannelId_SRB(tsc_SRB3) });

// Now send octetstrings to EUTRA

f_NR_SendRRCReconfigurationContentsToEUTRA (cs_NR_RadioBearer_ReleaseSRB3, v_SCGConfig, -, -, false);

// @siclog "Step 7-8" siclog@

v_SCGConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_SCG, -, { f_NR_LogicalChannelId_SRB(tsc_SRB1) });

f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_SCGConfig, -, -, false); // Now send octetstrings to EUTRA

}

After Change

function fl_TC_8_2_2_3_1_ENDC_NR_TestBody() runs on ENDC_NR_PTC

{// @sic R5-202647, R5-206340, R5s201348 sic@

var template (value) CellGroupConfig v_SCGConfig;

var template (value) RadioBearerConfig v_RadioBearerConfig1;

var template (value) RadioBearerConfig v_RadioBearerConfig2;

var template (omit) octetstring v_EncodedSCG_Config := omit;

var template (value) SRB_ToAddModList v_SRB_ToAddModList;

var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerToAddModList;

var template (value) NR_RadioBearerList_Type v_RadioBearerList;

var boolean v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1);

var template (value) UL_AM_RLC v_ParametersTX := f_NR_UL_AM_RLC(v_IsFR1);

var template (value) DL_AM_RLC v_ParametersRX := f_NR_DL_AM_RLC(v_IsFR1);

// @siclog "Step 1" siclog@

f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_OctetData (bit2oct(f_NR_ConvertCiphAlg2Bitstring(f_NR_AS_CipheringAlgorithm_Get())), "NR Ciphering Alg"));

f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_OctetData (bit2oct(f_NR_ConvertIntAlg2Bitstring(f_NR_AS_IntegrityAlgorithm_Get())), "NR Integrity Alg"));

//Set the NR ENDC Configuration for SRB1

v_SRB_ToAddModList := {

cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB1, cs_NR_PDCP_Config_SplitSRB(-, f_NR_LogicalChannelId_SRB(tsc_SRB1))),

cs_NR_SRB_ToAddMod_ReestablishPDCP(tsc_SRB2)

};

v_RadioBearerConfig1 := cs_38508_RadioBearerConfigDef({cs_NR_SRB_ToAddMod(tsc_SRB3)}, -, -, cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig(f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()), secondary));

v_RadioBearerConfig2 := cs_38508_RadioBearerConfigDef(v_SRB_ToAddModList, -, -, cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig(f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()), master));

v_RLC_BearerToAddModList := {

cs_RLC_BearerConfig_SRB_ReestablishRLC(tsc_SRB1),

cs_38508_RLC_BearerConfig_SRB(tsc_SRB3),

cs_38508_RLC_BearerConfig_DRB(tsc_NR_DRB2,

cs_38508_RLC_Config_AM_DRB(v_ParametersTX, v_ParametersRX),

cs_38508_LogicalChannelConfig_DRB,

true_)

};

v_SCGConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_SCG, v_RLC_BearerToAddModList);

//SS : configure SRB1 with RLCAckProhibit

v_RadioBearerList := {

cs_NR_SS_RadioBearer_Config(tsc_NR_RbId_SRB1, cs_NR_PDCP_Configuration_Proxy(crs_RlcBearerRouting_Eutra(eutra_Cell1)), cs_NR_RlcBearerConfig_AM_SRB( f_NR_LogicalChannelId_SRB(tsc_SRB1))),

cs_NR_SS_SRB3_Config

};

f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_RadioBearerList, cs_TimingInfo_Now); //Configure SRB1 in NR with PDCP set to proxy + SRB3 on NR

f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig1, v_SCGConfig, -, -, false);  // Now send octetstrings to EUTRA

f_NR_SendRRCReconfigurationContentsToEUTRA (v_RadioBearerConfig2, omit);  // Now send octetstrings to EUTRA

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA);

// Get Security info from EUTRA, calculate latest keys and configure SS

f_ENDC_GetKeysActivateSecurity(nr_Cell1, EUTRA);

// @siclog "Step 3" siclog@

// SS transmits an NR RRCReconfiguration message to reconfigure SCG DRB on SRB3/NR Cell 1

v_SCGConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_SCG, -, -, cs_NR_MAC_CellGroupConfig(-, -, cs_NR_BSR_ConfigDef(sf5, sf40)));

v_EncodedSCG_Config := bit2oct(encvalue(v_SCGConfig));

SRB.send(cas_NR_SRB3_RrcPdu_REQ(nr_Cell1, -, cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, -, v_EncodedSCG_Config)));

// @siclog "Step 4" siclog@

// Check: Does the UE transmit an NR RRCReconfigurationComplete message on SRB3/NR Cell 1?

SRB.receive(car_NR_SRB3_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");

f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger);

// @siclog "Step 5" siclog@

v_SCGConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_SCG, -, { f_NR_LogicalChannelId_SRB(tsc_SRB3) });

// Now send octetstrings to EUTRA

f_NR_SendRRCReconfigurationContentsToEUTRA (cs_NR_RadioBearer_ReleaseSRB3, v_SCGConfig, -, -, false);

// @siclog "Step 7-8" siclog@

v_SCGConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_SCG, -, { f_NR_LogicalChannelId_SRB(tsc_SRB1) });

f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_SCGConfig, -, -, false); // Now send octetstrings to EUTRA

}

## 2.3 Change 3

| Function name | New template cs_NR_RlcBearerConfig_AM_SRB |
| --- | --- |
| Reason for change | Template cs_NR_RlcBearerConfig_AM is used to configure SS side SRB. This will end up using template to configure IE “sn_FieldLength” with value snSize18. For Signalling bearers sn_FieldLength shall be set as snSize12. |
| Summary of change | New template cs_NR_RlcBearerConfig_AM_SRB to configure SS for SRB3 |
| TTCN module | RRCReconfig_ENDC_NR.ttcn |
| MCC160 Comment |  |

New template

template (value) NR_RlcBearerConfig_Type cs_NR_RlcBearerConfig_AM_SRB(template (omit) NR_LogicalChannelId_Type p_LogicalChannelId) :=

/* @status    APPROVED (ENDC, NR5GC, NR5GC_IRAT) */

//@sic R5s190109 change 8: added p_LogicalChannelId sic@

cs_NR_RlcBearerConfig_Common(cs_NR_SS_RLC_Configuration_AM(cs_38508_UL_RLC_SRB, cs_38508_DL_RLC_SRB), p_LogicalChannelId);