3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210092

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1526 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC idle mode test case 6.4.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - When the UE is switched off/on at step 6B/6C, TTCN should also release all its stored information about DRBs and PDU sessions. - Following agreed prose CR R5-205600, generic initial registration procedure 4.5.3.2-1 should be executed at step7. But TTCN is currently executing 4.9.4.2.2-1. - PUSCH-Config is not included in rrcResume, this can lead to PUSCH reception failures at step 18B |  |  |  |  |  |  |  |  |
| Summary of change: |  | - call f_NR5GC_MobileInfo_Init to reset MobileInfo() after step 6B - perform generic initial registration procedure at step 7 - include PUSCH-Config in rrcResume |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.2.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_6_4_2_1_NR5GC_TestBody |
| --- | --- |
| Reason for change | 1, When the UE is switched off/on at step 6B/6C, TTCN should also release all its stored information about DRBs and PDU sessions. 2, Following agreed prose CR R5-205600, generic initial registration procedure 4.5.3.2-1 should be executed at step7. But TTCN is currently executing 4.9.4.2.2-1. 3, PUSCH-Config is not included in rrcResume, this can lead to PUSCH reception failures at step 18B |
| Summary of change | 1, call f_NR5GC_MobileInfo_Init to reset MobileInfo() after step 6B 2, perform generic initial registration procedure at step 7 3, include PUSCH-Config in rrcResume |
| TTCN module | Inactive_CellReSelection_NR5GC |
| MCC160 Comment |  |

Before Change

function f_TC_6_4_2_1_NR5GC_TestBody() runs on NR5GC_PTC { // @sic R5s200377 R5-204415 R5-205600 sic@ //FR2 Power levels are FFS var integer v_T1_Cell1_RS_EPRE_FR1 := -115; var integer v_T2_Cell1_RS_EPRE_FR1 := tsc_NR_ServingCellSSS_EPRE_FR1; var integer v_T2_Cell2_RS_EPRE_FR1 := -78; // @sic R5-198879 sic@ var integer v_T0_Cell1_RS_EPRE_FR1 := tsc_NR_ServingCellSSS_EPRE_FR1; var integer v_T0_Cell2_RS_EPRE_FR1 := tsc_NR_NonSuitableOffCellSSS_EPRE; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2; var octetstring v_EncodedRrcSetup; var template(value) DL_CCCH_Message v_RrcSetup; var template(value) CellGroupConfig v_CellGroupConfig; var template(value) PDSCH_Config v_PDSCH_Config; var NR_DownlinkBWP_Type v_NR_DownlinkBWP_Cell2; var template(omit) NR_BWP_DownlinkDedicated_PDSCH_Config_Type v_BWP_DownlinkDedicated_PDSCH_Config; var integer v_SSB_IndexCell2; v_RrcSetup := f_NR_38508_RRCSetup(nr_Cell1); v_EncodedRrcSetup := f_NR_RRC_DL_CCCH_Encvalue(v_RrcSetup); v_CellPowerList_AtT0:={ cs_NR_CellPower(nr_Cell1, v_T0_Cell1_RS_EPRE_FR1, v_T0_Cell1_RS_EPRE_FR1), cs_NR_CellPower(nr_Cell2, v_T0_Cell2_RS_EPRE_FR1, v_T0_Cell2_RS_EPRE_FR1) }; v_CellPowerList_AtT1:={ cs_NR_CellPower(nr_Cell1, v_T1_Cell1_RS_EPRE_FR1, v_T1_Cell1_RS_EPRE_FR1) }; v_CellPowerList_AtT2:={ cs_NR_CellPower(nr_Cell1, v_T2_Cell1_RS_EPRE_FR1, v_T2_Cell1_RS_EPRE_FR1), cs_NR_CellPower(nr_Cell2, v_T2_Cell2_RS_EPRE_FR1, v_T2_Cell2_RS_EPRE_FR1) }; // @siclog "Step 1" siclog@ // The SS changes NR Cell 1 SSS level according to the row "T1" in table 6.4.2.1.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); // @siclog "Step 2 - Void" siclog@ // @siclog "Step 2A" siclog@ // Wait 15s (Note 1) f_Delay(15.0); // @siclog "Step 3" siclog@ // The SS transmits a Paging message. f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE); // @siclog "Step 4" siclog@ // Check: Does the UE attempt to transmit an uplink message within the next 10s? f_NR_Check_NoUplinkMessage(10.0); // @siclog "Step 5" siclog@ // The SS changes NR Cell 1 SSS levels according to the row "T0" in table 6.4.2.1.3.2-1/2. f_NR_SS_RRC_ReleaseSecurity(nr_Cell1, cs_TimingInfo_Now); f_NR_SS_SRBs_DRBs_Release(nr_Cell1, cs_TimingInfo_Now, f_NR_GetActiveDRBs(nr_Cell1)); f_NR_SS_SRBs_DRBs_Config(nr_Cell1, cs_TimingInfo_Now, omit); f_NR_SetCellPowerList(v_CellPowerList_AtT0); f_NR_SS_RachProcedureConfig(nr_Cell1, cs_NR_RachProcedureConfig_CBRA_Msg4Based( f_NR_CellInfo_GetPhysicalParameters(nr_Cell1), cs_NR_RachProcedureMsg4WithCcchMsg(v_EncodedRrcSetup))); // @siclog "Steps 6/6A - Void" siclog@ // @siclog "Step 6B" siclog@ // The UE is switched off. f_UT_SwitchOffUE(UT, false); // @siclog "Step 6C" siclog@ // The UE is switched on. f_UT_SwitchOnUE(UT, false); // @siclog "Step 7" siclog@ // Generic procedure specified in TS 38.508-1 Table 4.5.3.2-1 are performed, in which the RRCRelease message includes the suspendConfig with NR_RRC_INACTIVE condition thereby moving the UE to RRC_INACTIVE. f_NR5GC_RRC_ConnectedState3N_Def (nr_Cell1); f_NR_RRC_InactiveDef(nr_Cell1,-,false); f_NR_SS_AS_ActivateSecurity_Current(nr_Cell2,tsc_NR_38508_NextHopChainingCount); f_NR_SS_RachProcedureConfig(nr_Cell2, cs_NR_RachProcedureConfig_CBRA_Msg4Based(f_NR_CellInfo_GetPhysicalParameters(nr_Cell2), cs_NR_RachProcedureMsg4WithNoRrcMsg)); // @siclog "Step 8-15 Void" siclog@ // @siclog "Step 16" siclog@ // The SS changes NR Cell 2 SSS level according to the row "T2" in table 6.4.2.1.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT2); // @siclog "Step 17 - Void" siclog@ // @siclog "Step 18" siclog@ // Check: Does the UE transmit an RRCResumeRequest message on the NR Cell 2? SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell2, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, rna_Update))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 18"); // @siclog "Step 18A" siclog@ // The SS transmits an RRCResume message v_SSB_IndexCell2 := f_NR_CellInfo_GetSSB_Index(nr_Cell2); v_NR_DownlinkBWP_Cell2 := f_NR_CellInfo_GetDownlinkBWP(nr_Cell2, tsc_NR_BWP_Id); v_PDSCH_Config := v_NR_DownlinkBWP_Cell2.Pdsch.ConfigDedicated.R15; // Table 6.4.2.1.3.3-6 v_PDSCH_Config.dmrs_DownlinkForPDSCH_MappingTypeA := omit; v_PDSCH_Config.tci_StatesToAddModList[0] := cs_38508_TCI_State(v_SSB_IndexCell2); v_PDSCH_Config.prb_BundlingType := cs_NR_Prb_OmitBundleSize; v_BWP_DownlinkDedicated_PDSCH_Config := {setup := v_PDSCH_Config}; v_CellGroupConfig := f_NR_GetSCGConfigResume(nr_Cell2); v_CellGroupConfig.spCellConfig.spCellConfigDedicated.initialDownlinkBWP := cs_BWP_DownlinkDedicated_Def(-, v_BWP_DownlinkDedicated_PDSCH_Config); f_NR_SendRRCResume_Def(nr_Cell2, -, -, v_CellGroupConfig); // @siclog "Step 18B" siclog@ // The UE transmit an RRCResumeComplete message f_NR_RRCResumeComplete_Def(nr_Cell2); // @siclog "Step 19" siclog@ // The SS transmits an RRCRelease message including suspendConfig with NR_RRC_INACTIVE condition. f_NR_RRC_InactiveDef(nr_Cell2);

After Change

function f_TC_6_4_2_1_NR5GC_TestBody() runs on NR5GC_PTC { // @sic R5s200377 R5-204415 R5-205600 sic@ //FR2 Power levels are FFS var integer v_T1_Cell1_RS_EPRE_FR1 := -115; var integer v_T2_Cell1_RS_EPRE_FR1 := tsc_NR_ServingCellSSS_EPRE_FR1; var integer v_T2_Cell2_RS_EPRE_FR1 := -78; // @sic R5-198879 sic@ var integer v_T0_Cell1_RS_EPRE_FR1 := tsc_NR_ServingCellSSS_EPRE_FR1; var integer v_T0_Cell2_RS_EPRE_FR1 := tsc_NR_NonSuitableOffCellSSS_EPRE; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2; var octetstring v_EncodedRrcSetup; var template(value) DL_CCCH_Message v_RrcSetup; var template(value) CellGroupConfig v_CellGroupConfig; var template(value) PDSCH_Config v_PDSCH_Config; var NR_DownlinkBWP_Type v_NR_DownlinkBWP_Cell2; var template(omit) NR_BWP_DownlinkDedicated_PDSCH_Config_Type v_BWP_DownlinkDedicated_PDSCH_Config; var integer v_SSB_IndexCell2; var template (value) PUSCH_Config v_PUSCH_Config; v_RrcSetup := f_NR_38508_RRCSetup(nr_Cell1); v_EncodedRrcSetup := f_NR_RRC_DL_CCCH_Encvalue(v_RrcSetup); v_CellPowerList_AtT0:={ cs_NR_CellPower(nr_Cell1, v_T0_Cell1_RS_EPRE_FR1, v_T0_Cell1_RS_EPRE_FR1), cs_NR_CellPower(nr_Cell2, v_T0_Cell2_RS_EPRE_FR1, v_T0_Cell2_RS_EPRE_FR1) }; v_CellPowerList_AtT1:={ cs_NR_CellPower(nr_Cell1, v_T1_Cell1_RS_EPRE_FR1, v_T1_Cell1_RS_EPRE_FR1) }; v_CellPowerList_AtT2:={ cs_NR_CellPower(nr_Cell1, v_T2_Cell1_RS_EPRE_FR1, v_T2_Cell1_RS_EPRE_FR1), cs_NR_CellPower(nr_Cell2, v_T2_Cell2_RS_EPRE_FR1, v_T2_Cell2_RS_EPRE_FR1) }; // @siclog "Step 1" siclog@ // The SS changes NR Cell 1 SSS level according to the row "T1" in table 6.4.2.1.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); // @siclog "Step 2 - Void" siclog@ // @siclog "Step 2A" siclog@ // Wait 15s (Note 1) f_Delay(15.0); // @siclog "Step 3" siclog@ // The SS transmits a Paging message. f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE); // @siclog "Step 4" siclog@ // Check: Does the UE attempt to transmit an uplink message within the next 10s? f_NR_Check_NoUplinkMessage(10.0); // @siclog "Step 5" siclog@ // The SS changes NR Cell 1 SSS levels according to the row "T0" in table 6.4.2.1.3.2-1/2. f_NR_SS_RRC_ReleaseSecurity(nr_Cell1, cs_TimingInfo_Now); f_NR_SS_SRBs_DRBs_Release(nr_Cell1, cs_TimingInfo_Now, f_NR_GetActiveDRBs(nr_Cell1)); f_NR_SS_SRBs_DRBs_Config(nr_Cell1, cs_TimingInfo_Now, omit); f_NR_SetCellPowerList(v_CellPowerList_AtT0); f_NR_SS_RachProcedureConfig(nr_Cell1, cs_NR_RachProcedureConfig_CBRA_Msg4Based( f_NR_CellInfo_GetPhysicalParameters(nr_Cell1), cs_NR_RachProcedureMsg4WithCcchMsg(v_EncodedRrcSetup))); // @siclog "Steps 6/6A - Void" siclog@ // @siclog "Step 6B" siclog@ // The UE is switched off. f_UT_SwitchOffUE(UT, false); // @siclog "Step 6C" siclog@ // The UE is switched on. f_UT_SwitchOnUE(UT, false); f_NR5GC_MobileInfo_Init(); // @siclog "Step 7" siclog@ // Generic procedure specified in TS 38.508-1 Table 4.5.3.2-1 are performed, in which the RRCRelease message includes the suspendConfig with NR_RRC_INACTIVE condition thereby moving the UE to RRC_INACTIVE. f_NR_RRC_SetupRequest_Def(nr_Cell1); f_NR5GC_RRC_Idle_Steps3_20(nr_Cell1, TESTMode_OFF, noRrcConnectionRelease, Initial_NoSecurity); f_NR_RRC_InactiveDef(nr_Cell1,-,false); f_NR_SS_AS_ActivateSecurity_Current(nr_Cell2,tsc_NR_38508_NextHopChainingCount); f_NR_SS_RachProcedureConfig(nr_Cell2, cs_NR_RachProcedureConfig_CBRA_Msg4Based(f_NR_CellInfo_GetPhysicalParameters(nr_Cell2), cs_NR_RachProcedureMsg4WithNoRrcMsg)); // @siclog "Step 8-15 Void" siclog@ // @siclog "Step 16" siclog@ // The SS changes NR Cell 2 SSS level according to the row "T2" in table 6.4.2.1.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT2); // @siclog "Step 17 - Void" siclog@ // @siclog "Step 18" siclog@ // Check: Does the UE transmit an RRCResumeRequest message on the NR Cell 2? SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell2, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, rna_Update))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 18"); // @siclog "Step 18A" siclog@ // The SS transmits an RRCResume message v_SSB_IndexCell2 := f_NR_CellInfo_GetSSB_Index(nr_Cell2); v_NR_DownlinkBWP_Cell2 := f_NR_CellInfo_GetDownlinkBWP(nr_Cell2, tsc_NR_BWP_Id); v_PDSCH_Config := v_NR_DownlinkBWP_Cell2.Pdsch.ConfigDedicated.R15; // Table 6.4.2.1.3.3-6 v_PDSCH_Config.dmrs_DownlinkForPDSCH_MappingTypeA := omit; v_PDSCH_Config.tci_StatesToAddModList[0] := cs_38508_TCI_State(v_SSB_IndexCell2); v_PDSCH_Config.prb_BundlingType := cs_NR_Prb_OmitBundleSize; v_BWP_DownlinkDedicated_PDSCH_Config := {setup := v_PDSCH_Config}; v_CellGroupConfig := f_NR_GetSCGConfigResume(nr_Cell2); v_CellGroupConfig.spCellConfig.spCellConfigDedicated.initialDownlinkBWP := cs_BWP_DownlinkDedicated_Def(-, v_BWP_DownlinkDedicated_PDSCH_Config); v_PUSCH_Config := cs_38508_PUSCH_Config_FR1(cs_38508_DMRS_UplinkConfig(pos1), v_SSB_IndexCell2); v_CellGroupConfig.spCellConfig.spCellConfigDedicated.uplinkConfig.initialUplinkBWP.pusch_Config := {setup := v_PUSCH_Config}; f_NR_SendRRCResume_Def(nr_Cell2, -, -, v_CellGroupConfig); // @siclog "Step 18B" siclog@ // The UE transmit an RRCResumeComplete message f_NR_RRCResumeComplete_Def(nr_Cell2); // @siclog "Step 19" siclog@ // The SS transmits an RRCRelease message including suspendConfig with NR_RRC_INACTIVE condition. f_NR_RRC_InactiveDef(nr_Cell2); }