3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210096

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0662 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS over 5GS test case 7.12 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| s |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | Anritsu Ltd, Keysight |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add NR TC 7.12 MO Voice MO-MT UE with-without preconditions |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to 5GS IMS test case 7.12 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.12.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

3	Branches Executed	4

4	Execution Log Files	4

4.1	Qualcomm SM8350 + SDX60	4

5	References	4

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of test case 7.12 which is part of the IMS NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

Kanchan, Mohit

mohit.kanchan@keysight.com

## Verification Test Summary

Test Case: 	7.12

ATS Version:	iwd-TTCN3-B2020-09_D20wk50

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR, Keysight 5G Protocol Conformance Toolset

UE used:	Qualcomm SM8350

Verification Status:	PASS

# Corrections required

Please note, in addition to the changes mentioned below, R5s210037 CR is also needed for this test case to work.

## Change 1

| Function name | f_NR5GC_CallRelease |
| --- | --- |
| Reason for change | The function f_NR5GC_CallRelease has an extra coordination message expected in f_NR5GC_CallRelease from IMS component which is not sent |
| Summary of change | Removed awaiting of coordination message from IMS component in f_NR5GC_CallRelease |
| TTCN module | NR5GC_IMSProcedures.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR5GC_CallRelease(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_NR5GC_ModifyPDUSession_RemoveIMSVoice (p_NR_CellId); f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Call Release"); }

After Change:

function f_NR5GC_CallRelease(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); //REMOVED //f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_NR5GC_ModifyPDUSession_RemoveIMSVoice (p_NR_CellId); f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Call Release"); }

# Branches Executed

The test case was executed on NR band n41.

# Execution Log Files

## Qualcomm SM8350 + SDX60

The Qualcomm SM8350 + SDX60 IMS UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR and Keysight 5G Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu\TC_7_12_LOGS.xml

Keysight\TC_7_12_LOGS.html

- PICS/PIXIT parameter file:

Anritsu\ TC_7_12_PICS_PIXIT.txt
Keysight\ TC_7_12_PICS_PIXIT.xml

# References

| [1] | R5s210097 |
| --- | --- |