3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210102

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1530 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to f_NR5GC_ModifyPDUSession_RemoveIMSVoice |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation of this function, f_BuildDefaultQoSRules("2")is called which sets IEI to omit, and also assigns 2 as rule number. For IMS voice call, the rule number should be 7_Voice and IEI should be set as ‘7A’O. This needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Instead of calling f_BuildDefaultQoSRules("2"), called f_BuildDefaultQoSRules("7_Voice",'7A'O"). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IMS Testcases will fail a conformant UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | IMS voice call over NR testcases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	3

1.1	function f_NR5GC_ModifyPDUSession_RemoveIMSVoice	3

# Corrections required

## function f_NR5GC_ModifyPDUSession_RemoveIMSVoice

| Function name | f_NR5GC_ModifyPDUSession_RemoveIMSVoice |
| --- | --- |
| Reason for change | In the current TTCN implementation of this function, f_BuildDefaultQoSRules("2")is called which sets IEI to omit, and also assigns 2 as rule number. For IMS voice call, the rule number should be 7_Voice and IEI should be set as ‘7A’O. This needs to be corrected. |
| Summary of change | Instead of calling f_BuildDefaultQoSRules("2"), called f_BuildDefaultQoSRules("7_Voice",'7A'O"). |
| TTCN module | Common/NR5GC/ NR5GC_IMSProcedures |
| MCC160 Comment |  |

Before Change:

function f_NR5GC_ModifyPDUSession_RemoveIMSVoice(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; var boolean v_InterworkWithEPS := false; var O1_Type v_EPSBearer := '00'O; var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_SS_Config; var template (value) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), IMS_DNN); var IntegerList_Type v_DRBIdList := f_NR_GetDRBList_ForIMSPDUSession(); var template (value) NR_RLC_BearerToReleaseList_Type v_RLC_BearerToReleaseList; var NR_DRBInfo_Type v_DRBInfo; var template (value) DRB_ToReleaseList v_DRB_ToReleaseList; var integer i; var integer j := 0; if (isvalue(v_GSM_MobilityInfo.EPS_Bearer)) { // This is only set if the UE supports EPS v_InterworkWithEPS := true; v_EPSBearer := int2oct(valueof(v_GSM_MobilityInfo.EPS_Bearer), 1); } // build components of Reconfiguration message v_PiggyBackedMsgToSend := f_Get_NG_PDUSessionModificationCommand(v_GSM_MobilityInfo.SessionId, omit, -, -, -, -, f_BuildDefaultQoSRules( "2" ), cs_QoSFlowDescr({cs_QoS_Flow2(v_InterworkWithEPS,v_EPSBearer)})); …… }

After Change:

function f_NR5GC_ModifyPDUSession_RemoveIMSVoice(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { var NR_SRB_COMMON_IND v_ReceivedAsp; var boolean v_InterworkWithEPS := false; var O1_Type v_EPSBearer := '00'O; var template (value) NG_NAS_DL_Pdu_Type v_PiggyBackedMsgToSend; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) CellGroupConfig v_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_SS_Config; var template (value) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), IMS_DNN); var IntegerList_Type v_DRBIdList := f_NR_GetDRBList_ForIMSPDUSession(); var template (value) NR_RLC_BearerToReleaseList_Type v_RLC_BearerToReleaseList; var NR_DRBInfo_Type v_DRBInfo; var template (value) DRB_ToReleaseList v_DRB_ToReleaseList; var integer i; var integer j := 0; if (isvalue(v_GSM_MobilityInfo.EPS_Bearer)) { // This is only set if the UE supports EPS v_InterworkWithEPS := true; v_EPSBearer := int2oct(valueof(v_GSM_MobilityInfo.EPS_Bearer), 1); } // build components of Reconfiguration message v_PiggyBackedMsgToSend := f_Get_NG_PDUSessionModificationCommand(v_GSM_MobilityInfo.SessionId, omit, -, -, -, -, f_BuildDefaultQoSRules( "7_Voice",'7A'O ), cs_QoSFlowDescr({cs_QoS_Flow2(v_InterworkWithEPS, v_EPSBearer)})); …… }