3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210119

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4579 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_EUTRA_NB_Authentication_S13() |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2021-01-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current implementation KeNB is calculated incorrectly in the case where EARFCN = maxEARFCN (= 65535) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the checking to handle the case when p_EARFCN_DL = maxEARFCN (also corrected for other boundary cases) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | e.g. 8.2.4.3 and others |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_EUTRA_NB_Authentication_S13() |
| --- | --- |
| Reason for change | In the current implementation KeNB is calculated incorrectly in the case where EARFCN = maxEARFCN (= 65535) |
| Summary of change | Correct the checking to handle the case when p_EARFCN_DL = maxEARFCN (also corrected for other boundary cases) |
| TTCN module | EUTRA_NB_SecurityFunctions.ttcn |

Before change

function f_EUTRA_NB_Authentication_S13(KDF_Type p_KDF,

B256_Type p_KENB,

B256_Type p_NH,

PhysCellId p_PhyCellId,

ARFCN_ValueEUTRA_r9 p_EARFCN_DL,

boolean p_FromNH := false) // False for Deriving Kenb* from Kenb, else from NH

return B256_Type //@sic R5s100496 sic@

{ // @sic R5s100138 sic@

//@sic R5-165892 Band extension p_EARFCN_DL -> ARFCN_ValueEUTRA_r9 sic@

const octetstring const_S13_FC :='13'O; //@sic R5s100178 sic@

var octetstring v_S;

var B256_Type v_SourceKey;

// Generation of String

v_S := const_S13_FC;

//FC = 0x13

v_S :=   ( v_S & int2oct ( p_PhyCellId , 2 ) );

//P0 = PCI (target physical cell id)

v_S :=   ( v_S & '0002'O );

//L0 = length of PCI (i.e. 0x00 0x02)

if ( p_EARFCN_DL < maxEARFCN) {  //@sic R5-165892 Band extension sic@

v_S :=   ( v_S & int2oct ( p_EARFCN_DL , 2 ) );

v_S :=   ( v_S & '0002'O );

} else if ((p_EARFCN_DL > maxEARFCN_Plus1) and (p_EARFCN_DL < maxEARFCN2)) {

v_S :=   ( v_S & int2oct ( p_EARFCN_DL , 3 ) ); // @sic R5s170217 sic@

v_S :=   ( v_S & '0003'O );

}

//P1 = EARFCN-DL (target physical cell downlink frequency)

//L1 length of EARFCN-DL (i.e. L1 = 0x00 0x02 if EARFCN-DL is between 0 and 65535, and L1 = 0x00 0x03 if EARFCN-DL is between 65536 and 262143)

if ( p_FromNH == false) //  @sic R5s100496 sic@ Kenb* derived from Kenb

{

v_SourceKey := p_KENB; // @sic R5s160711 sic@

}

else // Kenb* derived from Fresh Derived NH

{

v_SourceKey := p_NH; // @sic R5s160711 sic@

};

return fx_KeyDerivationFunction ( p_KDF, v_SourceKey, v_S );//@sic R5s100496, R5s160711 sic@

};

After change

function f_EUTRA_NB_Authentication_S13(KDF_Type p_KDF,

B256_Type p_KENB,

B256_Type p_NH,

PhysCellId p_PhyCellId,

ARFCN_ValueEUTRA_r9 p_EARFCN_DL,

boolean p_FromNH := false) // False for Deriving Kenb* from Kenb, else from NH

return B256_Type //@sic R5s100496 sic@

{ // @sic R5s100138 sic@

//@sic R5-165892 Band extension p_EARFCN_DL -> ARFCN_ValueEUTRA_r9 sic@

const octetstring const_S13_FC :='13'O; //@sic R5s100178 sic@

var octetstring v_S;

var B256_Type v_SourceKey;

// Generation of String

v_S := const_S13_FC;

//FC = 0x13

v_S :=   ( v_S & int2oct ( p_PhyCellId , 2 ) );

//P0 = PCI (target physical cell id)

v_S :=   ( v_S & '0002'O );

//L0 = length of PCI (i.e. 0x00 0x02)

if ( p_EARFCN_DL <= maxEARFCN) {  //@sic R5-165892 Band extension sic@

v_S :=   ( v_S & int2oct ( p_EARFCN_DL , 2 ) );

v_S :=   ( v_S & '0002'O );

} else if ((p_EARFCN_DL >= maxEARFCN_Plus1) and (p_EARFCN_DL <= maxEARFCN2)) {

v_S :=   ( v_S & int2oct ( p_EARFCN_DL , 3 ) ); // @sic R5s170217 sic@

v_S :=   ( v_S & '0003'O );

}

//P1 = EARFCN-DL (target physical cell downlink frequency)

//L1 length of EARFCN-DL (i.e. L1 = 0x00 0x02 if EARFCN-DL is between 0 and 65535, and L1 = 0x00 0x03 if EARFCN-DL is between 65536 and 262143)

if ( p_FromNH == false) //  @sic R5s100496 sic@ Kenb* derived from Kenb

{

v_SourceKey := p_KENB; // @sic R5s160711 sic@

}

else // Kenb* derived from Fresh Derived NH

{

v_SourceKey := p_NH; // @sic R5s160711 sic@

};

return fx_KeyDerivationFunction ( p_KDF, v_SourceKey, v_S );//@sic R5s100496, R5s160711 sic@

};