3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210126

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4583 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to EUTRA EMM test case 9.2.3.2.11 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2021-01-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | IMS registration during steps 13-24 not handled correctly |  |  |  |  |  |  |  |  |
| Summary of change: |  | Reset IP and IMS before for cell I before new attach procedure at step 13-24 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.2.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## 2.1 Change 1

| Function name | fl_TC_9_2_3_2_11_Body() |
| --- | --- |
| Reason for change | IMS registration during steps 13-24 not handled correctly |
| Summary of change | Reset IP and IMS before for cell I before new attach procedure at step 13-24 |
| TTCN module | NAS_TrackingArea_UG_UTRAN.ttcn |

Before change

function fl_TC_9_2_3_2_11_Body() runs on EUTRA_PTC

{

......................

// @siclog "Step 9" siclog@

// Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds?  [1,F]

f_EUTRA_CheckNoAttachOn1Cell(eutra_CellG, 90.0); // @sic R5-115560 sic@

// EXCEPTION: Steps 10a1 to 10a2 describe behaviour that depends on the UE capability;

//            the "lower case letter" identifies a step sequence that takes place if a capability is supported

// IF pc_UTRAN or pc_GERAN THEN sets the cell type of Cell G to the "non-Suitable cell" and sets the cell type

// of Cell 5 or Cell 24 to the "Serving cell".

//@siclog "Steps 10a1 - 10a2" siclog@

v_CellPowerList := {

cs_CellPower(eutra_CellG, tsc_NonSuitableCellRS_EPRE),

cs_CellPower(eutra_CellI, tsc_NonSuitableCellRS_EPRE),

cs_CellPower(eutra_CellH, tsc_NonSuitableOffCellRS_EPRE)

};

f_EUTRA_SetCellPowerList(v_CellPowerList);

f_EUTRA_SendTriggerToOtherRAT(true); // @sic R5-112638 sic@

//@siclog "Step 11" siclog@

//Set the cell type of Cell I to the "Serving cell"

//Set the cell type of Cell 5 or Cell 24 to the "non-Suitable cell"

f_EUTRA_SetCellPower(eutra_CellI, tsc_ServingCellRS_EPRE);

// The following messages are to be observed on Cell I unless explicitly stated otherwise.

//@siclog "Step 12" siclog@

// Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message?

v_NasInd := f_EUTRA_RRC_ConnEst_DefWithNas(eutra_CellI,

v_RRC_TI,

cr_EstablishmentCause_Any,

cr_NAS_IndicationWithPiggybacking(tsc_SHT_NoSecurityProtection,

v_AttachReq,

cr_508_PDN_CONNECTIVITY_REQUEST(tsc_PdnRequest_InitialAttach,

f_GetPdnType())));

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 12");

//@siclog "Steps 13 - 24" siclog@

// Steps 5 to 13 of the generic test procedure in TS 36.508 subclause 4.5.2.3 are performed.

f_EUTRA_InitialRegistration_Step5_18b1(eutra_CellI, STATE2_IDLEUPDATE, Combined, v_NasInd);

……………………..

}

After change

function fl_TC_9_2_3_2_11_Body() runs on EUTRA_PTC

{

.............

// @siclog "Step 9" siclog@

// Check: Does the UE transmit an ATTACH REQUEST message in the next 30 seconds?  [1,F]

f_EUTRA_CheckNoAttachOn1Cell(eutra_CellG, 90.0); // @sic R5-115560 sic@

// EXCEPTION: Steps 10a1 to 10a2 describe behaviour that depends on the UE capability;

//            the "lower case letter" identifies a step sequence that takes place if a capability is supported

// IF pc_UTRAN or pc_GERAN THEN sets the cell type of Cell G to the "non-Suitable cell" and sets the cell type

// of Cell 5 or Cell 24 to the "Serving cell".

//@siclog "Steps 10a1 - 10a2" siclog@

v_CellPowerList := {

cs_CellPower(eutra_CellG, tsc_NonSuitableCellRS_EPRE),

cs_CellPower(eutra_CellI, tsc_NonSuitableCellRS_EPRE),

cs_CellPower(eutra_CellH, tsc_NonSuitableOffCellRS_EPRE)

};

f_EUTRA_SetCellPowerList(v_CellPowerList);

f_EUTRA_SendTriggerToOtherRAT(true); // @sic R5-112638 sic@

f_EUTRA_ResetIPandIMS (eutra_CellI);

//@siclog "Step 11" siclog@

//Set the cell type of Cell I to the "Serving cell"

//Set the cell type of Cell 5 or Cell 24 to the "non-Suitable cell"

f_EUTRA_SetCellPower(eutra_CellI, tsc_ServingCellRS_EPRE);

// The following messages are to be observed on Cell I unless explicitly stated otherwise.

//@siclog "Step 12" siclog@

// Check: Does the UE transmit an ATTACH REQUEST message including a PDN CONNECTIVITY REQUEST message?

v_NasInd := f_EUTRA_RRC_ConnEst_DefWithNas(eutra_CellI,

v_RRC_TI,

cr_EstablishmentCause_Any,

cr_NAS_IndicationWithPiggybacking(tsc_SHT_NoSecurityProtection,

v_AttachReq,

cr_508_PDN_CONNECTIVITY_REQUEST(tsc_PdnRequest_InitialAttach,

f_GetPdnType())));

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 12");

//@siclog "Steps 13 - 24" siclog@

// Steps 5 to 13 of the generic test procedure in TS 36.508 subclause 4.5.2.3 are performed.

f_EUTRA_InitialRegistration_Step5_18b1(eutra_CellI, STATE2_IDLEUPDATE, Combined, v_NasInd);

……………..

}