3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210134

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1550 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 9.1.5.1.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the currecnt TTCN implementation, according to the 38.523-1 Table 9.1.5.1.3.3.2-1 Steps 66-71 should be implemented according to Steps 15 – 20a1 of 38.508 Table 4.5.2.2-2. The current TTCN implementation does not take into account the Steps 19a1, 19Aa1, 19Aa2 and 20a1. This needs to be implemented. Also, updates are needed to function f_UT_ReadRejectedNSSAI and f_NR5GC_RRC_Idle_Extension as a result of the above. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Implemented the the Steps 19a1, 19Aa1, 19Aa2 and 20a1. Also updated the functions f_UT_ReadRejectedNSSAI and f_NR5GC_RRC_Idle_Extension. Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testcase will fail a conformant UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.1.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	3

1.1	f_UT_ReadRejectedNSSAI	3

1.2	fl_TC_9_1_5_1_3_Body	5

1.3	f_NR5GC_RRC_Idle_Extension	7

# Corrections required

## f_UT_ReadRejectedNSSAI

| Function name | f_UT_ReadRejectedNSSAI |
| --- | --- |
| Reason for change | In the currecnt TTCN implementation, the variable v_Rejected := regexp(v_ResultString, "\+C5GNSSAIRDP: [0-9]+,[""*""](0,1),(*)", 0); is assigned . This regular expression will allow a default configured NSSAI that contains a comma as well. It will also allow a rejected NSSAI which contains a comma. Also, the current implementation is unable to extract value when a UE returns +C5GNSSAIRDP: 2,"01:02",1,"02#0" or +C5GNSSAIRDP: 2,"01:02",1,"01#1" or +C5GNSSAIRDP: 2,"01:02" which are all valid output for AT command AT+C5GNSSAIRDP=1,"00101" This needs to be addressed. |
| Summary of change | Instead of v_Rejected := regexp(v_ResultString, "\+C5GNSSAIRDP: [0-9]+,[""*""](0,1),(*)", 0); Called: v_Rejected := regexp(v_ResultString, "\+C5GNSSAIRDP: [0-9]+,[^,]#(0,),(*)", 0); This is able to handle all the above mentioned ouputs of AT command AT+C5GNSSAIRDP=1,"00101" |
| TTCN module | Common/Common/UpperTesterFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

function f_UT_ReadRejectedNSSAI(UT_PTC_MTC_PORT p_Port, template (omit) charstring p_PLMN_ID := omit, boolean p_Access3GPP := true) return charstring { var charstring v_ResultString := f_UT_ReadNSSAI_DynamicParams(p_Port, Default_Rejected, p_PLMN_ID); var charstring v_Rejected := regexp(v_ResultString, "\+C5GNSSAIRDP: [0-9]+,[""*""](0,1),(*)", 0); // @sic R5s201142 sic@ if (p_Access3GPP) { return regexp(v_Rejected, "[0-9]+,""(*)""", 0); } else { return regexp(v_Rejected, "[0-9]+,""*"",[0-9]+,""(*)""", 0); } }

After Change:

function f_UT_ReadRejectedNSSAI(UT_PTC_MTC_PORT p_Port, template (omit) charstring p_PLMN_ID := omit, boolean p_Access3GPP := true) return charstring { var charstring v_ResultString := f_UT_ReadNSSAI_DynamicParams(p_Port, Default_Rejected, p_PLMN_ID); //var charstring v_Rejected := regexp(v_ResultString, "\+C5GNSSAIRDP: [0-9]+,[""*""](0,1),(*)", 0); // @sic R5s201142 sic@ var charstring v_Rejected := regexp(v_ResultString, "\+C5GNSSAIRDP: [0-9]+,[^,]#(0,),(*)", 0); //WA#9_1_5_1_3 if (p_Access3GPP) { return regexp(v_Rejected, "[0-9]+,""(*)""", 0); } else { return regexp(v_Rejected, "[0-9]+,""*"",[0-9]+,""(*)""", 0); } }

## fl_TC_9_1_5_1_3_Body

| Function name | fl_TC_9_1_5_1_3_Body |
| --- | --- |
| Reason for change | In the currecnt TTCN implementation, according to the 38.523-1 Table 9.1.5.1.3.3.2-1 Steps 66-71 should be implemented according to Steps 15 – 20a1 of 38.508 Table 4.5.2.2-2. The current TTCN implementation does not take into account the Steps 19a1, 19Aa1, 19Aa2 and 20a1. This needs to be implemented. |
| Summary of change | Implemented the the Steps 19a1, 19Aa1, 19Aa2 and 20a1. Please see below. |
| TTCN module | ATS_NR5GC/9_1_5/Initial_Registration_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> //@siclog "Step 66-71" siclog@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellC, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_NG_REGISTRATION_COMPLETE)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Complete Message Failed"); } f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellC, -, tsc_NR_RbId_SRB1, //@sic R5s200758 sic@ -, -, -, -, -, -, -, cs_S_NSSAI('22'O, '01'O, '02'O, omit)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 68"); f_NR_RRCRelease(v_NGC_NASCellC); <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> //@siclog "Step 66-71" siclog@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellC, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_NG_REGISTRATION_COMPLETE)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Complete Message Failed"); } if (pc_noOf_PDUsNewConnection >0 ){ //WA#9_1_5_1_3 f_NR_RRCRelease(v_NGC_NASCellC); f_NR5GC_RRC_Idle_Extension(v_NGC_NASCellC, TESTMode_OFF, noRrcConnectionRelease, -, -, cs_S_NSSAI('22'O, '01'O, '02'O, omit)); }else{ f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellC, -, tsc_NR_RbId_SRB1, //@sic R5s200758 sic@ -, -, -, -, -, -, -, cs_S_NSSAI('22'O, '01'O, '02'O, omit)); } f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 68"); f_NR_RRCRelease(v_NGC_NASCellC); <<SKIPPED CODE>>

## f_NR5GC_RRC_Idle_Extension

| Function name | f_NR5GC_RRC_Idle_Extension |
| --- | --- |
| Reason for change | In the current TTCN implementation, after Registration Accept transmission at Step 66, cs_S_NSSAI('22'O, '01'O, '02'O, omit) is passed as a paramtere to function f_NR5GC_PDUSessionEstablishment. As a result of change 1.2, the function f_NR5GC_RRC_Idle_Extension needs updating in order to allow the passing of NSSAI to function f_NR5GC_PDUSessionEstablishment. |
| Summary of change | Updated the function f_NR5GC_RRC_Idle_Extension to be able to pass NSSAI as an argument. Please see below. |
| TTCN module | Common/NR5GC/NR5GC_NASSteps.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR5GC_RRC_Idle_Extension(NR_CellId_Type p_NR_CellId, NR_TESTMODE_STATE_Type p_TestMode, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, template (value) UE_TestLoopModeA_NR_LB_Setup_Type p_UE_TestLoopModeA_NR_LB_Setup := cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O) runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (p_NR_CellId); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); var integer v_NoOfExistingSessions := f_NR5GC_MobileInfo_GetNoOfSessions(); // @sic R5-206291 sic@ timer t_Wait := 10.0; // Step 0 @sic R5-204399 sic@ t_Wait.start; alt { [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(p_NR_CellId, cr_38508_RRCSetupRequest(?, ?)))) { t_Wait.stop; } [] t_Wait.timeout { // Step 1 if (v_NoOfExistingSessions == 0) { // @sic R5-206291 sic@ /* Only trigger if there's no sessions established automatically to ensure there's (at least) one session established for the test body */ f_UT_RequestPDUSession(UT, 1, tsc_RandomAPN); } } } // Steps 2 - 4 v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, ?); // @sic R5s200489 sic@ if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0000'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } // Steps 5-6 // RRC Security Mode procedure f_NR_RRC_ActivateSecurity (p_NR_CellId, valueof(v_ReceivedMsg.SecurityProtection.NasCount)); // Use NasCount returned from Service Req // FFS - Prose DOES NOT specify PDU Session Status to be reflected back from Req message // Step 7 SRB.send (cas_NR_SRB_NasPdu_REQ(p_NR_CellId, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg(v_ReceivedMsg.Pdu.Msg.service_Request.pduSessionStatus)))); // @sic R5s201387 Baseline Moving sic@ // Step 8 f_NR5GC_PDUSessionEstablishment (p_NR_CellId, pc_noOf_PDUsNewConnection, tsc_NR_RbId_SRB1); // @sic R5-206291 sic@ //Step 9a1 if (p_ReleaseConnection == rrcConnectionRelease) { // Release RRC to enter RRC_IDLE f_NR_RRCRelease(p_NR_CellId); } else { // Step 8Aa1 - 8Aa2 select (p_TestMode) { case (TEST_LOOPModeA_ON) { f_NR5GC_CloseUE_TestLoopModeA (p_NR_CellId, p_UE_TestLoopModeA_NR_LB_Setup); } case (TEST_LOOPModeB_ON) { f_NR5GC_CloseUE_TestLoopModeB (p_NR_CellId, p_IpPduDelayTime); } case (PING_Or_TEST_LOOPModeB_ON) { if (not pc_IP_Ping) { f_NR5GC_CloseUE_TestLoopModeB (p_NR_CellId, p_IpPduDelayTime); } } } } }

After Change:

function f_NR5GC_RRC_Idle_Extension(NR_CellId_Type p_NR_CellId, NR_TESTMODE_STATE_Type p_TestMode, RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease, template (value) UE_TestLoopModeA_NR_LB_Setup_Type p_UE_TestLoopModeA_NR_LB_Setup := cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling, UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O, template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI /*WA#9_1_5_1_3*/) runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (p_NR_CellId); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); var integer v_NoOfExistingSessions := f_NR5GC_MobileInfo_GetNoOfSessions(); // @sic R5-206291 sic@ timer t_Wait := 10.0; // Step 0 @sic R5-204399 sic@ t_Wait.start; alt { [] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(p_NR_CellId, cr_38508_RRCSetupRequest(?, ?)))) { t_Wait.stop; } [] t_Wait.timeout { // Step 1 if (v_NoOfExistingSessions == 0) { // @sic R5-206291 sic@ /* Only trigger if there's no sessions established automatically to ensure there's (at least) one session established for the test body */ f_UT_RequestPDUSession(UT, 1, tsc_RandomAPN); } } } // Steps 2 - 4 v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, ?); // @sic R5s200489 sic@ if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0000'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } // Steps 5-6 // RRC Security Mode procedure f_NR_RRC_ActivateSecurity (p_NR_CellId, valueof(v_ReceivedMsg.SecurityProtection.NasCount)); // Use NasCount returned from Service Req // FFS - Prose DOES NOT specify PDU Session Status to be reflected back from Req message // Step 7 SRB.send (cas_NR_SRB_NasPdu_REQ(p_NR_CellId, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_ServiceAcceptMsg(v_ReceivedMsg.Pdu.Msg.service_Request.pduSessionStatus)))); // @sic R5s201387 Baseline Moving sic@ // Step 8 //WA#9_1_5_1_3 f_NR5GC_PDUSessionEstablishment (p_NR_CellId, pc_noOf_PDUsNewConnection, tsc_NR_RbId_SRB1); // @sic R5-206291 sic@ f_NR5GC_PDUSessionEstablishment (p_NR_CellId, pc_noOf_PDUsNewConnection, tsc_NR_RbId_SRB1, //@sic R5s200758 sic@ -, -, -, -, -, -, -, p_S_NSSAI); //Step 9a1 if (p_ReleaseConnection == rrcConnectionRelease) { // Release RRC to enter RRC_IDLE f_NR_RRCRelease(p_NR_CellId); } else { // Step 8Aa1 - 8Aa2 select (p_TestMode) { case (TEST_LOOPModeA_ON) { f_NR5GC_CloseUE_TestLoopModeA (p_NR_CellId, p_UE_TestLoopModeA_NR_LB_Setup); } case (TEST_LOOPModeB_ON) { f_NR5GC_CloseUE_TestLoopModeB (p_NR_CellId, p_IpPduDelayTime); } case (PING_Or_TEST_LOOPModeB_ON) { if (not pc_IP_Ping) { f_NR5GC_CloseUE_TestLoopModeB (p_NR_CellId, p_IpPduDelayTime); } } } } }