3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210135

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1551 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC test case 8.1.4.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For Steps 15-16 of test case 8.1.4.1.2, current TTCN implementation is: - Receive RRC Reconfiguration Complete for Step 15 - Execute f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps11_14 - Receive Registration Request for Step 16 UE can send Step 16 Registration Request 20ms after Step 15 RRCReconfigurationComplete. However, function f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps11_14 takes a lot of time to finish. For instance, the function f_NR_InitialiseRachProcedure takes around 250ms to finish. Main cause of delay is the f_NR_38508_RRCSetup which takes around 200ms to finish. It is suggested to move execution of f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps11_14 after receiving Registration Request for Step 16. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Move f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps11_14 after receiving Registration Request for Step 16. - Declare new function f_NR5GC_508RRC_IntraNR_HO_InterCell_Steps1_10. - Declare new function f_NR5GC_508RRC_IntraNR_HO_InterCell_CommonSteps1_10. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | fl_TC_8_1_4_1_2_TestBody |
| --- | --- |
| Reason for change | For Steps 15-16 of test case 8.1.4.1.2, current TTCN implementation is: - Receive RRC Reconfiguration Complete for Step 15 - Execute f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps11_14 - Receive Registration Request for Step 16 UE can send Step 16 Registration Request 20ms after Step 15 RRCReconfigurationComplete. However, function f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps11_14 takes a lot of time to finish. For instance, the function f_NR_InitialiseRachProcedure takes around 250ms to finish. Main cause of delay is the f_NR_38508_RRCSetup which takes around 200ms to finish. It is suggested to move execution of f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps11_14 after receiving Registration Request for Step 16. |
| Summary of change | Move f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps11_14 after receiving Registration Request for Step 16. |
| TTCN module | RRC_IntraNR_Handover_NR5GC |
| MCC160 Comment |  |

Before change:

function fl_TC_8_1_4_1_2_TestBody() runs on NR5GC_PTC

{

…

//@siclog "Steps 14-15" siclog@

//The SS transmits an RRCReconfiguration including dedicatedSIB1-Delivery containing SIB1 of NR Cell 3 to order the UE to perform handover to NR Cell 3

//Check: Does the UE transmit an RRCReconfigurationComplete message on NR Cell 3?

v_SIB1 := f_NR_GetSIB1(nr_Cell3);

v_SIB1.cellAccessRelatedInfo.plmn_IdentityList[0].trackingAreaCode := int2bit(2,24);

f_NR_CellInfo_SetTAC(nr_Cell3, int2bit(2,24)); //@sic R5s200309 sic@

//@sic R5s201303 sic@

//To avoid a possible collision between the final clean of the target cell and the UE sending a RegistrationRequest, the HO procedure is split

f_NR5GC_508RRC_IntraNR_HO_InterCell_Steps1_14(nr_Cell1,

nr_Cell3,

-,

false, // RBConfig_KeyChange := false

-,

v_SIB1); // SIB1

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15");

//@siclog "Step 16" siclog@

//Check: Does UE transmit a REGISTRATION REQUEST message to update the registration of the actual tracking area?

//This message will be received in SRB2, as this SRB was established after the HO

SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell3, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; //@sic R5s200309 sic@

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, Mobility, ?, -,-,-,-,-,-, *)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

else {

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 16");

}

//@sic R5s201303 sic@

//Final clean up of the target cell

f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps15_16(nr_Cell1);

//@siclog "Steps 17-18" siclog@

//SS responds with REGISTRATION ACCEPT message

//The UE send a REGISTRATION COMPLETE message

//@sic R5s200309 sic@

f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell3,

v_GMM_MobilityInfo,

-, -,

noRrcConnectionRelease,

-,

tsc_NR_RbId_SRB2); // On SRB2

//@siclog "Step 19" siclog@

//Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB #n associated with the first PDU session on NR Cell 3?

//@sic R5-204450 sic@

v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession();

v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList();

f_NR5GC_CheckDataPath(nr_Cell3,

f_NR5GC_GetPdnIndex(v_PDUSessionInfoList[0]),

dedicatedBearer,

v_DRBId,

"Step 19");

}// End of fl_TC_8_1_4_1_2_TestBody

After change:

function fl_TC_8_1_4_1_2_TestBody() runs on NR5GC_PTC

{

…

//@siclog "Steps 14-15" siclog@

//The SS transmits an RRCReconfiguration including dedicatedSIB1-Delivery containing SIB1 of NR Cell 3 to order the UE to perform handover to NR Cell 3

//Check: Does the UE transmit an RRCReconfigurationComplete message on NR Cell 3?

v_SIB1 := f_NR_GetSIB1(nr_Cell3);

v_SIB1.cellAccessRelatedInfo.plmn_IdentityList[0].trackingAreaCode := int2bit(2,24);

f_NR_CellInfo_SetTAC(nr_Cell3, int2bit(2,24)); //@sic R5s200309 sic@

//@sic R5s201303 sic@

//To avoid a possible collision between the final clean of the target cell and the UE sending a RegistrationRequest, the HO procedure is split

f_NR5GC_508RRC_IntraNR_HO_InterCell_Steps1_10(nr_Cell1,

nr_Cell3,

-,

false, // RBConfig_KeyChange := false

-,

v_SIB1); // SIB1

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15");

//@siclog "Step 16" siclog@

//Check: Does UE transmit a REGISTRATION REQUEST message to update the registration of the actual tracking area?

//This message will be received in SRB2, as this SRB was established after the HO

SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell3, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; //@sic R5s200309 sic@

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, Mobility, ?, -,-,-,-,-,-, *)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

else {

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 16");

}

//@sic R5s201303 sic@

//Final clean up of the target cell

f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps11_14(nr_Cell1, nr_Cell3);

f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps15_16(nr_Cell1);

//@siclog "Steps 17-18" siclog@

//SS responds with REGISTRATION ACCEPT message

//The UE send a REGISTRATION COMPLETE message

//@sic R5s200309 sic@

f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell3,

v_GMM_MobilityInfo,

-, -,

noRrcConnectionRelease,

-,

tsc_NR_RbId_SRB2); // On SRB2

//@siclog "Step 19" siclog@

//Check: Does the test result of generic test procedure in TS 38.508-1 subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB #n associated with the first PDU session on NR Cell 3?

//@sic R5-204450 sic@

v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession();

v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList();

f_NR5GC_CheckDataPath(nr_Cell3,

f_NR5GC_GetPdnIndex(v_PDUSessionInfoList[0]),

dedicatedBearer,

v_DRBId,

"Step 19");

}// End of fl_TC_8_1_4_1_2_TestBody

## 2.2 Change 2

| Function name | New function f_NR5GC_508RRC_IntraNR_HO_InterCell_Steps1_10 |
| --- | --- |
| Reason for change | As stated in Change 1, f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps11_14 will be moved after receiving Registration Request for Step 16. It is suggested to declare another function f_NR5GC_508RRC_IntraNR_HO_InterCell_Steps1_10. |
| Summary of change | Declare new function f_NR5GC_508RRC_IntraNR_HO_InterCell_Steps1_10. |
| TTCN module | NR5GC_Handover |
| MCC160 Comment |  |

New function:

function f_NR5GC_508RRC_IntraNR_HO_InterCell_Steps1_10(NR_CellId_Type                       p_SourceCellId,

NR_CellId_Type                       p_TargetCellId,

RNTI_Value                           p_RNTI_Value :=  tsc_C_RNTI_Value3,

boolean                              p_RBConfig_KeyChange := true,

template (omit)  MeasConfig          p_MeasConfig  := omit,

template (omit) SIB1                 p_SIB1 := omit,

template (omit) SystemInformation    p_SysInfo := omit,

template (omit) MasterKeyUpdate      p_MasterKeyUpdate := omit,

template (omit) ReconfigurationWithSync.rach_ConfigDedicated p_RACH_ConfigDedicated := omit

) runs on NR5GC_PTC

{ //@sic R5s2008190 sic@

var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_TargetCellId);

var template (value) NR_RachProcedureConfig_Type v_RachProcedureConfig := cs_NR_RachProcedureConfig_CRNTI_HO(v_NR_PhysicalParameters, p_RNTI_Value); //CBRA

var template (value) DL_DCCH_Message v_RRCReconfiguration;

var template (value) NR_DRBInfoList_Type v_DRBInfoList := f_NR5GC_MobileInfo_GetDRBInfoList();

var NextHopChainingCount v_NCC;

if (isvalue(p_RACH_ConfigDedicated.uplink)){ //CFRA

v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_TargetCellId);

v_RachProcedureConfig := cs_NR_RachProcedureConfig_CFRA(v_NR_PhysicalParameters);

}

if (isvalue(p_MasterKeyUpdate)){

v_NCC := valueof(p_MasterKeyUpdate.nextHopChainingCount);

} else {

v_NCC := tsc_NR_38508_NextHopChainingCount;

}

v_RRCReconfiguration := f_NR5GC_RRCReconfiguration_IntraNR_HO(p_TargetCellId,

p_RNTI_Value,

p_RBConfig_KeyChange,

p_MeasConfig,

p_SIB1,

p_SysInfo,

p_MasterKeyUpdate,

p_RACH_ConfigDedicated

);

f_NR5GC_508RRC_IntraNR_HO_InterCell_CommonSteps1_10(p_SourceCellId,

p_TargetCellId,

p_RNTI_Value,

p_RBConfig_KeyChange,

v_RRCReconfiguration,

v_RachProcedureConfig,

v_DRBInfoList,

-,

-,

-,

v_NCC,

p_RACH_ConfigDedicated,

p_MeasConfig

);

}

## 2.3 Change 3

| Function name | New function f_NR5GC_508RRC_IntraNR_HO_InterCell_CommonSteps1_10 |
| --- | --- |
| Reason for change | As stated in Change 1, f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps11_14 will be moved after receiving Registration Request for Step 16. It is suggested to declare another function f_NR5GC_508RRC_IntraNR_HO_InterCell_CommonSteps1_10. |
| Summary of change | Declare new function f_NR5GC_508RRC_IntraNR_HO_InterCell_CommonSteps1_10. |
| TTCN module |  |
| MCC160 Comment |  |

New function:

function f_NR5GC_508RRC_IntraNR_HO_InterCell_CommonSteps1_10(NR_CellId_Type                               p_SourceCellId,

NR_CellId_Type                               p_TargetCellId,

RNTI_Value                                   p_RNTI_Value :=  tsc_C_RNTI_Value3,

boolean                                      p_RBConfig_KeyChange := true,

template (value) DL_DCCH_Message             p_RRCReconfiguration,

template (value) NR_RachProcedureConfig_Type p_RachProcedureConfig,

template (value) NR_DRBInfoList_Type         p_DrbInfoList,

template (value) UL_GrantConfig_Type         p_UL_GrantConfig_SourceCell := cs_UL_GrantConfig_OnSR,

template (value) UL_GrantConfig_Type         p_UL_GrantConfig_TargetCell := cs_UL_GrantConfig_OnSR,

template (present) UL_DCCH_Message           p_RRCConnReconfigComplete := cr_38508_RRCReconfigurationComplete,

NextHopChainingCount                         p_NCC := tsc_NR_38508_NextHopChainingCount,

template (omit) ReconfigurationWithSync.rach_ConfigDedicated p_RACH_ConfigDedicated := omit,

template (omit)  MeasConfig                  p_MeasConfig  := omit

) runs on NR_BASE_PTC

{ //@sic R5s200819 sic@

f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps1_3(p_SourceCellId,

p_TargetCellId,

p_RBConfig_KeyChange,

p_DrbInfoList

);

f_NR5GC_508RRC_IntraNR_HO_InterCell_Common_Steps4_9( p_SourceCellId,

p_TargetCellId,

p_RNTI_Value,

p_RBConfig_KeyChange,

p_RRCReconfiguration,

p_RachProcedureConfig,

p_UL_GrantConfig_SourceCell,

p_UL_GrantConfig_TargetCell,

p_NCC,

p_RACH_ConfigDedicated);

// Step 10: Target Cell:    Receive RRCReconfigurationComplete

SRB.receive(car_NR_SRB1_RrcPdu_IND(p_TargetCellId, p_RRCConnReconfigComplete));

}