3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210138

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1552 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC test case 7.1.1.9.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Function f_NR_GetCellGroupConfigHO takes around 250ms to finish. This causes a delay during intra-cell handover procedure. It is suggested to prepare CellGroupConfig message before Step 1 similar to test case 7.1.2.3.11 and 7.1.2.2.6 implementation. - Open UE test loop mode and Deactivate test mode procedures should be executed before Postamble. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Prepare CellGroupConfig message before Step 1 and pass it to f_NR_RLC_ReconfigurationHO. - Add f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode before Postamble. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.9.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | fl_TC_7_1_1_9_1_NR_TestBody |
| --- | --- |
| Reason for change | - Function f_NR_GetCellGroupConfigHO takes around 250ms to finish. This causes a delay during intra-cell handover procedure. It is suggested to prepare CellGroupConfig message before Step 1 similar to test case 7.1.2.3.11 and 7.1.2.2.6 implementation. - Open UE test loop mode and Deactivate test mode procedures should be executed before Postamble. |
| Summary of change | - Prepare CellGroupConfig message before Step 1 and pass it to f_NR_RLC_ReconfigurationHO. - Add f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode before Postamble. |
| TTCN module | MAC_NR5GC |
| MCC160 Comment |  |

Before change:

function fl_TC_7_1_1_9_1_NR_TestBody(DRB_Identity p_NR_DRB_Id,

template (value) NR_RadioBearerList_Type p_NR_RadioBearerList) runs on NR5GC_PTC

{ //@sic R5-206320 sic@

var B6_Type v_LCID := int2bit( f_NR_LogicalChannelId_DRB(p_NR_DRB_Id),6);

var SubFrameTiming_Type v_SubFrameTiming;

var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList;

var template (value) NR_RLC_BearerToAddModList_Type v_NR_RLC_BearerToAddModList;       //@sic R5s201457 sic@

var RLC_Config.am.ul_AM_RLC.t_PollRetransmit  v_T_PollRetransmit := f_NR_UL_AM_T_PollRetransmit(f_NR_CellInfo_GetIsFR1(nr_Cell1));

var RLC_Config.am.dl_AM_RLC.t_Reassembly v_T_Reassembly:= f_NR_DL_AM_T_Reassembly(f_NR_CellInfo_GetIsFR1(nr_Cell1));

var template (value)  NR_RadioBearer_Type v_SS_DRB_Config := cs_NR_SS_RadioBearer_MAC_TestMode(p_NR_DRB_Id, cs_NR_MAC_TestModeInfo_NormalMode);

v_RLC_BearerConfigList := f_NR_ReconfigureRLC_AM(p_NR_DRB_Id,

-,

v_T_PollRetransmit,

-,

-,

-,

-,

v_T_Reassembly,

-,

true_);

v_NR_RLC_BearerToAddModList := f_NR5GC_GetExisting_BearerToAddModList(true_);         //@sic R5s201457 sic@

v_NR_RLC_BearerToAddModList[2] := v_RLC_BearerConfigList[0];                          //@sic R5s201457 sic@

f_TC_7_1_1_9_1_Step1to3(p_NR_DRB_Id,v_LCID);

//@siclog "Steps 3" siclog@

//SS transmits NR RRCReconfiguration message to perform MCG change with reconfigurationWithSync with the same SPCell

v_SubFrameTiming := f_NR_RLC_ReconfigurationHO (nr_Cell1, p_NR_DRB_Id, v_NR_RLC_BearerToAddModList, cs_38508_PDCP_Config(ms60, -, -, omit),v_SS_DRB_Config,true);//@sic R5-204418 sic@

f_TC_7_1_1_9_1_Step5to13(p_NR_DRB_Id,v_LCID, v_SubFrameTiming,p_NR_RadioBearerList);

//@siclog "Steps 14" siclog@

//SS transmits NR RRCReconfiguration message to perform MCG change with reconfigurationWithSync with the same SPCell

v_SubFrameTiming := f_NR_RLC_ReconfigurationHO (nr_Cell1, p_NR_DRB_Id, v_NR_RLC_BearerToAddModList, cs_38508_PDCP_Config(ms60, -, -, omit),v_SS_DRB_Config,true); //@sic R5-204418 sic@

f_TC_7_1_1_9_1_Step16to21(p_NR_DRB_Id, v_LCID, v_SubFrameTiming,p_NR_RadioBearerList);

//@siclog "Steps 22" siclog@

//SS transmits NR RRCReconfiguration message to perform MCG change with reconfigurationWithSync with the same SPCell

v_SubFrameTiming := f_NR_RLC_ReconfigurationHO (nr_Cell1, p_NR_DRB_Id, v_NR_RLC_BearerToAddModList, cs_38508_PDCP_Config(ms60, -, -, omit),v_SS_DRB_Config,true); //@sic R5-204418 sic@

f_TC_7_1_1_9_1_Step24to28(p_NR_DRB_Id, v_LCID,p_NR_RadioBearerList);

f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A);

}

After change:

function fl_TC_7_1_1_9_1_NR_TestBody(DRB_Identity p_NR_DRB_Id,

template (value) NR_RadioBearerList_Type p_NR_RadioBearerList) runs on NR5GC_PTC

{ //@sic R5-206320 sic@

var B6_Type v_LCID := int2bit( f_NR_LogicalChannelId_DRB(p_NR_DRB_Id),6);

var SubFrameTiming_Type v_SubFrameTiming;

var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList;

var template (value) NR_RLC_BearerToAddModList_Type v_NR_RLC_BearerToAddModList;       //@sic R5s201457 sic@

var RLC_Config.am.ul_AM_RLC.t_PollRetransmit  v_T_PollRetransmit := f_NR_UL_AM_T_PollRetransmit(f_NR_CellInfo_GetIsFR1(nr_Cell1));

var RLC_Config.am.dl_AM_RLC.t_Reassembly v_T_Reassembly:= f_NR_DL_AM_T_Reassembly(f_NR_CellInfo_GetIsFR1(nr_Cell1));

var template (value)  NR_RadioBearer_Type v_SS_DRB_Config := cs_NR_SS_RadioBearer_MAC_TestMode(p_NR_DRB_Id, cs_NR_MAC_TestModeInfo_NormalMode);

var template (value) CellGroupConfig v_CellGroupConfig;

v_RLC_BearerConfigList := f_NR_ReconfigureRLC_AM(p_NR_DRB_Id,

-,

v_T_PollRetransmit,

-,

-,

-,

-,

v_T_Reassembly,

-,

true_);

v_NR_RLC_BearerToAddModList := f_NR5GC_GetExisting_BearerToAddModList(true_);         //@sic R5s201457 sic@

v_NR_RLC_BearerToAddModList[2] := v_RLC_BearerConfigList[0];                          //@sic R5s201457 sic@

v_CellGroupConfig := f_NR_GetCellGroupConfigHO(nr_Cell1, v_NR_RLC_BearerToAddModList); //tsc_C_RNTI_Value3

f_TC_7_1_1_9_1_Step1to3(p_NR_DRB_Id,v_LCID);

//@siclog "Steps 3" siclog@

//SS transmits NR RRCReconfiguration message to perform MCG change with reconfigurationWithSync with the same SPCell

v_SubFrameTiming := f_NR_RLC_ReconfigurationHO (nr_Cell1, p_NR_DRB_Id, v_NR_RLC_BearerToAddModList, cs_38508_PDCP_Config(ms60, -, -, omit),v_SS_DRB_Config,true, v_CellGroupConfig);//@sic R5-204418 sic@

f_TC_7_1_1_9_1_Step5to13(p_NR_DRB_Id,v_LCID, v_SubFrameTiming,p_NR_RadioBearerList);

//@siclog "Steps 14" siclog@

//SS transmits NR RRCReconfiguration message to perform MCG change with reconfigurationWithSync with the same SPCell

v_SubFrameTiming := f_NR_RLC_ReconfigurationHO (nr_Cell1, p_NR_DRB_Id, v_NR_RLC_BearerToAddModList, cs_38508_PDCP_Config(ms60, -, -, omit),v_SS_DRB_Config,true, v_CellGroupConfig); //@sic R5-204418 sic@

f_TC_7_1_1_9_1_Step16to21(p_NR_DRB_Id, v_LCID, v_SubFrameTiming,p_NR_RadioBearerList);

//@siclog "Steps 22" siclog@

//SS transmits NR RRCReconfiguration message to perform MCG change with reconfigurationWithSync with the same SPCell

v_SubFrameTiming := f_NR_RLC_ReconfigurationHO (nr_Cell1, p_NR_DRB_Id, v_NR_RLC_BearerToAddModList, cs_38508_PDCP_Config(ms60, -, -, omit),v_SS_DRB_Config,true, v_CellGroupConfig); //@sic R5-204418 sic@

f_TC_7_1_1_9_1_Step24to28(p_NR_DRB_Id, v_LCID,p_NR_RadioBearerList);

f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1);

f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A);

}