3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210139

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1553 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC test case 8.1.5.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For test case 8.1.5.4.1, in case UE supports s1Cap, it will lead to TTCN error "EPS Supported but no bearer id calculated" because EPS bearer ID is not initialized. Additionally, if s1Cap is supported, QoS flow description should be updated with p_InterworkWithEPS set to True and with corresponding p_EPSBearerId. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add condition to initialize EPS bearer ID if s1Cap is supported. Variable v_QoSFlowDescr is also updated if s1Cap is supported. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.4.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | f_TC_8_1_5_4_1_NR5GC |
| --- | --- |
| Reason for change | For test case 8.1.5.4.1, in case UE supports s1Cap, it will lead to TTCN error "EPS Supported but no bearer id calculated" because EPS bearer ID is not initialized. Additionally, if s1Cap is supported, QoS flow description should be updated with p_InterworkWithEPS set to True and with corresponding p_EPSBearerId. |
| Summary of change | Add condition to initialize EPS bearer ID if s1Cap is supported. Variable v_QoSFlowDescr is also updated if s1Cap is supported. |
| TTCN module | RRC_Others_NR5GC |
| MCC160 Comment |  |

Before change:

function f_TC_8_1_5_4_1_NR5GC() runs on NR5GC_PTC

{// Counter check / Reception of CounterCheck message by the UE

var IntegerList_Type v_DRBList;

var integer v_FirstDRB;

var integer v_SecondDRB;

var integer v_PduSessionId;

var GSM_MobilityInfo_Type v_GSM_MobilityInfo_Type;

var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList;

var template (value) DRB_ToAddModList v_DRB_ToAddModList;

var template (value) QoS_Rules v_QoS_Rules;

var template (value) QoSFlowDescr v_QoSFlowDescr;

var template (value) QosFlow_Identification_Type v_QosFlowId;

var PDN_Index_Type v_PDN_Index;

var template (value) QoS_Rule v_QosRule1; //@sic R5s200733 sic@

var template (value) QoS_Rule v_QosRule4; //@sic R5s200733 sic@

f_NR5GC_Init(NR_1);

// Create and configure cell

f_NR_CellConfig_Def(nr_Cell1);

// Preamble: according to TS 38.508-1, clause 4.4A.3 Table 4.4A.3-1, Test Loop mode B. 2 DRBs are configured where DRB1 is defined as default DRB

// ----------------------------------------------------------------------------------------------------------------------------------------------

// Perform Registration and get PDU Establishment Request message

v_GSM_MobilityInfo_Type := f_MultipleQoSPreamble_Part1 (nr_Cell1);

v_PduSessionId := oct2int(v_GSM_MobilityInfo_Type.SessionId);

//Get DRB identities

v_DRBList:= f_NR_GetNextDRBIds(v_GSM_MobilityInfo_Type, true, 3); // First is Default DRB

v_FirstDRB := v_DRBList[0];

v_SecondDRB := v_DRBList[1];

//Note: In order to be able to start the IP handling during the preamble, QFI=1 is mapped temporarily to the default DRB (v_FirtDRB).

//      This mapping will be removed when moving to connected mode

v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1,

v_FirstDRB,

-, //RLC normal mode

-, //PDCP normal mode

-,

cs_SDAP_Configuration(v_PduSessionId, -, -, {1,2})), //Map QFI=1 to default in DRB to deal with initial IMS registration

f_NR_GetRadioBearerConfig_FullAM(nr_Cell1,

v_SecondDRB,

-, //RLC normal mode

-, //PDCP normal mode

-,

cs_SDAP_Configuration(v_PduSessionId, -, -, {5}))}; //@sic R5s200733 sic@

v_DRB_ToAddModList := {cs_38508_NRDRB_ToAddMod(cs_SDAP_Config(v_PduSessionId, true, {2}),//@sic R5s200733 sic@

v_FirstDRB,

cs_38508_PDCP_Config(ms300)),

cs_38508_NRDRB_ToAddMod(cs_SDAP_Config(v_PduSessionId, false, {5}), //@sic R5s200733 sic@

v_SecondDRB,

cs_38508_PDCP_Config(ms300))};

v_QosRule1 := f_BuildDefaultQoSRule ("1"); // @sic R5-202585, R5s200733 sic@

v_QosRule4 := f_BuildQoSRuleRemoteAccess("4"); //@sic R5s200733 sic@

v_QosRule4.dqrBit := '0'B; //@sic R5s200733 sic@

v_QoS_Rules := cs_QoS_Rules ({v_QosRule1, v_QosRule4, f_BuildQoSRuleRemoteAccess("5")}); //@sic R5s200733 sic@

v_QoSFlowDescr := cs_QoSFlowDescr ({cs_QoS_Flow1, cs_QoS_Flow2, cs_QoS_Flow3}); //@sic R5s200733 sic@

// Start IP handling in order for the TTCN to handle IPv6 address allocation,

// then stop it before the test body, so that TTCN can route the IP data

v_PDN_Index := f_NR5GC_GetPdnIndex(v_GSM_MobilityInfo_Type);

v_QosFlowId := cs_IP_QosFlowId(oct2int(v_GSM_MobilityInfo_Type.SessionId), 1); //Picked one QoS flow associated with the default DRB to start the IP handling

f_IP_Handling_StartPDN(IP, v_PDN_Index, cs_DrbInfo_NR(nr_Cell1, v_QosFlowId));

f_MultipleQoSPreamble_Part2 (nr_Cell1,

v_GSM_MobilityInfo_Type,

v_FirstDRB,

v_SecondDRB,

v_DRB_ToAddModList,

v_SS_Drb_ConfigList,

v_QoS_Rules,

v_QoSFlowDescr,

v_PDN_Index,

true); //@sic R5s201361, R5-206331 sic@

f_NR_AS_CipheringAlgorithm_Set(nea0);

f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1,

TEST_LOOPModeB_ON,

-,-,-,-,

v_DRB_ToAddModList,

v_SS_Drb_ConfigList);

f_NR_TestBody_Set(true);

f_TC_8_1_5_4_1_NR5GC_TestBody(v_DRBList, v_PduSessionId);

f_NR_TestBody_Set(false);

// Switch/Power off UE

f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1);

f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A);

} //end of f_TC_8_1_5_4_1_NR5GC

After change:

function f_TC_8_1_5_4_1_NR5GC() runs on NR5GC_PTC

{// Counter check / Reception of CounterCheck message by the UE

var IntegerList_Type v_DRBList;

var integer v_FirstDRB;

var integer v_SecondDRB;

var integer v_PduSessionId;

var GSM_MobilityInfo_Type v_GSM_MobilityInfo_Type;

var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList;

var template (value) DRB_ToAddModList v_DRB_ToAddModList;

var template (value) QoS_Rules v_QoS_Rules;

var template (value) QoSFlowDescr v_QoSFlowDescr;

var template (value) QosFlow_Identification_Type v_QosFlowId;

var PDN_Index_Type v_PDN_Index;

var template (value) QoS_Rule v_QosRule1; //@sic R5s200733 sic@

var template (value) QoS_Rule v_QosRule4; //@sic R5s200733 sic@

var O1_Type v_EPSBearerId  := '00'O; // default value

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;

f_NR5GC_Init(NR_1);

// Create and configure cell

f_NR_CellConfig_Def(nr_Cell1);

// Preamble: according to TS 38.508-1, clause 4.4A.3 Table 4.4A.3-1, Test Loop mode B. 2 DRBs are configured where DRB1 is defined as default DRB

// ----------------------------------------------------------------------------------------------------------------------------------------------

// Perform Registration and get PDU Establishment Request message

v_GSM_MobilityInfo_Type := f_MultipleQoSPreamble_Part1 (nr_Cell1);

v_PduSessionId := oct2int(v_GSM_MobilityInfo_Type.SessionId);

//Get DRB identities

v_DRBList:= f_NR_GetNextDRBIds(v_GSM_MobilityInfo_Type, true, 3); // First is Default DRB

v_FirstDRB := v_DRBList[0];

v_SecondDRB := v_DRBList[1];

//Note: In order to be able to start the IP handling during the preamble, QFI=1 is mapped temporarily to the default DRB (v_FirtDRB).

//      This mapping will be removed when moving to connected mode

v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1,

v_FirstDRB,

-, //RLC normal mode

-, //PDCP normal mode

-,

cs_SDAP_Configuration(v_PduSessionId, -, -, {1,2})), //Map QFI=1 to default in DRB to deal with initial IMS registration

f_NR_GetRadioBearerConfig_FullAM(nr_Cell1,

v_SecondDRB,

-, //RLC normal mode

-, //PDCP normal mode

-,

cs_SDAP_Configuration(v_PduSessionId, -, -, {5}))}; //@sic R5s200733 sic@

v_DRB_ToAddModList := {cs_38508_NRDRB_ToAddMod(cs_SDAP_Config(v_PduSessionId, true, {2}),//@sic R5s200733 sic@

v_FirstDRB,

cs_38508_PDCP_Config(ms300)),

cs_38508_NRDRB_ToAddMod(cs_SDAP_Config(v_PduSessionId, false, {5}), //@sic R5s200733 sic@

v_SecondDRB,

cs_38508_PDCP_Config(ms300))};

v_QosRule1 := f_BuildDefaultQoSRule ("1"); // @sic R5-202585, R5s200733 sic@

v_QosRule4 := f_BuildQoSRuleRemoteAccess("4"); //@sic R5s200733 sic@

v_QosRule4.dqrBit := '0'B; //@sic R5s200733 sic@

v_QoS_Rules := cs_QoS_Rules ({v_QosRule1, v_QosRule4, f_BuildQoSRuleRemoteAccess("5")}); //@sic R5s200733 sic@

v_QoSFlowDescr := cs_QoSFlowDescr ({cs_QoS_Flow1, cs_QoS_Flow2, cs_QoS_Flow3}); //@sic R5s200733 sic@

// Start IP handling in order for the TTCN to handle IPv6 address allocation,

// then stop it before the test body, so that TTCN can route the IP data

v_PDN_Index := f_NR5GC_GetPdnIndex(v_GSM_MobilityInfo_Type);

v_QosFlowId := cs_IP_QosFlowId(oct2int(v_GSM_MobilityInfo_Type.SessionId), 1); //Picked one QoS flow associated with the default DRB to start the IP handling

f_IP_Handling_StartPDN(IP, v_PDN_Index, cs_DrbInfo_NR(nr_Cell1, v_QosFlowId));

v_GMM_MobilityInfo := f_NR5GC_MobileInfo_GetGMM_MobilityInfo();

if (isvalue(v_GMM_MobilityInfo.GMMCap)) {

if (v_GMM_MobilityInfo.GMMCap.s1Cap == '1'B) {

v_GSM_MobilityInfo_Type.EPS_Bearer := valueof(f_GetNextEPSBearerId (v_GSM_MobilityInfo_Type, v_GMM_MobilityInfo));

v_EPSBearerId := hex2oct(int2hex(valueof(v_GSM_MobilityInfo_Type.EPS_Bearer), 1) & '0'H);

v_QoSFlowDescr := cs_QoSFlowDescr ({cs_QoS_Flow1(true, v_EPSBearerId),

cs_QoS_Flow2(true, v_EPSBearerId),

cs_QoS_Flow3})

}

}

f_MultipleQoSPreamble_Part2 (nr_Cell1,

v_GSM_MobilityInfo_Type,

v_FirstDRB,

v_SecondDRB,

v_DRB_ToAddModList,

v_SS_Drb_ConfigList,

v_QoS_Rules,

v_QoSFlowDescr,

v_PDN_Index,

true); //@sic R5s201361, R5-206331 sic@

f_NR_AS_CipheringAlgorithm_Set(nea0);

f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1,

TEST_LOOPModeB_ON,

-,-,-,-,

v_DRB_ToAddModList,

v_SS_Drb_ConfigList);

f_NR_TestBody_Set(true);

f_TC_8_1_5_4_1_NR5GC_TestBody(v_DRBList, v_PduSessionId);

f_NR_TestBody_Set(false);

// Switch/Power off UE

f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1);

f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A);

} //end of f_TC_8_1_5_4_1_NR5GC