3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210144

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0668 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_IMS_UpdateRequest_MessageHeaderTX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2021-01-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Currently the test case 17.2, fails with "not enough branches" error as the branches are getting refreshed twice in the function while creating the UPDATE message header. The functions ‘f_IMS_SipRequest_ViaTX’ and ‘f_IMS_ViaRefreshBranches’ refreshes the branches redundantly and later the SIP messages runs out of branches to use.(5 branches requested for one UPDATE msg) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed the redundant function ‘f_IMS_ViaRefreshBranches’ as via branches were already refreshed by ‘f_IMS_SipRequest_ViaTX’ for UPDATE msg header |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test cases with additional SIP messages sent by SS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 17.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

| Function name | f_IMS_UpdateRequest_MessageHeaderTX |
| --- | --- |
| Reason for change | Currently the test case 17.2, fails with "not enough branches" error as the branches are getting refreshed twice in the function while creating the UPDATE message header. The functions ‘f_IMS_SipRequest_ViaTX’ and ‘f_IMS_ViaRefreshBranches’ refreshes the branches redundantly and later the SIP messages runs out of branches to use.(5 branches requested for one UPDATE msg) |
| Summary of change | Removed the redundant function ‘f_IMS_ViaRefreshBranches’ as via branches were already refreshed by ‘f_IMS_SipRequest_ViaTX’ for UPDATE msg header |
| TTCN module | IMS_Procedures_CallControlCommon |
| MCC160 Comment |  |

Before change:

…

function f_IMS_UpdateRequest_MessageHeaderTX(template (value) INVITE_Request p_InviteRequest,

IMS_PreconditionIndication_Type p_PreconditionIndication := NoPreconditionIndication,

template (omit) Contact p_Contact := omit,

charstring p_CallerDomain := tsc_IMS_CallerDomain) runs on IMS_PTC return template (value) MessageHeader

{ /* @sic R5-160790, R5-160795: p_PreconditionIndication sic@ */

/* @sic R5-174544: SessionId removed sic@ */

/* @sic R5s200206 change 2.1: p_Contact and p_CallerDomain; p_Contact shall be present for MO Call sic@ */

/* NOTE: function does not support MCX yet */

var template (value) MessageHeader v_MessageHeader_Invite := p_InviteRequest.msgHeader;

var template (value) MessageHeader v_MessageHeader_Update := cs_MessageHeader_Dummy;

var integer v_CSeqValue := f_IMS_PTC_ImsInfo_CseqIncr(dialogLocal);

var template (value) Via v_Via := f_IMS_SipRequest_ViaTX(p_CallerDomain);      /* @sic R5-200457 sic@ */

var template (omit) Contact v_Contact;

var template (omit) Require v_Require := omit;

var template (omit) Supported v_Supported := omit;

select (p_PreconditionIndication) {

case (PreconditionRequired)  { v_Require := cs_Require(tsc_OptionTagList_precondition); }        /* @sic R5-160790 sic@ */

case (PreconditionSupported) { v_Supported := cs_Supported(tsc_OptionTagList_precondition); }    /* @sic R5-160795 sic@ */

}

if (fl_IMS_InviteRequestIsMO(p_InviteRequest)) {                              /* @sic R5s200206 change 2.1 sic@ */

v_Contact := p_Contact;

} else {

v_Contact := v_MessageHeader_Invite.contact;

}

v_MessageHeader_Update.via             := f_IMS_ViaRefreshBranches(v_Via);    /* @sic R5s130561 change 2: refresh branches sic@ */

v_MessageHeader_Update.toField         := f_IMS_RequestInDialog_ToHeaderTX();

v_MessageHeader_Update.fromField       := f_IMS_RequestInDialog_FromHeaderTX();

v_MessageHeader_Update.callId          := v_MessageHeader_Invite.callId;

v_MessageHeader_Update.cSeq            := cs_CseqDef(v_CSeqValue, "UPDATE");

v_MessageHeader_Update.maxForwards     := f_IMS_SipRequest_MaxForwardsTX();   /* @sic R5-203073: Max-Forwards for MCX sic@ */

v_MessageHeader_Update.contentType     := cs_ContentTypeSDP;

v_MessageHeader_Update.contact         := v_Contact;                          /* @sic R5-150692 sic@ */

v_MessageHeader_Update.require         := v_Require;                          /* @sic R5-160790 sic@ */

v_MessageHeader_Update.supported       := v_Supported;                        /* @sic R5-160795 sic@ */

return v_MessageHeader_Update;

}

....

After change:

…

function f_IMS_UpdateRequest_MessageHeaderTX(template (value) INVITE_Request p_InviteRequest,

IMS_PreconditionIndication_Type p_PreconditionIndication := NoPreconditionIndication,

template (omit) Contact p_Contact := omit,

charstring p_CallerDomain := tsc_IMS_CallerDomain) runs on IMS_PTC return template (value) MessageHeader

{ /* @sic R5-160790, R5-160795: p_PreconditionIndication sic@ */

/* @sic R5-174544: SessionId removed sic@ */

/* @sic R5s200206 change 2.1: p_Contact and p_CallerDomain; p_Contact shall be present for MO Call sic@ */

/* NOTE: function does not support MCX yet */

var template (value) MessageHeader v_MessageHeader_Invite := p_InviteRequest.msgHeader;

var template (value) MessageHeader v_MessageHeader_Update := cs_MessageHeader_Dummy;

var integer v_CSeqValue := f_IMS_PTC_ImsInfo_CseqIncr(dialogLocal);

var template (value) Via v_Via := f_IMS_SipRequest_ViaTX(p_CallerDomain);      /* @sic R5-200457 sic@ */

var template (omit) Contact v_Contact;

var template (omit) Require v_Require := omit;

var template (omit) Supported v_Supported := omit;

select (p_PreconditionIndication) {

case (PreconditionRequired)  { v_Require := cs_Require(tsc_OptionTagList_precondition); }        /* @sic R5-160790 sic@ */

case (PreconditionSupported) { v_Supported := cs_Supported(tsc_OptionTagList_precondition); }    /* @sic R5-160795 sic@ */

}

if (fl_IMS_InviteRequestIsMO(p_InviteRequest)) {                              /* @sic R5s200206 change 2.1 sic@ */

v_Contact := p_Contact;

} else {

v_Contact := v_MessageHeader_Invite.contact;

}

v_MessageHeader_Update.toField         := f_IMS_RequestInDialog_ToHeaderTX();

v_MessageHeader_Update.fromField       := f_IMS_RequestInDialog_FromHeaderTX();

v_MessageHeader_Update.callId          := v_MessageHeader_Invite.callId;

v_MessageHeader_Update.cSeq            := cs_CseqDef(v_CSeqValue, "UPDATE");

v_MessageHeader_Update.maxForwards     := f_IMS_SipRequest_MaxForwardsTX();   /* @sic R5-203073: Max-Forwards for MCX sic@ */

v_MessageHeader_Update.contentType     := cs_ContentTypeSDP;

v_MessageHeader_Update.contact         := v_Contact;                          /* @sic R5-150692 sic@ */

v_MessageHeader_Update.require         := v_Require;                          /* @sic R5-160790 sic@ */

v_MessageHeader_Update.supported       := v_Supported;                        /* @sic R5-160795 sic@ */

return v_MessageHeader_Update;

}…