3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210166

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1572 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 9.1.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation, after Step 6 following assignments are done : v_NAS_IncorrectSecurityParams := v_NAS_SecurityParams; v_AUTNSQN := substr(v_NAS_IncorrectSecurityParams.AuthParams.AUTN, 0, 48); v_AUTNSQN := v_AUTNSQN xor4b '111'B; // v_NAS_IncorrectSecurityParams.AuthParams.AUTN := v_AUTNSQN & substr(v_NAS_IncorrectSecurityParams.AuthParams.AUTN, 48, 80); These are unnecessary, as the variable v_AUTNSQN is not used anywhere in the testcase after this assignment. Moreover, the assignment : v_AUTNSQN := v_AUTNSQN xor4b '111'B; causes a runtime error. This is because according to TTCN specification ES 201 873-1 V4.12.1 Sec 7.1.5 the operator xor4b expects two operands of equal length but in the above assignment that is not the case. It is proposed to remove those assignments from the TTCN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed the assignments Please see below |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The testcase fail because of a run-time error |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required	4

1.1	function fl_TC_9_1_1_1_TestBody	4

# Corrections required

## function fl_TC_9_1_1_1_TestBody

| Function name | fl_TC_9_1_1_1_TestBody |
| --- | --- |
| Reason for change | In the current TTCN implementation, after Step 6 following assignments are done : v_NAS_IncorrectSecurityParams := v_NAS_SecurityParams; v_AUTNSQN := substr(v_NAS_IncorrectSecurityParams.AuthParams.AUTN, 0, 48); v_AUTNSQN := v_AUTNSQN xor4b '111'B; // v_NAS_IncorrectSecurityParams.AuthParams.AUTN := v_AUTNSQN & substr(v_NAS_IncorrectSecurityParams.AuthParams.AUTN, 48, 80); These are unnecessary, as the variable v_AUTNSQN is not used anywhere in the testcase after this assignment. Moreover, the assignment : v_AUTNSQN := v_AUTNSQN xor4b '111'B; causes a runtime error. This is because according to TTCN specification ES 201 873-1 V4.12.1 Sec 7.1.5 the operator xor4b expects two operands of equal length but in the above assignment that is not the case. It is proposed to remove those assignments from the TTCN. |
| Summary of change | Removed the assignments Please see below |
| TTCN module | ATS_NR5GC/9_1_1/Authentication_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> //@siclog "Step 7" siclog@ // start with the correct parameters, then change the AUTN v_NAS_IncorrectSecurityParams := v_NAS_SecurityParams; v_AUTNSQN := substr(v_NAS_IncorrectSecurityParams.AuthParams.AUTN, 0, 48); v_AUTNSQN := v_AUTNSQN xor4b '111'B; // v_NAS_IncorrectSecurityParams.AuthParams.AUTN := v_AUTNSQN & substr(v_NAS_IncorrectSecurityParams.AuthParams.AUTN, 48, 80); v_NAS_IncorrectSecurityParams := f_NG_EAP_AuthenticationInit(v_NAS_IncorrectSecurityParams, v_PLMN, hex2str(px_IMSI_Def), -, sqnFailure); // @sic R5s201021 sic@ <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> //@siclog "Step 7" siclog@ // start with the correct parameters, then change the AUTN v_NAS_IncorrectSecurityParams := v_NAS_SecurityParams; //REMOVED v_AUTNSQN := substr(v_NAS_IncorrectSecurityParams.AuthParams.AUTN, 0, 48); //REMOVED v_AUTNSQN := v_AUTNSQN xor4b '111'B; //REMOVED v_NAS_IncorrectSecurityParams.AuthParams.AUTN := v_AUTNSQN & substr(v_NAS_IncorrectSecurityParams.AuthParams.AUTN, 48, 80); v_NAS_IncorrectSecurityParams := f_NG_EAP_AuthenticationInit(v_NAS_IncorrectSecurityParams, v_PLMN, hex2str(px_IMSI_Def), -, sqnFailure); // @sic R5s201021 sic@ <<SKIPPED CODE>>