3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210172

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1277 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NR_RRCReconfigExistingDRBs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-09-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | According to TS38.331 5.3.7.2, MAC reset, spcell config released, RRCReconfiguration needs to add spCellConfig and MAC_CellGroupConfig. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add spCellConfig and MAC_CellGroupConfig. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.1.4, 8.1.5.6.1, 8.1.4.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1 Overview	3

2 Corrections required	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2019-06_D20wk25 related to the title of this CR.

Contact:	Ting Gao
	gaoting@starpointcomm.com

# Corrections required

## 2.1 Change 1

| Function name | f_NR_RRCReconfigExistingDRBs() |
| --- | --- |
| Reason for change | According to 5.3.7.2 in 38.331, MAC reset, spcell config released, RRCReconfiguration needs to add spCellConfig and MAC_CellGroupConfig. |
| Summary of change | Add spCellConfig and MAC_CellGroupConfig to RRCReconfiguration. |
| TTCN module | NR5GC_RRCSteps.ttcn |
| MCC160 Comment |  |

Before change:

function f_NR_RRCReconfigExistingDRBs(NR_CellId_Type p_CellId,

template (omit) NG_NAS_MSG_RequestList_Type p_NASMsg,

template (omit) NR_RLC_BearerToAddModList_Type p_RLCConfigList := omit,

template (omit) MAC_CellGroupConfig p_MACConfig := omit,

template (omit) DRB_ToAddModList p_DRBToAddMod := omit,

template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now,

template (omit) NR_RadioBearerList_Type p_SS_Drb_Config := omit,

boolean p_ReceiveForRRCreconfigurationComplete := true,

SRBs_AlreadyConfigured p_SRBs := SRB1,

template (omit) NR_RoutingInfoSUL_Type p_NR_RoutingInfoSUL := omit,

) runs on NR5GC_PTC

{ // Split function in 2 so can re-use for L2 reconfig without NAS message @sic R5-196745 sic@

// Split again so list of DRBs can be built elsewhere @sic R5-199103 sic@

var template (value) RadioBearerConfig v_RadioBearerConfig;

var template (value) CellGroupConfig v_CellGroupConfig;

var NR_RadioBearerList_Type v_SS_Config;

fl_NR_BuildRRCReconfigExistingDRBs (v_RadioBearerConfig, v_CellGroupConfig, v_SS_Config, p_CellId, p_RLCConfigList, p_MACConfig, p_DRBToAddMod, p_TimingInfo, p_SS_Drb_Config, p_SRBs);

f_ReconfigureDRBs (p_CellId, p_NASMsg, v_RadioBearerConfig, v_CellGroupConfig, v_SS_Config, -, p_TimingInfo, p_ReceiveForRRCreconfigurationComplete, p_NR_RoutingInfoSUL);

}

After change:

function f_NR_RRCReconfigExistingDRBs(NR_CellId_Type p_CellId,

template (omit) NG_NAS_MSG_RequestList_Type p_NASMsg,

template (omit) NR_RLC_BearerToAddModList_Type p_RLCConfigList := omit,

template (omit) MAC_CellGroupConfig p_MACConfig := omit,

template (omit) DRB_ToAddModList p_DRBToAddMod := omit,

template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now,

template (omit) NR_RadioBearerList_Type p_SS_Drb_Config := omit,

boolean p_ReceiveForRRCreconfigurationComplete := true,

SRBs_AlreadyConfigured p_SRBs := SRB1,

template (omit) NR_RoutingInfoSUL_Type p_NR_RoutingInfoSUL := omit,

boolean  p_MAC_config := false) runs on NR5GC_PTC

{ // Split function in 2 so can re-use for L2 reconfig without NAS message @sic R5-196745 sic@

// Split again so list of DRBs can be built elsewhere @sic R5-199103 sic@

var template (value) RadioBearerConfig v_RadioBearerConfig;

var template (value) CellGroupConfig v_CellGroupConfig;

var NR_RadioBearerList_Type v_SS_Config;

var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig;

var template(value) SpCellConfig v_SpCellConfig := f_NR_GetSpCellConfigDef(p_CellId);

if ( p_MAC_config == true){

v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(p_CellId);

fl_NR_BuildRRCReconfigExistingDRBs (v_RadioBearerConfig, v_CellGroupConfig, v_SS_Config, p_CellId, p_RLCConfigList, v_MAC_CellGroupConfig, p_DRBToAddMod, p_TimingInfo, p_SS_Drb_Config, p_SRBs)

v_CellGroupConfig.spCellConfig := v_SpCellConfig;

v_CellGroupConfig.physicalCellGroupConfig:=  cs_38508_PhysicalCellGroupConfig(23);

}else{

fl_NR_BuildRRCReconfigExistingDRBs (v_RadioBearerConfig, v_CellGroupConfig, v_SS_Config, p_CellId, p_RLCConfigList, p_MACConfig, p_DRBToAddMod, p_TimingInfo, p_SS_Drb_Config, p_SRBs);

}

f_ReconfigureDRBs (p_CellId, p_NASMsg, v_RadioBearerConfig, v_CellGroupConfig, v_SS_Config, -, p_TimingInfo, p_ReceiveForRRCreconfigurationComplete, p_NR_RoutingInfoSUL,p_Special);

}