3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210175

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1321 | rev | 2 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NR5GC_PDUSessionEstablishment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-10-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN impementation of this function, the timer t_Wait is set to 10s, which is in-line with 38.508-1 Table 4.5A.2.2.2-2, Step 1. For the UEs that trigger a PDU Session Establishment REQ automatically after Registration is done, this timer is fine. For the UEs that need a manual AT command to trigger the PDU Session Establishment REQ, this duration of 10s is too short. By the time the operator is done with typing/copy-paste of the AT command this timer expires causing testcase failure. To avoid this from happening, it is proposed that this timer be extended to 30s instead. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed the value of t_Wait to 30s. A Prose CR will be raised for this. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testcase will fail a conformant UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All SA testcases when run manually |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.508-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	3

1.1	function f_EUTRA38_TAUReqFromN1	3

# Corrections required

## function f_NR5GC_PDUSessionEstablishment

| Function name | f_NR5GC_PDUSessionEstablishment |
| --- | --- |
| Reason for change | In the current TTCN impementation of this function, the timer t_Wait is set to 10s, which is in-line with 38.508-1 Table 4.5A.2.2.2-2, Step 1. For the UEs that trigger a PDU Session Establishment REQ automatically after Registration isdone, this timer is fine. For the UEs that need a manual AT command to trigger the PDU Session Establishment REQ, this duration of 10s is too short. By the time the operator is done with typing/copy-paste of the AT command this timer expires causing testcase failure. To avoid this from happening, it is proposed that this timer be extended to 30s instead. |
| Summary of change | Changed the value of t_Wait to 30s. A Prose CR will be raised for this. |
| TTCN module | \Common\NR5GC\NR5GC_NASSteps.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> var boolean v_RxdMsg; var default v_MyDefaultVar := null; timer t_Wait := 10.0; // Clear store of msgs received in parallel f_NR5GC_MsgInDefault_ResetStoreOfPDUSessionEstReq(); <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> var boolean v_RxdMsg; var default v_MyDefaultVar := null; timer t_Wait := 30.0; //WA#R&S // Clear store of msgs received in parallel f_NR5GC_MsgInDefault_ResetStoreOfPDUSessionEstReq(); <<SKIPPED CODE>>