3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210176

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1340 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 9.3.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-10-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - In the current TTCN impementation of testcase 9.3.1.2, fl_TC_9_3_1_2_NR5GC_TestBodyf_NR5GC_IRAT_S1ToN1_IdleModef_NR5GC_IRAT_MobilityRegistration is calld, in which the rrc connection is NOT released. Within the function f_NR5GC_IRAT_MobilityRegistration only the Registration (Mobility) is done, but SRB2 is never configured. After this, a timer is started to capture the PDU Session Modification REQ on same rrc connection for 10s. Here, the PDU Session Modification REQ is expected on SRB2, but as explained above SRB2 is not configured yet. Thus the UE will send the PDU Session Modification REQ on SRB1, not SRB2. TTCN needs to be corrected for this. - Similarly, when the flow is fl_TC_9_3_1_2_NR5GC_TestBodyf_NR5GC_IRAT_S1ToN1_IdleMode f_NR5GC_IRAT_S1ToN1_SessionModification,the PDU Session Modification REQ is expected on SRB2, but as explained above SRB2 is not configured yet. This also needs to be corrected. - Later in the function f_NR5GC_IRAT_S1ToN1_SessionModification f_NR5GC_MobileInfo_AddPDUSession(valueof(v_GSM_MobilityInfo)); is called. It is incorrect to use valueof here. This needs to be corrected. - After this , in the function f_NR5GC_IRAT_S1ToN1_SessionModification f_Get_NG_PDUSessionModificationCommand is called. In this function f_Get_NG_PDUSessionModificationCommand, the condition isvalue(p_GSM_MobilityInfo) is used to assign PTI value. This is incorrect. If even one parameter within p_GSM_MobilityInfo is set to omit, this condition will return false and the a PTI value different from what was received in the PDU Session Modification REQ will be assigned for the PDU Session Modification Command. This is incorrect, and needs to be adressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Instead of expecting PDU Session Modification REQ on SRB2, changed the TTCN to expect it on SRB1. - Instead of expecting PDU Session Modification REQ on SRB2, changed the TTCN to expect it on SRB1. - Instead of calling f_NR5GC_IRAT_S1ToN1_SessionModification f_NR5GC_MobileInfo_AddPDUSession(valueof(v_GSM_MobilityInfo)); called f_NR5GC_IRAT_S1ToN1_SessionModification f_NR5GC_MobileInfo_AddPDUSession(v_GSM_MobilityInfo); - In the function f_NR5GC_PDUSessionEstablishmentf_NR5GC_PDUSessionEstablishment_InnerLoop instead of if (isvalue(p_GSM_MobilityInfo)) { // Modification was triggered by UE sending Request message v_PDU_SessionId := p_GSM_MobilityInfo.SessionId; v_PTI := valueof(p_GSM_MobilityInfo.PTI); if (isvalue(p_GSM_MobilityInfo.AlwaysOn)) { v_AlwaysOn := cs_AlwaysOnNotAllowed; } } Modified the code to : if (ispresent(p_GSM_MobilityInfo)) { // Modification was triggered by UE sending Request message v_PDU_SessionId := p_GSM_MobilityInfo.SessionId; v_PTI := valueof(p_GSM_MobilityInfo.PTI); if (isvalue(p_GSM_MobilityInfo.AlwaysOn)) { v_AlwaysOn := cs_AlwaysOnNotAllowed; } } Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testcase will fail a conformant UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.2 and other IRAT testcases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	4

1	Corrections required	5

1.1	function f_NR5GC_IRAT_S1ToN1_IdleMode	5

1.2	function f_NR5GC_IRAT_S1ToN1_SessionModification	8

1.3	function f_Get_NG_PDUSessionModificationCommand	13

# Corrections required

## function f_NR5GC_IRAT_S1ToN1_IdleMode

| Function name | f_NR5GC_IRAT_S1ToN1_IdleMode |
| --- | --- |
| Reason for change | In the current TTCN impementation of testcase 9.3.1.2, fl_TC_9_3_1_2_NR5GC_TestBodyf_NR5GC_IRAT_S1ToN1_IdleModef_NR5GC_IRAT_MobilityRegistration is called, in which the rrc connection is NOT released. Within the function f_NR5GC_IRAT_MobilityRegistration only the Registration (Mobility) is done, but SRB2 is never configured. After this, a timer is started to capture the PDU Session Modification REQ on same rrc connection for 10s. Here, the PDU Session Modification REQ is expected on SRB2, but as explained above SRB2 is not configured yet. Thus the UE will send the PDU Session Modification REQ on SRB1, not SRB2. TTCN needs to be corrected for this. |
| Summary of change | Instead of expecting PDU Session Modification REQ on SRB2, changed the TTCN to expect it on SRB1. |
| TTCN module | \NR5GC_IRAT\Commmon\NR5GC_IRAT_CommonFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> t_Wait.start; // @sic R5-204399 sic@ alt { // @sic R5-204401 sic@ [] SRB.check(receive(car_NR_SRB_NasPdu_IND(p_CellId, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp ) { t_Wait.stop; if (ischosen(v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Modification_Request)) { if (p_FirstTimeS1ToN1 and p_IwkN26 == NOT_SUPPORTED) { f_NR5GC_IRAT_S1ToN1_SessionModification(p_CellId, v_PduSessionInfoList, false); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Not expecting PDU Session Modification message"); } t_Wait.start; // restart timer in case there are now PDU Establishment messages repeat; } else if (ischosen(v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request)) { t_Wait.stop; f_NR5GC_PDUSessionEstablishment (p_CellId, pc_noOf_PDUsSameConnection, tsc_NR_RbId_SRB1, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, "", v_FailWhenK_LessN); v_SRB := tsc_NR_RbId_SRB2; // @sic R5-204401 sic@ FFS: This may be temporary, depending on further updates to PDU session modification/establishment procedures // @sic R5-204401 sic@ FFS: haven't implemented option where Modification messages arrive during or after PDU Establishment procedure started if (pc_noOf_PDUsNewConnection >0 ){ f_NR_RRCRelease(p_CellId); f_NR5GC_RRC_Idle_Extension(p_CellId, TESTMode_OFF, noRrcConnectionRelease); } } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Wrong NAS Message Received"); } } [] t_Wait.timeout { // Do nothing as no (more) PDU Modification/Establishment messages received in the last 10s, so carry on } }<<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> t_Wait.start; // @sic R5-204399 sic@ alt { // @sic R5-204401 sic@ [] SRB.check(receive(car_NR_SRB_NasPdu_IND(p_CellId, tsc_NR_RbId_SRB1 /*WA#9_3_1_2 tsc_NR_RbId_SRB2*/, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp ) { t_Wait.stop; if (ischosen(v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Modification_Request)) { if (p_FirstTimeS1ToN1 and p_IwkN26 == NOT_SUPPORTED) { f_NR5GC_IRAT_S1ToN1_SessionModification(p_CellId, v_PduSessionInfoList, false); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Not expecting PDU Session Modification message"); } t_Wait.start; // restart timer in case there are now PDU Establishment messages repeat; } else if (ischosen(v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg.pdu_Session_Establishment_Request)) { t_Wait.stop; f_NR5GC_PDUSessionEstablishment (p_CellId, pc_noOf_PDUsSameConnection, tsc_NR_RbId_SRB1, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, "", v_FailWhenK_LessN); v_SRB := tsc_NR_RbId_SRB2; // @sic R5-204401 sic@ FFS: This may be temporary, depending on further updates to PDU session modification/establishment procedures // @sic R5-204401 sic@ FFS: haven't implemented option where Modification messages arrive during or after PDU Establishment procedure started if (pc_noOf_PDUsNewConnection >0 ){ f_NR_RRCRelease(p_CellId); f_NR5GC_RRC_Idle_Extension(p_CellId, TESTMode_OFF, noRrcConnectionRelease); } } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Wrong NAS Message Received"); } } [] t_Wait.timeout { // Do nothing as no (more) PDU Modification/Establishment messages received in the last 10s, so carry on } } <<SKIPPED CODE>>

## function f_NR5GC_IRAT_S1ToN1_SessionModification

| Function name | f_NR5GC_IRAT_S1ToN1_SessionModification |
| --- | --- |
| Reason for change | This change is a follow on from Change 1.1 above. when the flow is fl_TC_9_3_1_2_NR5GC_TestBodyf_NR5GC_IRAT_S1ToN1_IdleMode f_NR5GC_IRAT_S1ToN1_SessionModification,the PDU Session Modification REQ is expected on SRB2, but as explained above SRB2 is not configured yet. This needs to be corrected. Later in the function f_NR5GC_IRAT_S1ToN1_SessionModification f_NR5GC_MobileInfo_AddPDUSession(valueof(v_GSM_MobilityInfo)); is called. It is incorrect to use valueof here. This needs to be corrected. Also, the argument p_ExpectedNumberOfNewPDUSessions is by default assigned (pc_noOf_PDNsSameConnection + pc_noOf_PDNsNewConnection). This needs to be changed to (pc_noOf_PDUsSameConnection + pc_noOf_PDUsNewConnection |
| Summary of change | Instead of expecting PDU Session Modification REQ on SRB2, changed the TTCN to expect it on SRB1. Instead of calling f_NR5GC_IRAT_S1ToN1_SessionModification f_NR5GC_MobileInfo_AddPDUSession(valueof(v_GSM_MobilityInfo)); called f_NR5GC_IRAT_S1ToN1_SessionModification f_NR5GC_MobileInfo_AddPDUSession(v_GSM_MobilityInfo); Changed the parameters as explained above. Please see below |
| TTCN module | \NR5GC_IRAT\Commmon\NR5GC_IRAT_CommonFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR5GC_IRAT_S1ToN1_SessionModification(NR_CellId_Type p_CellId, EUTRA_NR_PduSessionInfoList_Type p_PduSessionsFromLTE, boolean p_FailWhenK_LessN := true, integer p_ExpectedNumberOfNewPDUSessions := (pc_noOf_PDNsSameConnection + pc_noOf_PDNsNewConnection)) runs on NR5GC_PTC return boolean { var template (value) PDUSessionInfoList_Type v_FromLTEList; var NG_NAS_UL_PduList_Type v_PduSessionModificationRequestList := {}; var IntegerList_Type v_PDU_SessionIdList := {}; var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_UL_Pdu_Type v_NG_NAS_UL_Pdu; var NG_NAS_UL_Message_Type v_GSMMsg; var integer v_PDU_SessionId; var template (value) RadioBearerConfig v_RadioBearerConfig; var template(value) CellGroupConfig v_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_RadioBearerList; // list of DRBs to be configured at the SS (SRB2 alredy configured in SS at cell creation/RRC release procedure => not needed here) var template (value) SDAP_ConfigurationList_Type v_SDAP_ConfigurationList; var boolean v_ReregisterIMS := false; var template (value) NG_NAS_DL_Pdu_Type v_NG_NAS_DL_Pdu; var NR_DRBInfoList_Type v_NewDRBInfoList; var integer N := p_ExpectedNumberOfNewPDUSessions; // number of PDU SESSION MODIFICATION REQUEST messages being expected var integer K := 0; // number of PDU SESSION MODIFICATION REQUEST messages already processed (i.e. the corresponding PDU SESSION MODIFICATION COMMAND and PDU SESSION MODIFICATION COMPLETE have been sent/received) var integer L := 0; // number of PDU SESSION MODIFICATION REQUEST messages being received var boolean v_WaitForComplete := false; var template (present) NG_UE_SM_Cap v_NG_UE_SM_Cap := ?; // according to 38.508-1 Table 4.5A.2C.2.3-1 var template (present) IntegrityProtMaxDataRate v_IntegrityProtMaxDataRate := ?; // according to 38.508-1 Table 4.5A.2C.2.3-1 var integer i; timer t_Wait := 10.0; // @sic R5-204330 sic@ // Build the GSMMobilityInfo for all received PDUs, but don't store it (some may be established, rather than modified) for (i := 0; i < N; i := i + 1) { v_FromLTEList[i] := cs_GSM_MobilityInfo(int2oct(p_PduSessionsFromLTE[i].PDU_SessionId, 1), -, p_PduSessionsFromLTE[i].IsIMS); // @sic R5s200564 sic@ } t_Wait.start; // @sic R5-204330 sic@ while (K < N) { alt { // @sic R5-204330 sic@ [] SRB.receive(car_NR_SRB_NasPdu_IND(p_CellId, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { v_NG_NAS_UL_Pdu := v_ReceivedAsp.Signalling.Nas[0].Pdu; v_GSMMsg := v_NG_NAS_UL_Pdu.PiggybackedPduList[0].Msg; if (ischosen(v_GSMMsg.pdu_Session_Modification_Request)) { t_Wait.stop; v_PduSessionModificationRequestList[L] := v_NG_NAS_UL_Pdu; L := L + 1; if (L > N) { // @sic R5-204330 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "L > N"); } } else if (v_WaitForComplete and ischosen(v_GSMMsg.pdu_Session_Modification_Complete)) { if (not f_Check_NG_PDUSessionModificationComplete(v_NG_NAS_UL_Pdu, v_GSM_MobilityInfo.SessionId, v_GSM_MobilityInfo.PTI)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Invalid PDU SESSION MODIFICATION COMPLETE"); } v_WaitForComplete := false; K := K + 1; } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Unexpected NAS PDU"); } if (not v_WaitForComplete and (K < L)) { // => process the next PDU SESSION MODIFICATION REQUEST v_NG_NAS_UL_Pdu := v_PduSessionModificationRequestList[K]; if (not f_Check_NG_PDUSessionModificationReq(v_GSM_MobilityInfo, valueof(v_FromLTEList), v_NG_NAS_UL_Pdu, -, -, v_NG_UE_SM_Cap, -, -, -, v_IntegrityProtMaxDataRate, -, -, -, -, -, cr_NG_Request_Type('101'B))) { // v_GSM_MobilityInfo is out parameter of f_Check_NG_PDUSessionModificationReq f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Invalid PDU SESSION MODIFICATION REQUEST"); } // ok, this PDU is to be modified, so can now store it in our list v_PDU_SessionId := oct2int(v_GSM_MobilityInfo.SessionId); // @sic R5s200564 sic@ if (f_IntegerList_Search(v_PDU_SessionIdList, v_PDU_SessionId) < 0) { v_PDU_SessionIdList[K] := v_PDU_SessionId; } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "There has been a PDU SESSION MODIFICATION REQUEST with the same session id already"); } f_NR5GC_MobileInfo_AddPDUSession(valueof(v_GSM_MobilityInfo)); for (i := 0; i < N; i := i + 1) { if (p_PduSessionsFromLTE[i].PDU_SessionId == oct2int(v_GSM_MobilityInfo.SessionId)) { f_IP_ChangeDrbMappingNR(IP, p_CellId, cs_IP_QosFlowId(p_PduSessionsFromLTE[i].PDU_SessionId, p_PduSessionsFromLTE[i].QFI), p_PduSessionsFromLTE[i].PdnIndex); v_ReregisterIMS := v_ReregisterIMS or p_PduSessionsFromLTE[i].IsIMS; break; } } <<SKIPPED CODE>>

After Change:

function f_NR5GC_IRAT_S1ToN1_SessionModification(NR_CellId_Type p_CellId, EUTRA_NR_PduSessionInfoList_Type p_PduSessionsFromLTE, boolean p_FailWhenK_LessN := true, //WA#9_3_1_2 integer p_ExpectedNumberOfNewPDUSessions := (pc_noOf_PDNsSameConnection + pc_noOf_PDNsNewConnection)) runs on NR5GC_PTC return boolean integer p_ExpectedNumberOfNewPDUSessions := (pc_noOf_PDUsSameConnection + pc_noOf_PDUsNewConnection)) runs on NR5GC_PTC return boolean { var template (value) PDUSessionInfoList_Type v_FromLTEList; var NG_NAS_UL_PduList_Type v_PduSessionModificationRequestList := {}; var IntegerList_Type v_PDU_SessionIdList := {}; var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_UL_Pdu_Type v_NG_NAS_UL_Pdu; var NG_NAS_UL_Message_Type v_GSMMsg; var integer v_PDU_SessionId; var template (value) RadioBearerConfig v_RadioBearerConfig; var template(value) CellGroupConfig v_CellGroupConfig; var template (value) NR_RadioBearerList_Type v_RadioBearerList; // list of DRBs to be configured at the SS (SRB2 alredy configured in SS at cell creation/RRC release procedure => not needed here) var template (value) SDAP_ConfigurationList_Type v_SDAP_ConfigurationList; var boolean v_ReregisterIMS := false; var template (value) NG_NAS_DL_Pdu_Type v_NG_NAS_DL_Pdu; var NR_DRBInfoList_Type v_NewDRBInfoList; var integer N := p_ExpectedNumberOfNewPDUSessions; // number of PDU SESSION MODIFICATION REQUEST messages being expected var integer K := 0; // number of PDU SESSION MODIFICATION REQUEST messages already processed (i.e. the corresponding PDU SESSION MODIFICATION COMMAND and PDU SESSION MODIFICATION COMPLETE have been sent/received) var integer L := 0; // number of PDU SESSION MODIFICATION REQUEST messages being received var boolean v_WaitForComplete := false; var template (present) NG_UE_SM_Cap v_NG_UE_SM_Cap := ?; // according to 38.508-1 Table 4.5A.2C.2.3-1 var template (present) IntegrityProtMaxDataRate v_IntegrityProtMaxDataRate := ?; // according to 38.508-1 Table 4.5A.2C.2.3-1 var integer i; timer t_Wait := 10.0; // @sic R5-204330 sic@ // Build the GSMMobilityInfo for all received PDUs, but don't store it (some may be established, rather than modified) for (i := 0; i < N; i := i + 1) { v_FromLTEList[i] := cs_GSM_MobilityInfo(int2oct(p_PduSessionsFromLTE[i].PDU_SessionId, 1), -, p_PduSessionsFromLTE[i].IsIMS); // @sic R5s200564 sic@ } t_Wait.start; // @sic R5-204330 sic@ while (K < N) { alt { // @sic R5-204330 sic@ [] SRB.receive(car_NR_SRB_NasPdu_IND(p_CellId, tsc_NR_RbId_SRB1 /*WA#9_3_1_2 tsc_NR_RbId_SRB2*/, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { v_NG_NAS_UL_Pdu := v_ReceivedAsp.Signalling.Nas[0].Pdu; v_GSMMsg := v_NG_NAS_UL_Pdu.PiggybackedPduList[0].Msg; if (ischosen(v_GSMMsg.pdu_Session_Modification_Request)) { t_Wait.stop; v_PduSessionModificationRequestList[L] := v_NG_NAS_UL_Pdu; L := L + 1; if (L > N) { // @sic R5-204330 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "L > N"); } } else if (v_WaitForComplete and ischosen(v_GSMMsg.pdu_Session_Modification_Complete)) { if (not f_Check_NG_PDUSessionModificationComplete(v_NG_NAS_UL_Pdu, v_GSM_MobilityInfo.SessionId, v_GSM_MobilityInfo.PTI)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Invalid PDU SESSION MODIFICATION COMPLETE"); } v_WaitForComplete := false; K := K + 1; } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Unexpected NAS PDU"); } if (not v_WaitForComplete and (K < L)) { // => process the next PDU SESSION MODIFICATION REQUEST v_NG_NAS_UL_Pdu := v_PduSessionModificationRequestList[K]; if (not f_Check_NG_PDUSessionModificationReq(v_GSM_MobilityInfo, valueof(v_FromLTEList), v_NG_NAS_UL_Pdu, -, -, v_NG_UE_SM_Cap, -, -, -, v_IntegrityProtMaxDataRate, -, -, -, -, -, cr_NG_Request_Type('101'B))) { // v_GSM_MobilityInfo is out parameter of f_Check_NG_PDUSessionModificationReq f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Invalid PDU SESSION MODIFICATION REQUEST"); } // ok, this PDU is to be modified, so can now store it in our list v_PDU_SessionId := oct2int(v_GSM_MobilityInfo.SessionId); // @sic R5s200564 sic@ if (f_IntegerList_Search(v_PDU_SessionIdList, v_PDU_SessionId) < 0) { v_PDU_SessionIdList[K] := v_PDU_SessionId; } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "There has been a PDU SESSION MODIFICATION REQUEST with the same session id already"); } //WA#9_3_1_2 f_NR5GC_MobileInfo_AddPDUSession(valueof(v_GSM_MobilityInfo)); f_NR5GC_MobileInfo_AddPDUSession(v_GSM_MobilityInfo); //WA#9_3_1_2 for (i := 0; i < N; i := i + 1) { if (p_PduSessionsFromLTE[i].PDU_SessionId == oct2int(v_GSM_MobilityInfo.SessionId)) { f_IP_ChangeDrbMappingNR(IP, p_CellId, cs_IP_QosFlowId(p_PduSessionsFromLTE[i].PDU_SessionId, p_PduSessionsFromLTE[i].QFI), p_PduSessionsFromLTE[i].PdnIndex); v_ReregisterIMS := v_ReregisterIMS or p_PduSessionsFromLTE[i].IsIMS; break; } } <<SKIPPED CODE>>

## function f_Get_NG_PDUSessionModificationCommand

| Function name | f_Get_NG_PDUSessionModificationCommand |
| --- | --- |
| Reason for change | In the function f_NR5GC_IRAT_S1ToN1_SessionModification f_Get_NG_PDUSessionModificationCommand is called. In this function f_Get_NG_PDUSessionModificationCommand, the condition isvalue(p_GSM_MobilityInfo) is used to assign PTI value. This is incorrect. If even one parameter within p_GSM_MobilityInfo is set to omit, this condition will return false and the a PTI value different from what was received in the PDU Session Modification REQ will be assigned for the PDU Session Modification Command. This is incorrect, and needs to be adressed. |
| Summary of change | In the function f_NR5GC_PDUSessionEstablishmentf_NR5GC_PDUSessionEstablishment_InnerLoop instead of if (isvalue(p_GSM_MobilityInfo)) { // Modification was triggered by UE sending Request message v_PDU_SessionId := p_GSM_MobilityInfo.SessionId; v_PTI := valueof(p_GSM_MobilityInfo.PTI); if (isvalue(p_GSM_MobilityInfo.AlwaysOn)) { v_AlwaysOn := cs_AlwaysOnNotAllowed; } } Modified the code to : if (ispresent(p_GSM_MobilityInfo)) { // Modification was triggered by UE sending Request message v_PDU_SessionId := p_GSM_MobilityInfo.SessionId; v_PTI := valueof(p_GSM_MobilityInfo.PTI); if (isvalue(p_GSM_MobilityInfo.AlwaysOn)) { v_AlwaysOn := cs_AlwaysOnNotAllowed; } } Please see below. |
| TTCN module | Common\NG_NAS\NG_NASTemplateFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> if (isvalue(p_GSM_MobilityInfo)) { // Modification was triggered by UE sending Request message v_PDU_SessionId := p_GSM_MobilityInfo.SessionId; v_PTI := valueof(p_GSM_MobilityInfo.PTI); if (isvalue(p_GSM_MobilityInfo.AlwaysOn)) { v_AlwaysOn := cs_AlwaysOnNotAllowed; } } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> //WA#9_3_1_2 if (isvalue(p_GSM_MobilityInfo)) { // Modification was triggered by UE sending Request message if (ispresent(p_GSM_MobilityInfo)) { // Modification was triggered by UE sending Request message v_PDU_SessionId := p_GSM_MobilityInfo.SessionId; v_PTI := valueof(p_GSM_MobilityInfo.PTI); if (isvalue(p_GSM_MobilityInfo.AlwaysOn)) { v_AlwaysOn := cs_AlwaysOnNotAllowed; } } <<SKIPPED CODE>>