3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210177

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0648 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS 5GS SMS test case 9.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd, Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-10-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To add IMS 5GS SMS test case 9.3 to the IMS/NR5GC ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to 5GS IMS SMS test case 9.3 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

3	Execution Log Files	5

3.1	Qualcomm SM8350	5

4	References	6

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of test case 9.3 which is part of the IMS NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Narendra Kalahasti

narendra.kalahasti@anritsu.com

Mohit Kanchan

mohit.kanchan@keysight.com

## Verification Test Summary

Test Case: 	9.3

ATS Version:	iwd-TTCN3-B2019-06_D20wk38

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

Keysight 5G Protocol Conformance Toolset

UE used:	Qualcomm SM8350

Verification Status:	PASS

# Corrections required

In addition to the below changes, the changes of R5s201327 and change 1 of R5s201329 are required and applied.

## 2.1 Change 1

| Function name | f_IMS_NR5GC_MobileOriginating_ConcSMS() |
| --- | --- |
| Reason for change | From the conformance requirements (23.040 0.2.3.24): If 7 bit data is used and the TP UD Header does not finish on a septet boundary then fill bits are inserted after the last Information Element Data octet up to the next septet boundary so that there is an integral number of septets for the entire TP UD header. This is to ensure that the SM itself starts on an septet boundary so that an earlier Phase mobile shall be capable of displaying the SM itself although the TP UD Header in the TP UD field may not be understood. Here the expected TP UD header (in template cr_TP_UserDataHeader_ConcSMS) consists of 6 octects = 48 bits. So, the UE will insert 1 fill bit so that the SMS text starts at bit 49 (septet boundary). The message data needs to allow for this. |
| Summary of change | Pass 1-bit string of fill bits into the packing function for the expected message data (following change 1 of R5s201329). |
| TTCN module | IMS_NR5GC_CC_TestCaseImplementation_9.ttcn |
| MCC160 Comment |  |

Before

function f_IMS_NR5GC_MobileOriginating_ConcSMS(template (present) RP_DATA_ul_Type p_SMS_RP_DATA_ul_SUBMIT_IMS,

template (present) O1_Type p_MsgRef,

O1_Type p_SN_SMS) runs on IMS_PTC return O1_Type

{

var IMS_DATA_REQ v_IMS_DATA_REQ;

var MESSAGE_Request v_MESSAGE_RequestRX;

var octetstring v_EncodedSMS;

var O1_Type v_MsgRef;

var RP_DATA_ul_Type v_RP_DATA_ul;

var RP_UserData v_RP_UserData_SUBMIT;

var octetstring v_EncodedUDH;

var TP_UserDataHeader_Type v_MoUDH;

// @siclog "Step 1 resp 2" siclog@

// Step 1, 5, 9 UE sends a short message FFS segment

// Check: Does the UE send a SIP MESSAGE request including a vnd.3gpp.sms payload that contains the first/second/third segment of the concatenated SMS?

v_IMS_DATA_REQ := f_IMS_NR5GC_MO_SMS_ConcSMS(p_SMS_RP_DATA_ul_SUBMIT_IMS); // 1st, 2nd, 3rd SMS segment passed

v_MESSAGE_RequestRX := v_IMS_DATA_REQ.Request.Message;

// Decode and check received SMS message

v_EncodedSMS := v_MESSAGE_RequestRX.messageBody.smsMessage;

v_RP_DATA_ul := f_SMS_PDU_DecodeAndMatchRPDATA(v_EncodedSMS, p_SMS_RP_DATA_ul_SUBMIT_IMS);

v_RP_UserData_SUBMIT := v_RP_DATA_ul.rP_UserData;

// Decode and check received User Data Header, note first MsgRef used by UE

v_EncodedUDH := v_RP_UserData_SUBMIT.tP_PDU.SMS_SUBMIT.tP_UserData;

v_MoUDH := f_TP_UserDataWithHeader_DecodeAndMatch (v_EncodedUDH, cr_TP_UserDataHeader_ConcSMS(p_MsgRef, p_SN_SMS, f_CharPacking_IA5toGsm7Bit(tsc_FoxShort, SMS_Packing)));

v_MsgRef := v_MoUDH.tP_UDH_IE_Record[0].tP_UDH.tP_UDH_ConcSMS.tP_UserDataHeaderMsgRef; // Msg Reference chosen by the UE for the Concatenated messsage

f_IMS_PreliminaryPass(__FILE__, __LINE__, "UE sends SIP Message containing a short message");

// Step 2-4, 6-8, 10-12 of 9.3 resp 3-5, 7-9, 11-13 of 18.2a

f_IMS_NR5GC_MO_SMS_Step2to4 (v_IMS_DATA_REQ);

return v_MsgRef;

}

After

function f_IMS_NR5GC_MobileOriginating_ConcSMS(template (present) RP_DATA_ul_Type p_SMS_RP_DATA_ul_SUBMIT_IMS,

template (present) O1_Type p_MsgRef,

O1_Type p_SN_SMS) runs on IMS_PTC return O1_Type

{

var IMS_DATA_REQ v_IMS_DATA_REQ;

var MESSAGE_Request v_MESSAGE_RequestRX;

var octetstring v_EncodedSMS;

var O1_Type v_MsgRef;

var RP_DATA_ul_Type v_RP_DATA_ul;

var RP_UserData v_RP_UserData_SUBMIT;

var octetstring v_EncodedUDH;

var TP_UserDataHeader_Type v_MoUDH;

// @siclog "Step 1 resp 2" siclog@

// Step 1, 5, 9 UE sends a short message FFS segment

// Check: Does the UE send a SIP MESSAGE request including a vnd.3gpp.sms payload that contains the first/second/third segment of the concatenated SMS?

v_IMS_DATA_REQ := f_IMS_NR5GC_MO_SMS_ConcSMS(p_SMS_RP_DATA_ul_SUBMIT_IMS); // 1st, 2nd, 3rd SMS segment passed

v_MESSAGE_RequestRX := v_IMS_DATA_REQ.Request.Message;

// Decode and check received SMS message

v_EncodedSMS := v_MESSAGE_RequestRX.messageBody.smsMessage;

v_RP_DATA_ul := f_SMS_PDU_DecodeAndMatchRPDATA(v_EncodedSMS, p_SMS_RP_DATA_ul_SUBMIT_IMS);

v_RP_UserData_SUBMIT := v_RP_DATA_ul.rP_UserData;

// Decode and check received User Data Header, note first MsgRef used by UE

v_EncodedUDH := v_RP_UserData_SUBMIT.tP_PDU.SMS_SUBMIT.tP_UserData;

v_MoUDH := f_TP_UserDataWithHeader_DecodeAndMatch (v_EncodedUDH, cr_TP_UserDataHeader_ConcSMS(p_MsgRef, p_SN_SMS, f_CharPacking_IA5toGsm7Bit(tsc_FoxShort, SMS_Packing), ‘0’B));

v_MsgRef := v_MoUDH.tP_UDH_IE_Record[0].tP_UDH.tP_UDH_ConcSMS.tP_UserDataHeaderMsgRef; // Msg Reference chosen by the UE for the Concatenated messsage

f_IMS_PreliminaryPass(__FILE__, __LINE__, "UE sends SIP Message containing a short message");

// Step 2-4, 6-8, 10-12 of 9.3 resp 3-5, 7-9, 11-13 of 18.2a

f_IMS_NR5GC_MO_SMS_Step2to4 (v_IMS_DATA_REQ);

return v_MsgRef;

}

# Execution Log Files

## Qualcomm SM8350

The Qualcomm SM8350 5G IMS UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR and Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu/TC_9_3_LOG.xml

Keysight/TC_9_3_LOG.html

In the log file (in .html format) the complete test case execution can be seen using band n41. All TLI events are presented and message contents are fully decoded and can be verified. Preliminary verdicts and the final test case verdict can be seen in the log file.

- PICS/PIXIT parameter file:

Anritsu/TC_9_3_PIXIT.txt

Keysight/TC_9_3_PIXIT.xml

# References

| [1] | R5s201447 Supporting information for addition of IMS 5GS SMS test case 9.3 |
| --- | --- |