GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210178

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1367 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC test case 6.3.1.7 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Datang LinkTester |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-10-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add Rel-15 NR5GC test case 6.3.1.7 to the NR5GC ATS |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC test case 6.3.1.7 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.7.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	7

2.4	Change 4	8

3	Branches executed	9

4	Execution Log Files	9

4.1	HiSilicon Balong 5000	9

5	References	10

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 6.3.1.7 which is part of the NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Fu Wang
	wangfu@datangmobile.cn

## Verification Test Summary

Test Case: 	6.3.1.7

ATS Version:	iwd-TTCN3-B2019-06_D20wk38

System Simulator used:	Datang LinkTester 5G Protocol Conformance Test System ECT9610

UE used:	HiSilicon Balong 5000

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_6_3_1_7_NR5GC |
| --- | --- |
| Reason for change | - NR cell needs to support ims-emergency. |
| Summary of change | - Use function f_NR_CellInfo_SetSIB1_ims_EmergencySupport to configure IE ims-EmergencySupport of sib1. |
| TTCN module | SOR_NR5GC.ttcn |
| MCC160 Comment |  |

Before change

function f_TC_6_3_1_7_NR5GC() runs on NR5GC_PTC

{ //Steering of UE in roaming during registration/security check unsuccessful but emergency service pending to be activated

//@sic R5-204410 sic@ Cell change

f_NR5GC_Init_NAS(NR_4);//need to update to the default configuration

f_NR_CellInfo_SetPLMN(nr_Cell11, tsc_NR_HPLMN_002_11);//@sic R5-204410 sic@

//Create and configure cell

f_NR_CellConfig_Def(nr_Cell11);

// Equip UE with the USIM configuration 10 specified in table 6.4.1-10 TS 38.508-1

f_UT_USIM_Insert(UT, "USIM configuration 10 specified in table 6.4.1-10 TS 38.508-1");

f_UT_AutomaticPLMN_Select_WithSwitchOnOff (UT);

//UE's Last Registered PLMN (RPLMN) needs to be cleared by using Registration Reject procedure as defined in TS 38.508-1 [4] Subclause 4.9.8

//@sic R5-204410 sic@

f_NR5GC_Preamble(nr_Cell11,STATE_OFF_0A);

f_NR_TestBody_Set(true);

fl_TC_6_3_1_7_TestBody();

f_NR_TestBody_Set(false);

// Finish

f_NR_Postamble(nr_Cell11, STATE_CONNECTED_3A);

}

After change

function f_TC_6_3_1_7_NR5GC() runs on NR5GC_PTC

{ //Steering of UE in roaming during registration/security check unsuccessful but emergency service pending to be activated

//@sic R5-204410 sic@ Cell change

f_NR5GC_Init_NAS(NR_4);//need to update to the default configuration

f_NR_CellInfo_SetPLMN(nr_Cell11, tsc_NR_HPLMN_002_11);//@sic R5-204410 sic@

f_NR_CellInfo_SetSIB1_ims_EmergencySupport (nr_Cell11, true_);

//Create and configure cell

f_NR_CellConfig_Def(nr_Cell11);

// Equip UE with the USIM configuration 10 specified in table 6.4.1-10 TS 38.508-1

f_UT_USIM_Insert(UT, "USIM configuration 10 specified in table 6.4.1-10 TS 38.508-1");

f_UT_AutomaticPLMN_Select_WithSwitchOnOff (UT);

//UE's Last Registered PLMN (RPLMN) needs to be cleared by using Registration Reject procedure as defined in TS 38.508-1 [4] Subclause 4.9.8

//@sic R5-204410 sic@

f_NR5GC_Preamble(nr_Cell11,STATE_OFF_0A);

f_NR_TestBody_Set(true);

fl_TC_6_3_1_7_TestBody();

f_NR_TestBody_Set(false);

// Finish

f_NR_Postamble(nr_Cell11, STATE_CONNECTED_3A);

}

## Change 2

| Function name | fl_TC_6_3_1_7_TestBody |
| --- | --- |
| Reason for change | - According to 24.501 5.5.1.2.5, when the cause of registration reject is congestion, security context should not be deleted, the registration request should be integrity protected in step 15. - SSC_Mode of pdu Session EstablishmentAccept should include IEI. |
| Summary of change | - Use function f_NR5GC_SorRegistration_6317 in step 15. - Set IEI of SSC_Mode to “A”. |
| TTCN module | SOR_NR5GC.ttcn |
| MCC160 Comment |  |

Before change

function fl_TC_6_3_1_7_TestBody() runs on NR5GC_PTC

{ //@sic R5-204410 sic@

var NAS_PlmnId v_PlmnID_2 := f_NR_Asn2Nas_PlmnId(tsc_NR_HPLMN_002_11);    //@sic R5-204410 sic@

var O2_Type v_AccessTechId :='0800'O;

var octetstring v_Data := v_PlmnID_2 & v_AccessTechId;

var template (value) SOR_Header v_Header := crs_NG_SOR_Header('1'B,'1'B,'1'B,'0'B);

//@siclog "Step 1" siclog@

//Power on the UE

//@siclog "Step 1A1-1A13" siclog@

//Steps 1 to 13 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed    //@sic R5-204410 sic@

f_NR5GC_Preamble_Steps1_13(nr_Cell11);

//@siclog "Step 1B" siclog@

//SS transmits a REGISTRATION REJECT message with cause #22 (Congestion) and T3346 set to 3 minutes.    //@sic R5-204410 sic@

SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell11,

tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1

cs_TimingInfo_Now,

cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, tsc_NR5GCCause_Congestion), cs_GPRS_Timer2_3_IEI('5F'O, '001'B, '00011'B)))));

//@siclog "Step 1C" siclog@

//The SS releases the RRC connection    //@sic R5-204410 sic@

f_NR_RRCRelease(nr_Cell11);

//@siclog "Step 2" siclog@

//The UE is made to establish an emergency PDU session. This can be done by an AT/MMI command

f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, -, true);

//@siclog "Step 3" siclog@

//The UE transmits an RRCSetupRequest on NR Cell 13 indicating emergencency registration?

SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_emergency))));

f_NR_PreliminaryPass(__FILE__, __LINE__, "RRCSetupRequest received");//@sic R5s200305 sic@

//@siclog "Steps 4-5" siclog@

//Steps 3 to 4 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed

//@siclog "Step 6" siclog@

//The UE transmit a REGISTRATION REQUEST message.5GS registration type can set to either "initial registration" according to TS 24.501 clause 5.5.1.2.6A or "emergency registration" according to TS 24.501 clause 5.5.1.2.

//@siclog "Steps 7-14" siclog@

//Steps 5 to 13 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed on NR Cell 13

//@siclog "Step 15" siclog@

//The SS transmits a DLInformationTransfer message and a REGISTRATION ACCEPT message containing

//steering of roaming information, includes unmatched SOR-MAC-IAUSF that will result in unsuccessful security check while on UE reception

f_NR5GC_SorRegistration(nr_Cell11,v_Data,v_Header,true,(Initial_NoSecurity,Emergency));

//@siclog "Step 16" siclog@

//The UE transmits an ULInformationTransfer message and REGISTRATION COMPLETE message without including an SOR transparent container

f_NR5GC_Registration_Complete(nr_Cell11);

//@siclog "Step 17" siclog@

//Check: Does the UE transmit an UL NAS TRANSPORT message with "Request type" set to "initial emergency request", and a PDU SESSION ESTABLISHMENT REQUEST message on NR Cell 13

//@siclog "Step 18" siclog@

//The SS transmits a PDU SESSION ESTABLISHMENT ACCEPT message.

f_NR5GC_PDUSessionEstablishment (nr_Cell11, pc_noOf_PDUsSameConnection, tsc_NR_RbId_SRB1,-,

omit,omit,

-,-,-,-,-,

-,-,-,-,

cs_SSC_Mode(-, '001'B),

-,-,-,-,-,-,-,

cr_NG_Request_Type('011'B));

if (pc_noOf_PDUsNewConnection >0 ){

f_NR_RRCRelease(nr_Cell11);

f_NR5GC_RRC_Idle_Extension(nr_Cell11, TESTMode_OFF, noRrcConnectionRelease);

}

}//End of fl_TC_6_3_1_7_TestBody

After change

function fl_TC_6_3_1_7_TestBody() runs on NR5GC_PTC

{ //@sic R5-204410 sic@

var NAS_PlmnId v_PlmnID_2 := f_NR_Asn2Nas_PlmnId(tsc_NR_HPLMN_002_11);    //@sic R5-204410 sic@

var O2_Type v_AccessTechId :='0800'O;

var octetstring v_Data := v_PlmnID_2 & v_AccessTechId;

var template (value) SOR_Header v_Header := crs_NG_SOR_Header('1'B,'1'B,'1'B,'0'B);

//@siclog "Step 1" siclog@

//Power on the UE

//@siclog "Step 1A1-1A13" siclog@

//Steps 1 to 13 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed    //@sic R5-204410 sic@

f_NR5GC_Preamble_Steps1_13(nr_Cell11);

//@siclog "Step 1B" siclog@

//SS transmits a REGISTRATION REJECT message with cause #22 (Congestion) and T3346 set to 3 minutes.    //@sic R5-204410 sic@

SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell11,

tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1

cs_TimingInfo_Now,

cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, tsc_NR5GCCause_Congestion), cs_GPRS_Timer2_3_IEI('5F'O, '001'B, '00011'B)))));

//@siclog "Step 1C" siclog@

//The SS releases the RRC connection    //@sic R5-204410 sic@

f_NR_RRCRelease(nr_Cell11);

//@siclog "Step 2" siclog@

//The UE is made to establish an emergency PDU session. This can be done by an AT/MMI command

f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, -, true);

//@siclog "Step 3" siclog@

//The UE transmits an RRCSetupRequest on NR Cell 13 indicating emergencency registration?

SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_emergency))));

f_NR_PreliminaryPass(__FILE__, __LINE__, "RRCSetupRequest received");//@sic R5s200305 sic@

//@siclog "Steps 4-5" siclog@

//Steps 3 to 4 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed

//@siclog "Step 6" siclog@

//The UE transmit a REGISTRATION REQUEST message.5GS registration type can set to either "initial registration" according to TS 24.501 clause 5.5.1.2.6A or "emergency registration" according to TS 24.501 clause 5.5.1.2.

//@siclog "Steps 7-14" siclog@

//Steps 5 to 13 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed on NR Cell 13

//@siclog "Step 15" siclog@

//The SS transmits a DLInformationTransfer message and a REGISTRATION ACCEPT message containing

//steering of roaming information, includes unmatched SOR-MAC-IAUSF that will result in unsuccessful security check while on UE reception

f_NR5GC_SorRegistration_6317(nr_Cell11,v_Data,v_Header,true,(Initial_NoSecurity,Emergency));

//@siclog "Step 16" siclog@

//The UE transmits an ULInformationTransfer message and REGISTRATION COMPLETE message without including an SOR transparent container

f_NR5GC_Registration_Complete(nr_Cell11);

//@siclog "Step 17" siclog@

//Check: Does the UE transmit an UL NAS TRANSPORT message with "Request type" set to "initial emergency request", and a PDU SESSION ESTABLISHMENT REQUEST message on NR Cell 13

//@siclog "Step 18" siclog@

//The SS transmits a PDU SESSION ESTABLISHMENT ACCEPT message.

f_NR5GC_PDUSessionEstablishment (nr_Cell11, pc_noOf_PDUsSameConnection, tsc_NR_RbId_SRB1,-,

omit,omit,

-,-,-,-,-,

-,-,-,-,

cs_SSC_Mode('A'H, '001'B),

-,-,-,-,-,-,-,

cr_NG_Request_Type('011'B));

if (pc_noOf_PDUsNewConnection >0 ){

f_NR_RRCRelease(nr_Cell11);

f_NR5GC_RRC_Idle_Extension(nr_Cell11, TESTMode_OFF, noRrcConnectionRelease);

}

}//End of fl_TC_6_3_1_7_TestBody

## Change 3

| Function name | f_NR_CellInfo_SetSIB1_ims_EmergencySupport |
| --- | --- |
| Reason for change | - Add function to complete change 1.1. |
| Summary of change | - Add function to set ims_EmergencySupport of sib 1. |
| TTCN module | NR_CellInfo.ttcn |
| MCC160 Comment |  |

New function

function f_NR_CellInfo_SetSIB1_ims_EmergencySupport(NR_CellId_Type p_NR_CellId,

template (omit) SIB1.ims_EmergencySupport p_ims_EmergencySupport ) runs on NR_BASE_PTC

{

var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId);

if (isvalue(p_ims_EmergencySupport)) {

v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.ims_EmergencySupport:= valueof(p_ims_EmergencySupport);

} else {

v_NR_CellInfo.Sysinfo.BcchInfo.SIB1.message_.c1.systemInformationBlockType1.ims_EmergencySupport:= omit;

}

f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo);

}

## Change 4

| Function name | f_NR5GC_SorRegistration_6317 |
| --- | --- |
| Reason for change | - Add function to complete change 2.1. |
| Summary of change | - Set the 4rh parameter of f_NR_RRC_ConnEst_DefWithNas to default value. |
| TTCN module | SOR_NR5GC.ttcn |
| MCC160 Comment |  |

New function

function f_NR5GC_SorRegistration_6317(NR_CellId_Type p_NR_CellId,

octetstring p_Data,

template (value) SOR_Header p_Header,

boolean p_MacError:= false,

template (present) TypeOfRegistration_Type p_RegType := Initial_NoSecurity) runs on NR5GC_PTC return O16_Type

{

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var O16_Type v_Mac_AMF;

var O16_Type v_Mac_UE;

var B128_Type v_Mac_A17;

var B128_Type v_Mac_A18;

var NG_NAS_SecurityParams_Type v_NAS_SecurityParams;

var O2_Type v_Counter :='0001'O;

var template (value) NG_NAS_DL_Message_Type v_MsgToSend;

var TypeOfRegistration_Type v_TypeOfRegistration;

//Steps 2 to 13 of the registration procedure described in TS 38.508 subclause 4.5.2.2-2 are performed on NR Cell 13

v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (p_NR_CellId, tsc_NR_RRC_TI_Def, ?);

select (v_ReceivedMsg.Pdu.Msg.registration_Request.registrationType.registrationType) {

case (tsc_NG_RegistrationEmergency) {

v_TypeOfRegistration := Emergency;

}

case (tsc_NG_RegistrationInitial) {

v_TypeOfRegistration := Initial_NoSecurity;

}

case else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration type incorrect");

}

}

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, v_TypeOfRegistration)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo,

p_NR_CellId, v_ReceivedMsg,

f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (p_NR_CellId)),

TESTMode_OFF,

Initial_NoSecurity);

//calculate v_Mac_AMF, which is used in REGISTRATION ACCEPT message

v_NAS_SecurityParams := f_NR5GC_Security_Get();

v_Mac_A17 :=f_NG_Authentication_A17(bit2oct(encvalue(p_Header)),

v_NAS_SecurityParams.KAUSF,

v_Counter,

p_Data,

v_NAS_SecurityParams.KDF);

v_Mac_AMF := bit2oct(v_Mac_A17);

if(p_MacError)

{

v_Mac_AMF := not4b(v_Mac_AMF);//unmatched SOR-MAC-IAUSF

};

//calculate v_Mac_UE, which is used in REGISTRATION COMPLETE message after successful check

v_Mac_A18 :=f_NG_Authentication_A18(v_NAS_SecurityParams.KAUSF,

v_Counter,

v_NAS_SecurityParams.KDF);

v_Mac_UE :=bit2oct(v_Mac_A18);

//The SS transmits an DLInformationTransfer message and an REGISTRATION ACCEPT message containing expected steering of roaming information

v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure,

f_NR5GC_GetMobileIdGUTI (p_NR_CellId),

cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (p_NR_CellId)),

{bit2oct(f_NR_CellInfo_GetTAC(p_NR_CellId))}),

-,-,-,-,-,-,-,-,-,-,-,-,-,-,-,-,-,

cs_NG_SORTransparentContainer_IeiIel(p_Header, v_Mac_AMF, v_Counter, p_Data));

// Registration Accept

SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId,

tsc_NR_RbId_SRB1, // must be on SRB1

cs_TimingInfo_Now,

cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend)));

return v_Mac_UE;

}

# Branches executed

NR Band 41,Nea2,Nia2.

# Execution Log Files

## HiSilicon Balong 5000

The HiSilicon Balong 5000 passed this test case on Datang LinkTester 5G Protocol Conformance Test System ECT9610. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test Case Execution log file:
\ TC_6_3_1_7_NR5GC_N41.log

- PIXCIT settings used:
\ Pics(HiSilicon).xml

\ Pixit(TC_6_3_1_7_NR5GC).xml

# References

| [1] | R5s201456: Supporting information for agreement of NR5GC test case 6.3.1.7 in FR1 |
| --- | --- |