3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210182

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1379 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC IRAT test case 6.2.1.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-11-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - The UE will not trigger deregistration in postamble if automatic PLMN search is ongoing - EUTRA PTC will wait for NR capabilities during the registration procedure at step 2a2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add 300s delay to allow automatic PLMN selection to complete - Transfer NR capabilities to EUTRA |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1 Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2019-06_D20wk38 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_6_2_1_4_NR5GC_EUTRA |
| --- | --- |
| Reason for change | The UE will not trigger deregistration in postamble if automatic PLMN search is ongoing |
| Summary of change | Add 300s delay to allow automatic PLMN selecton to complete |
| TTCN module | Idle_PLMNSelection_NR5GC_EUTRA |
| MCC160 Comment |  |

Before Change

function f_TC_6_2_1_4_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { // Inter-RAT PLMN Selection / Selection of correct RAT from the OPLMN list / Manual mode ………………… // @siclog "Step 2a5" siclog@ // Set the UE in Automatic PLMN selection mode f_UT_AutomaticPLMN_Select(UT); // @sic R5-203375 sic@ f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); f_EUTRA_TestBody_Set(false); f_EUTRA38_MultiPDN_Postamble(eutra_Cell1, E1_IDLE); // @sic R5-204475 sic@ }else { // @siclog "Step 2b4" siclog@ // The SS adjusts the NR Cell power levels according to row "T2" in table 6.2.1.4.3.2-1/2 f_EUTRA_NR_WaitForCoOrd_Trigger(NR); f_EUTRA38_SetCellPower(eutra_Cell1, tsc_NonSuitableOffCellRS_EPRE, tsc_NonSuitableOffCellRS_EPRE); // @sic R5-203375 sic@ f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); //Wait for end of test case f_EUTRA_NR_WaitForCoOrd_Trigger(NR, "Postamble"); f_EUTRA_TestBody_Set(false); f_EUTRA_ReleaseAllCells(); } }

After Change

function f_TC_6_2_1_4_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { // Inter-RAT PLMN Selection / Selection of correct RAT from the OPLMN list / Manual mode ………………………….. // @siclog "Step 2a5" siclog@ // Set the UE in Automatic PLMN selection mode f_UT_AutomaticPLMN_Select(UT); // @sic R5-203375 sic@ f_Delay(300.0); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); f_EUTRA_TestBody_Set(false); f_EUTRA38_MultiPDN_Postamble(eutra_Cell1, E1_IDLE); // @sic R5-204475 sic@ }else { // @siclog "Step 2b4" siclog@ // The SS adjusts the NR Cell power levels according to row "T2" in table 6.2.1.4.3.2-1/2 f_EUTRA_NR_WaitForCoOrd_Trigger(NR); f_EUTRA38_SetCellPower(eutra_Cell1, tsc_NonSuitableOffCellRS_EPRE, tsc_NonSuitableOffCellRS_EPRE); // @sic R5-203375 sic@ f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger); //Wait for end of test case f_EUTRA_NR_WaitForCoOrd_Trigger(NR, "Postamble"); f_EUTRA_TestBody_Set(false); f_EUTRA_ReleaseAllCells(); } }

## Change 2

| Function name | f_TC_6_2_1_4_NR5GC |
| --- | --- |
| Reason for change | - EUTRA PTC will wait for NR capabilities during the registration procedure at step 2a2 - The UE will not trigger deregistration in postamble if automatic PLMN search is ongoing |
| Summary of change | - send NR Capability to EUTRA - Add delay to allow automatic PLMN selection to complete |
| TTCN module | Idle_PLMNSelection_NR5GC_NR |
| MCC160 Comment |  |

Before Change

function f_TC_6_2_1_4_NR5GC() runs on NR5GC_PTC { // Inter-RAT PLMN Selection / Selection of correct RAT from the OPLMN list / Manual mode …………… // @siclog "Step 1" siclog@ // Power on the UE f_UT_SwitchOnUE(UT); if(pc_Available_PLMNs_AcT_Ind){ f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger); // @siclog "Step 2a4" siclog@ // The SS adjusts the NR Cell power levels according to row "T1" in table 6.2.1.4.3.2-1/2 f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); f_NR_SetCellPowerList(v_CellPowerList_T1); // @sic R5-203375 sic@ f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger); //Wait for end of test case f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); f_NR_TestBody_Set(false); f_NR_ReleaseAllCells(); }else { f_UT_ManualPLMN_Select(UT, f_NR_Asn2Nas_PlmnId (valueof(tsc_NR_HPLMN_002_11))); // @sic R5s200772 sic@ // @siclog "Step 2b2" siclog@ // Does the test result of generic test procedure in TS 38.508-1[4] table 4.5.2.2-2 is performed and the UE is camped on NR Cell 1 f_NR5GC_RRC_IdleState1N_Def(nr_Cell1, TESTMode_OFF, -, Initial_NoSecurity); //@siclog "Step 2b3" siclog@ //Check: Is PLMN2 indicated as registered PLMN by the UE v_PLMN := f_ConvertPLMNtoString(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN(nr_Cell1))); f_UT_PLMN_Check(UT, v_PLMN); f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 6.2.1.4 Step 2b3"); // @siclog "Step 2b4" siclog@ // The SS adjusts the NR Cell power levels according to row "T2" in table 6.2.1.4.3.2-1/2 f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger); f_NR_SetCellPowerList(v_CellPowerList_T2); // @sic R5-203375 sic@ f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); // @siclog "Step 2b5" siclog@ // Set the UE in Automatic PLMN selection mode f_UT_AutomaticPLMN_Select(UT); // @sic R5-203375 sic@ f_NR_TestBody_Set(false); //Tell EUTRA PTC that test case is over and perform postamble f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger("Postamble")); // Switch/Power off UE f_NR_Postamble(nr_Cell1, STATE_IDLE_1A); } }

After Change

function f_TC_6_2_1_4_NR5GC() runs on NR5GC_PTC { // Inter-RAT PLMN Selection / Selection of correct RAT from the OPLMN list / Manual mode …………………………. // @siclog "Step 1" siclog@ // Power on the UE f_UT_SwitchOnUE(UT); if(pc_Available_PLMNs_AcT_Ind){ f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger); f_NR5GC_IRAT_UECapabilityTransfer(nr_Cell1); // @siclog "Step 2a4" siclog@ // The SS adjusts the NR Cell power levels according to row "T1" in table 6.2.1.4.3.2-1/2 f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); f_NR_SetCellPowerList(v_CellPowerList_T1); // @sic R5-203375 sic@ f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger); //Wait for end of test case f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); f_NR_TestBody_Set(false); f_NR_ReleaseAllCells(); }else { f_UT_ManualPLMN_Select(UT, f_NR_Asn2Nas_PlmnId (valueof(tsc_NR_HPLMN_002_11))); // @sic R5s200772 sic@ // @siclog "Step 2b2" siclog@ // Does the test result of generic test procedure in TS 38.508-1[4] table 4.5.2.2-2 is performed and the UE is camped on NR Cell 1 f_NR5GC_RRC_IdleState1N_Def(nr_Cell1, TESTMode_OFF, -, Initial_NoSecurity); //@siclog "Step 2b3" siclog@ //Check: Is PLMN2 indicated as registered PLMN by the UE v_PLMN := f_ConvertPLMNtoString(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN(nr_Cell1))); f_UT_PLMN_Check(UT, v_PLMN); f_NR_PreliminaryPass(__FILE__, __LINE__, "Test Case 6.2.1.4 Step 2b3"); // @siclog "Step 2b4" siclog@ // The SS adjusts the NR Cell power levels according to row "T2" in table 6.2.1.4.3.2-1/2 f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger); f_NR_SetCellPowerList(v_CellPowerList_T2); // @sic R5-203375 sic@ f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA); // @siclog "Step 2b5" siclog@ // Set the UE in Automatic PLMN selection mode f_UT_AutomaticPLMN_Select(UT); // @sic R5-203375 sic@ f_Delay(300.0); f_NR_TestBody_Set(false); //Tell EUTRA PTC that test case is over and perform postamble f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger("Postamble")); // Switch/Power off UE f_NR_Postamble(nr_Cell1, STATE_IDLE_1A); }