3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210191

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1399 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC test case 7.1.1.5.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-11-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | PDSCH HARQ timing indicator should be based on K1 value for the used subcarrier spacing. The current implementation at step 5 is using the value for SCS 15kHz only. |  |  |  |  |  |  |  |  |
| Summary of change: |  | No value should for PDSCH HARQ timing indicator should be passed to f_NR_SS_CellConfig_DciDlInfo_NoRetransmission(), so the value is based on used subcarrier spacing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.5.2.ENDC, 7.1.1.5.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2019-06_D20wk38 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

The following changes are applied on iwd-TTCN3-B2019-06_D20wk38 plus TTCN CR R5s201436

## Change 1

| Function name | f_TC_7_1_1_5_2_NR_TestBody |
| --- | --- |
| Reason for change | PDSCH HARQ timing indicator should be based on K1 value for the used subcarrier spacing. The current implementation at step 5 is using the value for SCS 15kHz only. |
| Summary of change | No value should for PDSCH HARQ timing indicator should be passed to f_NR_SS_CellConfig_DciDlInfo_NoRetransmission, so the value is based on used subcarrier spacing. |
| TTCN module | DRX_MAC_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_7_1_1_5_2_NR_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC

{ /* L2/MAC

* 7.1.1.5.2 : DRX operation / Short cycle not configured / Long DRX command MAC control element reception

*/

………………..

//@siclog "Step 5" siclog@

//@sic R5s200859 sic@

//@sic R5s200899 sic@

v_Timing := f_NR_SubFrameTiming_AddSlots(v_Timing1, v_SlotsInframe);

v_CE_SubPDUListDL := { cs_NR_MAC_CE_SubPDU_DL( cs_NR_MAC_LongDRXCommandSubHeader ) };  //

v_MAC_PDUListDL2 := { cs_NR_DL_MAC_PDU(v_CE_SubPDUListDL, omit,  cs_NR_MAC_Padding_SubPDU('000000'O)) };   // 2 byte sub headers +3byte Padding header= 40 bits

DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing), v_MAC_PDUListDL2, cs_NR_HarqProcessAssignment(tsc_HarqProcessId+1)));

// K = 7

v_Timing := f_NR_SubFrameTiming_AddSlots(v_Timing,6);

f_NR_SS_CellConfig_DciDlInfo_NoRetransmission(nr_Cell1,cs_TimingInfo_NR(v_Timing),-,'101'B);

…………………

};

After Change

function f_TC_7_1_1_5_2_NR_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC

{ /* L2/MAC

* 7.1.1.5.2 : DRX operation / Short cycle not configured / Long DRX command MAC control element reception

*/

……………..

//@siclog "Step 5" siclog@

//@sic R5s200859 sic@

//@sic R5s200899 sic@

v_Timing := f_NR_SubFrameTiming_AddSlots(v_Timing1, v_SlotsInframe);

v_CE_SubPDUListDL := { cs_NR_MAC_CE_SubPDU_DL( cs_NR_MAC_LongDRXCommandSubHeader ) };  //

v_MAC_PDUListDL2 := { cs_NR_DL_MAC_PDU(v_CE_SubPDUListDL, omit,  cs_NR_MAC_Padding_SubPDU('000000'O)) };   // 2 byte sub headers +3byte Padding header= 40 bits

DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing), v_MAC_PDUListDL2, cs_NR_HarqProcessAssignment(tsc_HarqProcessId+1)));

// K = 7

v_Timing := f_NR_SubFrameTiming_AddSlots(v_Timing,6);

f_NR_SS_CellConfig_DciDlInfo_NoRetransmission(nr_Cell1,cs_TimingInfo_NR(v_Timing));

……………

};