3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210195

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1406 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC idle more test case 6.3.1.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-11-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | At step 11 UE is sending PDU SESSION ESTABLISHMENT REQUEST for emergency PDU session. Current TTCN implementation is expecting PDU sessions to be established according to pc_noOf_PDUsSameConnection and pc_noOf_PDUsNewConnection. This may fail if pc_noOf_PDUsSameConnection is 0 or >1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | At step 11 expect establishment of 1 (emergency) PDU session only in same connection, for emergency session establishment |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.7.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2019-06_D20wk38 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_6_3_1_7_TestBody |
| --- | --- |
| Reason for change | At step 11 UE is sending PDU SESSION ESTABLISHMENT REQUEST for emergency PDU session. Current TTCN implementation is expecting PDU sessions to be established according to pc_noOf_PDUsSameConnection and pc_noOf_PDUsNewConnection. This may fail if pc_noOf_PDUsSameConnection is 0 or >1. |
| Summary of change | At step 11 expect establishment of 1 (emergency) PDU session only in same connection, for emergency session establishment |
| TTCN module | NR5GC\6_3_1\SOR_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_6_3_1_7_TestBody() runs on NR5GC_PTC { //@sic R5-204410 sic@ var NAS_PlmnId v_PlmnID_2 := f_NR_Asn2Nas_PlmnId(tsc_NR_HPLMN_002_11); //@sic R5-204410 sic@ var O2_Type v_AccessTechId :='0800'O; var octetstring v_Data := v_PlmnID_2 & v_AccessTechId; var template (value) SOR_Header v_Header := crs_NG_SOR_Header('1'B,'1'B,'1'B,'0'B); //@siclog "Step 1" siclog@ //Power on the UE //@siclog "Step 1A1-1A13" siclog@ //Steps 1 to 13 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed //@sic R5-204410 sic@ f_NR5GC_Preamble_Steps1_13(nr_Cell11); //@siclog "Step 1B" siclog@ //SS transmits a REGISTRATION REJECT message with cause #22 (Congestion) and T3346 set to 3 minutes. //@sic R5-204410 sic@ SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell11, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, tsc_NR5GCCause_Congestion), cs_GPRS_Timer2_3_IEI('5F'O, '001'B, '00011'B))))); //@siclog "Step 1C" siclog@ //The SS releases the RRC connection //@sic R5-204410 sic@ f_NR_RRCRelease(nr_Cell11); //@siclog "Step 2" siclog@ //The UE is made to establish an emergency PDU session. This can be done by an AT/MMI command f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, -, true); //@siclog "Step 3" siclog@ //The UE transmits an RRCSetupRequest on NR Cell 13 indicating emergencency registration? SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_emergency)))); f_NR_PreliminaryPass(__FILE__, __LINE__, "RRCSetupRequest received");//@sic R5s200305 sic@ //@siclog "Steps 4-5" siclog@ //Steps 3 to 4 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed //@siclog "Step 6" siclog@ //The UE transmit a REGISTRATION REQUEST message.5GS registration type can set to either "initial registration" according to TS 24.501 clause 5.5.1.2.6A or "emergency registration" according to TS 24.501 clause 5.5.1.2. //@siclog "Steps 7-14" siclog@ //Steps 5 to 13 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed on NR Cell 13 //@siclog "Step 15" siclog@ //The SS transmits a DLInformationTransfer message and a REGISTRATION ACCEPT message containing //steering of roaming information, includes unmatched SOR-MAC-IAUSF that will result in unsuccessful security check while on UE reception f_NR5GC_SorRegistration_6317(nr_Cell11,v_Data,v_Header,true,(Initial_NoSecurity,Emergency)); //@siclog "Step 16" siclog@ //The UE transmits an ULInformationTransfer message and REGISTRATION COMPLETE message without including an SOR transparent container f_NR5GC_Registration_Complete(nr_Cell11); //@siclog "Step 17" siclog@ //Check: Does the UE transmit an UL NAS TRANSPORT message with "Request type" set to "initial emergency request", and a PDU SESSION ESTABLISHMENT REQUEST message on NR Cell 13 //@siclog "Step 18" siclog@ //The SS transmits a PDU SESSION ESTABLISHMENT ACCEPT message. f_NR5GC_PDUSessionEstablishment (nr_Cell11, pc_noOf_PDUsSameConnection, tsc_NR_RbId_SRB1,-, omit,omit, -,-,-,-,-, -,-,-,-, cs_SSC_Mode('A'H, '001'B), -,-,-,-,-,-,-, cr_NG_Request_Type('011'B)); if (pc_noOf_PDUsNewConnection >0 ){ f_NR_RRCRelease(nr_Cell11); f_NR5GC_RRC_Idle_Extension(nr_Cell11, TESTMode_OFF, noRrcConnectionRelease); } }//End of fl_TC_6_3_1_7_TestBody

After Change:

function fl_TC_6_3_1_7_TestBody() runs on NR5GC_PTC { //@sic R5-204410 sic@ var NAS_PlmnId v_PlmnID_2 := f_NR_Asn2Nas_PlmnId(tsc_NR_HPLMN_002_11); //@sic R5-204410 sic@ var O2_Type v_AccessTechId :='0800'O; var octetstring v_Data := v_PlmnID_2 & v_AccessTechId; var template (value) SOR_Header v_Header := crs_NG_SOR_Header('1'B,'1'B,'1'B,'0'B); //@siclog "Step 1" siclog@ //Power on the UE //@siclog "Step 1A1-1A13" siclog@ //Steps 1 to 13 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed //@sic R5-204410 sic@ f_NR5GC_Preamble_Steps1_13(nr_Cell11); //@siclog "Step 1B" siclog@ //SS transmits a REGISTRATION REJECT message with cause #22 (Congestion) and T3346 set to 3 minutes. //@sic R5-204410 sic@ SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell11, tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1 cs_TimingInfo_Now, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_NG_REGISTRATION_REJECT(cs_GMM_GSM_Cause(omit, tsc_NR5GCCause_Congestion), cs_GPRS_Timer2_3_IEI('5F'O, '001'B, '00011'B))))); //@siclog "Step 1C" siclog@ //The SS releases the RRC connection //@sic R5-204410 sic@ f_NR_RRCRelease(nr_Cell11); //@siclog "Step 2" siclog@ //The UE is made to establish an emergency PDU session. This can be done by an AT/MMI command f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, -, true); //@siclog "Step 3" siclog@ //The UE transmits an RRCSetupRequest on NR Cell 13 indicating emergencency registration? SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_emergency)))); f_NR_PreliminaryPass(__FILE__, __LINE__, "RRCSetupRequest received");//@sic R5s200305 sic@ //@siclog "Steps 4-5" siclog@ //Steps 3 to 4 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed //@siclog "Step 6" siclog@ //The UE transmit a REGISTRATION REQUEST message.5GS registration type can set to either "initial registration" according to TS 24.501 clause 5.5.1.2.6A or "emergency registration" according to TS 24.501 clause 5.5.1.2. //@siclog "Steps 7-14" siclog@ //Steps 5 to 13 of the registration procedure described in TS 38.508-1 [4] Table 4.5.2.2-2 are performed on NR Cell 13 //@siclog "Step 15" siclog@ //The SS transmits a DLInformationTransfer message and a REGISTRATION ACCEPT message containing //steering of roaming information, includes unmatched SOR-MAC-IAUSF that will result in unsuccessful security check while on UE reception f_NR5GC_SorRegistration_6317(nr_Cell11,v_Data,v_Header,true,(Initial_NoSecurity,Emergency)); //@siclog "Step 16" siclog@ //The UE transmits an ULInformationTransfer message and REGISTRATION COMPLETE message without including an SOR transparent container f_NR5GC_Registration_Complete(nr_Cell11); //@siclog "Step 17" siclog@ //Check: Does the UE transmit an UL NAS TRANSPORT message with "Request type" set to "initial emergency request", and a PDU SESSION ESTABLISHMENT REQUEST message on NR Cell 13 //@siclog "Step 18" siclog@ //The SS transmits a PDU SESSION ESTABLISHMENT ACCEPT message. f_NR5GC_PDUSessionEstablishment (nr_Cell11, 1, tsc_NR_RbId_SRB1,-, omit,omit, -,-,-,-,-, -,-,-,-, cs_SSC_Mode(-, '001'B), -,-,-,-,-,-,-, cr_NG_Request_Type('011'B)); /* REMOVED if (pc_noOf_PDUsNewConnection >0 ){ f_NR_RRCRelease(nr_Cell11); f_NR5GC_RRC_Idle_Extension(nr_Cell11, TESTMode_OFF, noRrcConnectionRelease); }*/ }//End of fl_TC_6_3_1_7_TestBody