3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210196

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1410 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to ENDC RLC testcase 7.1.2.3.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation at Steps 8 and 16, the intention of the NR RRCReconfiguration message is to change the pollPDU and it is not required to include the other IEs such as MAC-CellGroupConfig, PhysicalCellGroupConfig and SPCellConfig, as their inclusion, especially, SPCellConfig would un-necessarily trigger a PRACH procedure from the UE which serves no purpose as the configuration of the SPCellConfig has remained the same as before. Attention is also drawn to the fact that in the SA version of this TC, the three IEs are not being included in the NR RRCReconfiguration message |  |  |  |  |  |  |  |  |
| Summary of change: |  | Set the IEs MAC-CellGroupConfig, PhysicalCellGroupConfig and SPCellConfig to omit in the NR RRCReconfiguartion message. NOTE: A Prose CR R5-205671 is raised at the RAN5#89e. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | An un-necessary PRACH procedure shall be triggered from the UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.3.6.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-205671 is raised. |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	1

1	Corrections required	3

1.1	function f_NR_RLC_Reconfiguration ()	3

1.2	New function f_NR_ENDC_ReConfigAM_UM_7123x	4

# Corrections required

## function f_NR_RLC_Reconfiguration ()

| Function name | f_NR_RLC_Reconfiguration () |
| --- | --- |
| Reason for change | In the current TTCN implementation at Steps 8 and 16, the intention of the NR RRCReconfiguration message is to change the pollPDU and it is not required to include the other IEs such as MAC-CellGroupConfig, PhysicalCellGroupConfig and SPCellConfig, as their inclusion, especially, SPCellConfig would un-necessarily trigger a PRACH procedure from the UE which serves no purpose as the configuration of the SPCellConfig has remained the same as before. Attention is also drawn to the fact that in the SA version of this TC, the three IEs are not being included in the NR RRCReconfiguration message |
| Summary of change | Set the IEs MAC-CellGroupConfig, PhysicalCellGroupConfig and SPCellConfig to omit in the NR RRCReconfiguartion message. NOTE: A Prose CR shall be raised at the next RAN5. |
| TTCN module | Common_NR_TC/7_1_2/RLC_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR_RLC_Reconfiguration(NR_CellId_Type p_CellId, integer p_NoOfNR_AMBearersN, integer p_NoOfNR_UMBearersM, template (value) NR_RLC_BearerToAddModList_Type p_RLC_BearerConfigList, template (omit) NR_RadioBearerList_Type p_SS_Drb_ConfigList := omit) runs on NR_BASE_PTC { if (f_GetTestcaseAttrib_ENDC(testcasename())) { //ENDC tc f_NR_ENDC_ReConfigAM_UM(p_CellId, p_NoOfNR_AMBearersN, p_NoOfNR_UMBearersM, p_SS_Drb_ConfigList, p_RLC_BearerConfigList, -, -, -, -, RRC_CONNECTED); // @sic R5s190800, R5s201010 sic@ } else { f_NR5GC_ReConfigAM_UM(p_CellId, p_RLC_BearerConfigList); } }

After Change:

function f_NR_RLC_Reconfiguration(NR_CellId_Type p_CellId, integer p_NoOfNR_AMBearersN, integer p_NoOfNR_UMBearersM, template (value) NR_RLC_BearerToAddModList_Type p_RLC_BearerConfigList, template (omit) NR_RadioBearerList_Type p_SS_Drb_ConfigList := omit) runs on NR_BASE_PTC { if (f_GetTestcaseAttrib_ENDC(testcasename())) { //ENDC tc f_NR_ENDC_ReConfigAM_UM_7123x(p_CellId, p_NoOfNR_AMBearersN, p_NoOfNR_UMBearersM, p_SS_Drb_ConfigList, p_RLC_BearerConfigList, -, -, -, -, RRC_CONNECTED); // @sic R5s190800, R5s201010 sic@ } else { f_NR5GC_ReConfigAM_UM(p_CellId, p_RLC_BearerConfigList); } }

## New function f_NR_ENDC_ReConfigAM_UM_7123x

| Function name | f_NR_ENDC_ReConfigAM_UM_7123x |
| --- | --- |
| Reason for change | In the current TTCN implementation at Steps 8 and 16, the intention of the NR RRCReconfiguration message is to change the pollPDU and it is not required to include the other IEs such as MAC-CellGroupConfig, PhysicalCellGroupConfig and SPCellConfig, as their inclusion, especially, SPCellConfig would un-necessarily trigger a PRACH procedure from the UE which serves no purpose as the configuration of the SPCellConfig has remained the same as before. Attention is also drawn to the fact that in the SA version of this TC, the three IEs are not being included in the NR RRCReconfiguration message |
| Summary of change | Set the IEs MAC-CellGroupConfig, PhysicalCellGroupConfig and SPCellConfig to omit in the NR RRCReconfiguartion message. NOTE: A Prose CR shall be raised at the next RAN5. |
| TTCN module | Common/NR/NR_CommonFunctions.ttcn |
| MCC160 Comment |  |

function f_NR_ENDC_ReConfigAM_UM_7123x(NR_CellId_Type p_CellId, integer p_NoOfNR_AMBearersN, integer p_NoOfNR_UMBearersM, template (omit) NR_RadioBearerList_Type p_SS_Drb_ConfigList := omit, template (omit) NR_RLC_BearerToAddModList_Type p_RLC_BearerConfigList := omit, template (omit) SRB_ToAddModList p_SRB_ToAddModList := omit, template (omit) DRB_ToAddModList p_DRB_ToAddModList := omit, template (omit) SecurityConfig p_SecurityConfig := omit, template (omit) MAC_CellGroupConfig p_MAC_CellGroupConfig := omit, NR_RRC_STATE_Type p_RRC_State := RRC_IDLE, template (omit) PDSCH_ServingCellConfig.nrofHARQ_ProcessesForPDSCH p_NrofHARQ_ProcessesForPDSCH := omit ) runs on NR_BASE_PTC // @sic R5s201010 sic@ { // @sic R5-203533: added p_NrofHARQ_ProcessesForPDSCH sic@ var DRB_Identity v_DrbId := tsc_NR_DRB2; // for ENDC only var template (value) CellGroupConfig v_SCGConfig; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var template (value) DRB_ToAddModList v_DRB_ToAddModList; var template (value) SecurityConfig v_SecurityConfig; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var template (value) RLC_Config v_RLC_Config; var EUTRA_NR_Coordination_MSG v_Coordination_MSG_Value; var integer i; var boolean v_IsFR1:= f_NR_CellInfo_GetIsFR1(p_CellId); var template (omit) PDCP_Config.drb.statusReportRequired v_StatusReportRequired := true_; for (i:=0; i < p_NoOfNR_AMBearersN + p_NoOfNR_UMBearersM; i:=i+1) { if (i < p_NoOfNR_AMBearersN) { v_SS_Drb_ConfigList[i] := f_NR_GetRadioBearerConfig_FullAM(p_CellId, v_DrbId); v_RLC_Config := cs_38508_RLC_Config_AM_DRB(f_NR_UL_AM_RLC(v_IsFR1), f_NR_DL_AM_RLC(v_IsFR1)); } else { v_SS_Drb_ConfigList[i] := f_NR_GetRadioBearerConfig_FullUM(p_CellId, v_DrbId); //ENDC function: Sdap=omit v_RLC_Config := cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)); v_StatusReportRequired := omit; } v_RLC_BearerConfigList[i] := cs_38508_RLC_BearerConfig_DRB(v_DrbId, v_RLC_Config, cs_38508_LogicalChannelConfig_DRB); v_DRB_ToAddModList[i] := cs_38508_DRB_ToAddMod(v_DrbId, cs_38508_PDCP_Config (-, -, -, -, v_StatusReportRequired)); // @sic R5s200696, R5s200998 sic@ v_DrbId := v_DrbId + 1; } v_SecurityConfig := cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig (f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()), secondary); if (isvalue(p_SS_Drb_ConfigList)) { v_SS_Drb_ConfigList := valueof(p_SS_Drb_ConfigList); } if (isvalue(p_RLC_BearerConfigList)) { v_RLC_BearerConfigList := valueof(p_RLC_BearerConfigList); } if (isvalue(p_DRB_ToAddModList)) { v_DRB_ToAddModList := valueof(p_DRB_ToAddModList); } if (isvalue(p_SecurityConfig)) { v_SecurityConfig := valueof(p_SecurityConfig); } f_NR_SS_CommonRadioBearerConfig(p_CellId, v_SS_Drb_ConfigList); v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(p_SRB_ToAddModList, v_DRB_ToAddModList, -, v_SecurityConfig); v_SCGConfig := f_NR_GetSCGConfigDef(p_CellId); v_SCGConfig.mac_CellGroupConfig := omit; v_SCGConfig.physicalCellGroupConfig := omit; v_SCGConfig.spCellConfig := omit; v_SCGConfig.rlc_BearerToAddModList := v_RLC_BearerConfigList; f_NR_SendRRCReconfigurationContentsToEUTRA(v_RadioBearerConfig, v_SCGConfig, -, -, false); //@sic R5s190541, R5s200647 sic@ // Get Security info from EUTRA, calculate latest keys and configure SS // as only need to do this for the first reconfiguration @sic R5s200647, R5s201010 sic@ if (p_RRC_State != RRC_CONNECTED) { f_ENDC_GetKeysActivateSecurity(p_CellId, EUTRA); } //RRCReconfigurationComplete not yet received from E-UTRA - Wait for it @sic R5s200647 sic@ EUTRA.receive(cmr_EUTRA_NR_OctetData ("Complete")) -> value v_Coordination_MSG_Value; f_NR_CheckReceived_RRCReconfigurationComplete(valueof(v_Coordination_MSG_Value)); }