3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210197

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1412 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR MAC test case 7.1.1.5.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-11-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - PDSCH HARQ timing indicator should be based on K1 value for the used subcarrier spacing. The current implementation at step 5 is using the value for SCS 15kHz only. - K1 value should be used as per SCS at step 11 - v_timing should be used at step5. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - No value should for PDSCH HARQ timing indicator should be passed to f_NR_SS_CellConfig_DciDlInfo_NoRetransmission, so the value is based on used subcarrier spacing. Note : Similar CR R5s201493 has been raised for 7.1.1.5.2 - Using v_NR_K1 for K1 value at step 11 - Correct v_timing3 to v_timing at step5 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.5.4.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1 Change 1	3

2.2 Change 2	9

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2019-06_D20wk38 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

The following changes are applied on  iwd-TTCN3-B2019-06_D20wk38 and CR R5s201437

## 2.1 Change 1

| Function name | f_TC_7_1_1_5_4_NR_TestBody |
| --- | --- |
| Reason for change | - PDSCH HARQ timing indicator should be based on K1 value for the used subcarrier spacing. The current implementation at step 5 is using the value for SCS 15kHz only. - K1 value should be used as per SCS at step 11 - v_timing should be used at step5. |
| Summary of change | - No value should for PDSCH HARQ timing indicator should be passed to f_NR_SS_CellConfig_DciDlInfo_NoRetransmission, so the value is based on used subcarrier spacing. Note : Similar CR R5s201493 has been raised for 7.1.1.5.2 - Using v_NR_K1 for K1 value at step 11 - Correct v_timing3 to v_timing at step5 |
| TTCN module | DRX_MAC_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_7_1_1_5_4_NR_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC

{ /* L2/MAC

* 7.1.1.5.4 : DRX Operation / Short cycle configured / DRX command MAC control element reception

*/

const UInt_Type tsc_DrxStartOffset := 7; // in ms @sic R5-202609 sic@

const UInt_Type tsc_HarqProcessId :=0; //Arbitrarily selected

// As per 38.508-1 Table 4.6.3-192: TDD-UL-DL-Config TDD UL DL Periodicity is 5 slots for all SCS configs used in Signalling (15,30,120)

const UInt_Type tsc_DRXRetransTimer_DL := 80;// in slots

const UInt_Type tsc_DRX_HARQ_RTT_TimerDL := 4;// in slots 56/14

var UInt_Type v_ShortDRXCycle :=  80; //@sic R5s200867 sic@

var SubFrameTiming_Type v_Timing  := valueof(cs_SubFrameTiming(0, 0, 0));

var SubFrameTiming_Type v_Timing1 := v_Timing;

var SubFrameTiming_Type v_Timing2 := v_Timing;

var SubFrameTiming_Type v_Timing3 := v_Timing;

var integer v_SlotsInSubframe := f_NR_CellInfo_GetSlotsNumberPerSubframe(nr_Cell1);

var integer v_SlotsInframe := v_SlotsInSubframe * 10;

var NR_MAC_SDU_Type v_EncodedRlcPdu1;

var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL;

var template (omit) NR_MAC_CE_SubPDU_DL_List_Type v_CE_SubPDUListDL;

var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL;

var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL2;

var integer v_NR_K1:= 5;

var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1);

timer t_T7115 := 0.05;

timer t_Watchdog := 5.0;

select (v_SCS) {

case(kHz15)  { v_NR_K1 := 7; }   /* 7 */

case(kHz30)  { v_NR_K1 := 5; }   /* 5 */

case(kHz120) { v_NR_K1 := 8; }   /* 8 */

case else    { FatalError(__FILE__, __LINE__, "Invalid SubcarrierSpacing"); }

}

// stop periodic UL grants and restart automatic grant allocation instead; periodic TA is kept stopped:

f_NR_ULGrantConfiguration_Start(nr_Cell1,-, -, -, cs_NR_UplinkTimeAlignment_Stop);

// suppress number of DL retransmissions:

f_NR_SS_CellConfig_DciDlInfo_NoRetransmission(nr_Cell1);

// to configure SS to report reception of HARQ ACK or NACK

f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable));

//@siclog "Step 3" siclog@

//f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Error1AndNormal)); //@sic R5s200867 sic@

v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B)));

v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) };

v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) };

//@sic R5s200867 sic@

v_Timing1 := f_NR_GetNextSendOccasion_7_1_1_5_X(nr_Cell1, v_ShortDRXCycle, tsc_DrxStartOffset);

//v_Timing1 := f_NR_SubFrameTiming_AddSlots(v_Timing1, v_ShortDRXCycle * v_SlotsInSubframe * 2);

// X= selected the last DL slot, InactivityTimer< X< on-DurationTimer -1

//v_Timing2 := f_NR_SubFrameTiming_AddSlots(v_Timing1, v_SlotsInframe + 9);

// X= selected the last DL slot in Frame 1, as on duration timer =20 ms

//v_Timing2 := f_NR_SubFrameTiming_AddSlots(v_Timing1, f_NR_DRX_LastDL_Slot(nr_Cell1));

// The other things in Step 3 automatically taken care by SS

// step 3 and 7 should be on one HARQ process

DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing1), v_MAC_PDUListDL, cs_NR_HarqProcessAssignment(tsc_HarqProcessId)));

//@siclog "Step 5" siclog@

// 5 ms later configuere SS for normal CRC error mode

//v_Timing := f_NR_SubFrameTiming_AddSlots(v_Timing2, 5);

//f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Normal,cs_TimingInfo_NR(v_Timing)));

// In the Last PDCCH occasion befor on duration timer expiry

v_Timing := f_NR_SubFrameTiming_AddSlots(v_Timing1, v_SlotsInframe); //@sic R5s200926 sic@

//v_Timing3 := f_NR_SubFrameTiming_AddSlots(v_Timing2,v_SlotsInframe);

v_CE_SubPDUListDL := { cs_NR_MAC_CE_SubPDU_DL( cs_NR_MAC_DRXCommandSubHeader ) };  //

v_MAC_PDUListDL2 := { cs_NR_DL_MAC_PDU(v_CE_SubPDUListDL, omit,  cs_NR_MAC_Padding_SubPDU('000000'O)) };   // 2 byte sub headers +3byte Padding header= 40 bits

DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing3), v_MAC_PDUListDL2, cs_NR_HarqProcessAssignment(tsc_HarqProcessId+1)));

//k1 =7

v_Timing := f_NR_SubFrameTiming_AddSlots(v_Timing,7);

f_NR_SS_CellConfig_DciDlInfo_NoRetransmission(nr_Cell1,cs_TimingInfo_NR(v_Timing),-,'101'B);

//@siclog "Step 6A" siclog@

//@sic R5s200926 sic@

v_Timing := f_NR_SubFrameTiming_AddSlots(v_Timing1, 2*v_SlotsInframe);

DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing), v_MAC_PDUListDL, cs_NR_HarqProcessAssignment(tsc_HarqProcessId+1)));

//@siclog "Step 7" siclog@

v_Timing3 := f_NR_SubFrameTiming_AddSlots(v_Timing1, 3 * v_ShortDRXCycle * v_SlotsInSubframe);

//v_Timing := f_NR_SubFrameTiming_AddSlots(v_Timing2, v_SlotsInframe+5);

f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Error1AndNormal,cs_TimingInfo_NR(v_Timing3)));

// With HARQ RTT = 4slots, and DL-UL Periodicity =5 slots, it should be in the first PDCCH occasion in Drx-RetransmissionTimerDL

//v_Timing3 := f_NR_SubFrameTiming_AddSlots(v_Timing, v_SlotsInframe);

//v_Timing := f_NR_SubFrameTiming_AddSlots(v_Timing2, 2*v_SlotsInframe);

DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing3), v_MAC_PDUListDL, cs_NR_HarqProcessAssignment(tsc_HarqProcessId)));

//@siclog "Step 8A" siclog@

v_Timing := f_NR_SubFrameTiming_AddSlots(v_Timing3, v_NR_K1 + tsc_DRX_HARQ_RTT_TimerDL-1);

v_Timing := f_NR_LastDL_Slot(nr_Cell1,v_Timing); //@sic R5s201105 sic@

DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing), v_MAC_PDUListDL2, cs_NR_HarqProcessAssignment(tsc_HarqProcessId+1)));

//@siclog "Step 9" siclog@

//v_Timing := f_NR_SubFrameTiming_AddSlots(v_Timing2, 3*v_SlotsInframe+5);

//f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Normal,cs_TimingInfo_NR(v_Timing)));

//@sic R5s201105 sic@

v_Timing := f_NR_SubFrameTiming_AddSlots(v_Timing, 4*v_SlotsInSubframe);

v_Timing := f_NR_NextDL_Slot(nr_Cell1,v_Timing);

DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing), v_MAC_PDUListDL2, cs_NR_HarqProcessAssignment(tsc_HarqProcessId+1)));

//@siclog "Step 11" siclog@

// HARQ Retransmision timer =80 ms

v_Timing := f_NR_SubFrameTiming_AddSlots(v_Timing3, 7+tsc_DRX_HARQ_RTT_TimerDL+tsc_DRXRetransTimer_DL-1);

v_Timing := f_NR_LastDL_Slot(nr_Cell1,v_Timing);//@sic R5s201105 sic@

//v_Timing := f_NR_SubFrameTiming_AddSlots(v_Timing2, 8*v_SlotsInframe);

DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing), v_MAC_PDUListDL, cs_NR_HarqProcessAssignment(tsc_HarqProcessId)));

t_Watchdog.start;

//@siclog "Step 4" siclog@

// Check: Does the UE transmit a HARQ NACK for the DL MAC PDU in Step 3?

SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");

//@siclog "Step 6" siclog@

// Check: Does the UE transmit a HARQ NACK for the DL MAC PDU in Step 5?

SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6");

//@siclog "Step 6B" siclog@

t_T7115.start(f_NR_SetTimerToleranceMax(nr_Cell1, l2Timer, 0.05));

alt{

[] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack))

{

t_T7115.stop;

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 6B");

}

[]t_T7115.timeout

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6B");

}

}

//@siclog "Step 8" siclog@

// Check: Does the UE transmit a HARQ NACK for the DL MAC PDU in Step 7?

SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, nack));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8");

//@siclog "Step 8B" siclog@

// Check: Does the UE transmit a HARQ NACK for the DL MAC PDU in Step 8A?

SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8B");

//@siclog "Step 10" siclog@

// Check: Does the UE transmit a HARQ NACK for the DL MAC PDU in Step 9?

SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10");

//@siclog "Step 12" siclog@

// Check: Does the UE transmit a HARQ NACK for the DL MAC PDU in Step 11?

SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 12");//@sic R5s201105 sic@

t_Watchdog.stop;

};

After Change

function f_TC_7_1_1_5_4_NR_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC

{ /* L2/MAC

* 7.1.1.5.4 : DRX Operation / Short cycle configured / DRX command MAC control element reception

*/

const UInt_Type tsc_DrxStartOffset := 7; // in ms @sic R5-202609 sic@

const UInt_Type tsc_HarqProcessId :=0; //Arbitrarily selected

// As per 38.508-1 Table 4.6.3-192: TDD-UL-DL-Config TDD UL DL Periodicity is 5 slots for all SCS configs used in Signalling (15,30,120)

const UInt_Type tsc_DRXRetransTimer_DL := 80;// in slots

const UInt_Type tsc_DRX_HARQ_RTT_TimerDL := 4;// in slots 56/14

var UInt_Type v_ShortDRXCycle :=  80; //@sic R5s200867 sic@

var SubFrameTiming_Type v_Timing  := valueof(cs_SubFrameTiming(0, 0, 0));

var SubFrameTiming_Type v_Timing1 := v_Timing;

var SubFrameTiming_Type v_Timing2 := v_Timing;

var SubFrameTiming_Type v_Timing3 := v_Timing;

var integer v_SlotsInSubframe := f_NR_CellInfo_GetSlotsNumberPerSubframe(nr_Cell1);

var integer v_SlotsInframe := v_SlotsInSubframe * 10;

var NR_MAC_SDU_Type v_EncodedRlcPdu1;

var template (value) NR_MAC_SDU_SubPDU_List_Type v_SDU_SubPDUListDL;

var template (omit) NR_MAC_CE_SubPDU_DL_List_Type v_CE_SubPDUListDL;

var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL;

var template (value) NR_MAC_PDUList_Type v_MAC_PDUListDL2;

var integer v_NR_K1:= 5;

var SubcarrierSpacing v_SCS := f_NR_CellInfo_GetSCS(nr_Cell1);

timer t_T7115 := 0.05;

timer t_Watchdog := 5.0;

select (v_SCS) {

case(kHz15)  { v_NR_K1 := 7; }   /* 7 */

case(kHz30)  { v_NR_K1 := 5; }   /* 5 */

case(kHz120) { v_NR_K1 := 8; }   /* 8 */

case else    { FatalError(__FILE__, __LINE__, "Invalid SubcarrierSpacing"); }

}

// stop periodic UL grants and restart automatic grant allocation instead; periodic TA is kept stopped:

f_NR_ULGrantConfiguration_Start(nr_Cell1,-, -, -, cs_NR_UplinkTimeAlignment_Stop);

// suppress number of DL retransmissions:

f_NR_SS_CellConfig_DciDlInfo_NoRetransmission(nr_Cell1);

// to configure SS to report reception of HARQ ACK or NACK

f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable));

//@siclog "Step 3" siclog@

//f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Error1AndNormal)); //@sic R5s200867 sic@

v_EncodedRlcPdu1 := f_NR_RLC_AMD_FullPDU_Encvalue(0, tsc_NR_P_NoPoll, f_NR_PDCP_PDU_Encvalue(cs_NR_PDCP_PDU_18B(0, crs_NR_PDCP_SDU_36B)));

v_SDU_SubPDUListDL := { cs_NR_MAC_SDU_SubPDU_LI8(tsc_LCID_SCG_Bearer, v_EncodedRlcPdu1) };

v_MAC_PDUListDL := { cs_NR_DL_MAC_PDU_SDU_SubPDU_Padding(v_SDU_SubPDUListDL) };

//@sic R5s200867 sic@

v_Timing1 := f_NR_GetNextSendOccasion_7_1_1_5_X(nr_Cell1, v_ShortDRXCycle, tsc_DrxStartOffset);

//v_Timing1 := f_NR_SubFrameTiming_AddSlots(v_Timing1, v_ShortDRXCycle * v_SlotsInSubframe * 2);

// X= selected the last DL slot, InactivityTimer< X< on-DurationTimer -1

//v_Timing2 := f_NR_SubFrameTiming_AddSlots(v_Timing1, v_SlotsInframe + 9);

// X= selected the last DL slot in Frame 1, as on duration timer =20 ms

//v_Timing2 := f_NR_SubFrameTiming_AddSlots(v_Timing1, f_NR_DRX_LastDL_Slot(nr_Cell1));

// The other things in Step 3 automatically taken care by SS

// step 3 and 7 should be on one HARQ process

DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing1), v_MAC_PDUListDL, cs_NR_HarqProcessAssignment(tsc_HarqProcessId)));

//@siclog "Step 5" siclog@

// 5 ms later configuere SS for normal CRC error mode

//v_Timing := f_NR_SubFrameTiming_AddSlots(v_Timing2, 5);

//f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Normal,cs_TimingInfo_NR(v_Timing)));

// In the Last PDCCH occasion befor on duration timer expiry

v_Timing := f_NR_SubFrameTiming_AddSlots(v_Timing1, v_SlotsInframe); //@sic R5s200926 sic@

//v_Timing3 := f_NR_SubFrameTiming_AddSlots(v_Timing2,v_SlotsInframe);

v_CE_SubPDUListDL := { cs_NR_MAC_CE_SubPDU_DL( cs_NR_MAC_DRXCommandSubHeader ) };  //

v_MAC_PDUListDL2 := { cs_NR_DL_MAC_PDU(v_CE_SubPDUListDL, omit,  cs_NR_MAC_Padding_SubPDU('000000'O)) };   // 2 byte sub headers +3byte Padding header= 40 bits

DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing), v_MAC_PDUListDL2, cs_NR_HarqProcessAssignment(tsc_HarqProcessId+1)));

//k1 =7

v_Timing := f_NR_SubFrameTiming_AddSlots(v_Timing,7);

f_NR_SS_CellConfig_DciDlInfo_NoRetransmission(nr_Cell1,cs_TimingInfo_NR(v_Timing));

//@siclog "Step 6A" siclog@

//@sic R5s200926 sic@

v_Timing := f_NR_SubFrameTiming_AddSlots(v_Timing1, 2*v_SlotsInframe);

DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing), v_MAC_PDUListDL, cs_NR_HarqProcessAssignment(tsc_HarqProcessId+1)));

//@siclog "Step 7" siclog@

v_Timing3 := f_NR_SubFrameTiming_AddSlots(v_Timing1, 3 * v_ShortDRXCycle * v_SlotsInSubframe);

//v_Timing := f_NR_SubFrameTiming_AddSlots(v_Timing2, v_SlotsInframe+5);

f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Error1AndNormal,cs_TimingInfo_NR(v_Timing3)));

// With HARQ RTT = 4slots, and DL-UL Periodicity =5 slots, it should be in the first PDCCH occasion in Drx-RetransmissionTimerDL

//v_Timing3 := f_NR_SubFrameTiming_AddSlots(v_Timing, v_SlotsInframe);

//v_Timing := f_NR_SubFrameTiming_AddSlots(v_Timing2, 2*v_SlotsInframe);

DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing3), v_MAC_PDUListDL, cs_NR_HarqProcessAssignment(tsc_HarqProcessId)));

//@siclog "Step 8A" siclog@

v_Timing := f_NR_SubFrameTiming_AddSlots(v_Timing3, v_NR_K1 + tsc_DRX_HARQ_RTT_TimerDL-1);

v_Timing := f_NR_LastDL_Slot(nr_Cell1,v_Timing); //@sic R5s201105 sic@

DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing), v_MAC_PDUListDL2, cs_NR_HarqProcessAssignment(tsc_HarqProcessId+1)));

//@siclog "Step 9" siclog@

//v_Timing := f_NR_SubFrameTiming_AddSlots(v_Timing2, 3*v_SlotsInframe+5);

//f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestMode_DL_SCH_CRC_REQ(nr_Cell1, Normal,cs_TimingInfo_NR(v_Timing)));

//@sic R5s201105 sic@

v_Timing := f_NR_SubFrameTiming_AddSlots(v_Timing, 4*v_SlotsInSubframe);

v_Timing := f_NR_NextDL_Slot(nr_Cell1,v_Timing);

DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing), v_MAC_PDUListDL2, cs_NR_HarqProcessAssignment(tsc_HarqProcessId+1)));

//@siclog "Step 11" siclog@

// HARQ Retransmision timer =80 ms

v_Timing := f_NR_SubFrameTiming_AddSlots(v_Timing3, v_NR_K1+tsc_DRX_HARQ_RTT_TimerDL+tsc_DRXRetransTimer_DL-1);

v_Timing := f_NR_LastDL_Slot(nr_Cell1,v_Timing);//@sic R5s201105 sic@

//v_Timing := f_NR_SubFrameTiming_AddSlots(v_Timing2, 8*v_SlotsInframe);

DRB.send(cas_NR_DRB_COMMON_REQ_MAC_PDUList(nr_Cell1, p_NR_DRB_Id, cs_TimingInfo_NR(v_Timing), v_MAC_PDUListDL, cs_NR_HarqProcessAssignment(tsc_HarqProcessId)));

t_Watchdog.start;

//@siclog "Step 4" siclog@

// Check: Does the UE transmit a HARQ NACK for the DL MAC PDU in Step 3?

SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");

//@siclog "Step 6" siclog@

// Check: Does the UE transmit a HARQ NACK for the DL MAC PDU in Step 5?

SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6");

//@siclog "Step 6B" siclog@

t_T7115.start(f_NR_SetTimerToleranceMax(nr_Cell1, l2Timer, 0.05));

alt{

[] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack))

{

t_T7115.stop;

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 6B");

}

[]t_T7115.timeout

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6B");

}

}

//@siclog "Step 8" siclog@

// Check: Does the UE transmit a HARQ NACK for the DL MAC PDU in Step 7?

SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, nack));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8");

//@siclog "Step 8B" siclog@

// Check: Does the UE transmit a HARQ NACK for the DL MAC PDU in Step 8A?

SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8B");

//@siclog "Step 10" siclog@

// Check: Does the UE transmit a HARQ NACK for the DL MAC PDU in Step 9?

SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10");

//@siclog "Step 12" siclog@

// Check: Does the UE transmit a HARQ NACK for the DL MAC PDU in Step 11?

SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 12");//@sic R5s201105 sic@

t_Watchdog.stop;

};

## 2.2 Change 2

| Function name | f_TC_7_1_1_5_4_ENDC_NR() |
| --- | --- |
| Reason for change | For ENDC option, short DRX cycle is not getting configured |
| Summary of change | Passing short DRX cycle config to cs_NR_DRX_Config_Def |
| TTCN module | DRX_MAC_ENDC_NR.ttcn |
| MCC160 Comment |  |

Before Change

function f_TC_7_1_1_5_4_ENDC_NR() runs on ENDC_NR_PTC

{// DRX operation / Short cycle configured / Long DRX command MAC control element reception

var DRB_Identity v_DRBId :=tsc_NR_DRB2;

var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig;

var template (value)  NR_RadioBearerList_Type v_SS_Drb_ConfigList  := {cs_NR_SS_RadioBearer_MAC_TestMode(tsc_NR_DRB2, cs_NR_MAC_TestModeInfo_NormalMode)};

var template (value) CellGroupConfig v_CellGroupConfig;

//Init Cell parameters

f_ENDC_NR_Init();

//Create and configure NR cell1

f_NR_CellConfig_Def(nr_Cell1);

// Get NR capabilities sent during EUTRA preamble

f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_Only);

f_NR_AS_CipheringAlgorithm_Set(nea0);

//Configure on SS the SCG DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode

f_NR_ENDC_ConfigAM_UM(nr_Cell1, 1, 0, v_SS_Drb_ConfigList);

// Wait for Trigger from EUTRA indicating start of Test Body

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA);

f_NR_TestBody_Set(true);

//@siclog steps 1-2 logsic@

v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1);

//@sic R5s200928 sic@

v_MAC_CellGroupConfig.drx_Config := cs_NR_DRX_Config_Setup ( cs_NR_DRX_Config_Def( cs_NR_DRX_OnDurationTimerMs40, ms10, 56, 56, sl80, sl80, cs_NR_DRX_LongCycleStartOffsetMs640_7, -, 0));

f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig);

v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_SCG,

-,

-,

v_MAC_CellGroupConfig);

f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_CellGroupConfig);

v_SS_Drb_ConfigList  := {cs_NR_SS_RadioBearer_MAC_TestMode(tsc_NR_DRB2, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDL_UL)};

f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList);

//@siclog steps 3-12 logsic@

f_TC_7_1_1_5_4_NR_TestBody(v_DRBId);

f_NR_TestBody_Set(false);

// Send Trigger to EUTRA indicating Finish of Test Body

f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger);

//Wait from coordination AFTER the connection is released in E-UTRA

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble");

//Postamble

f_NR_ReleaseAllCells();

};

After Change

function f_TC_7_1_1_5_4_ENDC_NR() runs on ENDC_NR_PTC

{// DRX operation / Short cycle configured / Long DRX command MAC control element reception

var DRB_Identity v_DRBId :=tsc_NR_DRB2;

var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig;

var template (value)  NR_RadioBearerList_Type v_SS_Drb_ConfigList  := {cs_NR_SS_RadioBearer_MAC_TestMode(tsc_NR_DRB2, cs_NR_MAC_TestModeInfo_NormalMode)};

var template (value) CellGroupConfig v_CellGroupConfig;

//Init Cell parameters

f_ENDC_NR_Init();

//Create and configure NR cell1

f_NR_CellConfig_Def(nr_Cell1);

// Get NR capabilities sent during EUTRA preamble

f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_Only);

f_NR_AS_CipheringAlgorithm_Set(nea0);

//Configure on SS the SCG DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode

f_NR_ENDC_ConfigAM_UM(nr_Cell1, 1, 0, v_SS_Drb_ConfigList);

// Wait for Trigger from EUTRA indicating start of Test Body

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA);

f_NR_TestBody_Set(true);

//@siclog steps 1-2 logsic@

v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1);

//@sic R5s200928 sic@

v_MAC_CellGroupConfig.drx_Config := cs_NR_DRX_Config_Setup ( cs_NR_DRX_Config_Def( cs_NR_DRX_OnDurationTimerMs40, ms10, 56, 56, sl80, sl80, cs_NR_DRX_LongCycleStartOffsetMs640_7, cs_ShortDRX_Config_715X, 0));

f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig);

v_CellGroupConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_SCG,

-,

-,

v_MAC_CellGroupConfig);

f_NR_SendRRCReconfigurationContentsToEUTRA (omit, v_CellGroupConfig);

v_SS_Drb_ConfigList  := {cs_NR_SS_RadioBearer_MAC_TestMode(tsc_NR_DRB2, cs_NR_MAC_TestModeInfo_NoHeaderManipulationDL_UL)};

f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_SS_Drb_ConfigList);

//@siclog steps 3-12 logsic@

f_TC_7_1_1_5_4_NR_TestBody(v_DRBId);

f_NR_TestBody_Set(false);

// Send Trigger to EUTRA indicating Finish of Test Body

f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger);

//Wait from coordination AFTER the connection is released in E-UTRA

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble");

//Postamble

f_NR_ReleaseAllCells();

};