3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210198

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1413 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for template cs_NR_SS_SpCellConfig |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-11-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The template cs_NR_SS_SpCellConfig sets the field “MAC_CellGroupConfig.R15” and “PhysicalCellGroupConfig.R15” to the value of input paramters which are defined as “template (omit)” and can be passed as “omit”. The field physicalCellGroupConfig in NRSpCellConfig_Type is defined as type NR_ASN1_PhysicalCellGroupConfig_Type, which is defined as : type union NR_ASN1_PhysicalCellGroupConfig_Type { PhysicalCellGroupConfig R15 } Here “R15” is a union element, not an optional field so it cannot legally be set to “omit”. If “R15” is set to omit the resulting value is not fully defined and can give a run-time error if a “valueof” operation is performed. Similarly for type NR_ASN1_MAC_CellGroupConfig_Type. Note: The issue has been exposed following the implementation of MCC comments to R5s201322 which adds “valueof(p_NR_SYSTEM_CTRL_REQ.Request))” in function f_NR_SS_CommonCellConfig(). This gives a run-time error for cases where MAC_CellGroupConfig or PhysicalCellGroupConfig have been set to “omit” in SpCellConfig. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the template paramters to use types MAC_CellGroupConfig and PhysicalCellGroupConfig, instead of NR_ASN1_MAC_CellGroupConfig_Type and NR_ASN1_PhysicalCellGroupConfig_Type. Note : This allows “omit” to be passed into the template. For cases where values are passed for this paramters, “R15” needs to be now set in the calling function or template. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case execution can give run-time errors |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1 Change 1	4

2.2 Change 2	5

2.3 Change 3	6

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2019-06_D20wk38 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## 2.1 Change 1

| Function name | Template cs_NR_SS_SpCellConfig |
| --- | --- |
| Reason for change | The template cs_NR_SS_SpCellConfig sets the field “MAC_CellGroupConfig.R15” and “PhysicalCellGroupConfig.R15” to the value of input paramters defined as “template (omit)” and can be passed as “omit”. The field physicalCellGroupConfig in NRSpCellConfig_Type is defined as type NR_ASN1_PhysicalCellGroupConfig_Type, which is defined as : type union NR_ASN1_PhysicalCellGroupConfig_Type { PhysicalCellGroupConfig R15 } Here “R15” is a union element, not an optional field so it cannot legally be set to “omit”. If “R15” is set to omit the resulting value is not fully defined and can give a run-time error if a “valueof” operation is performed. Similarly for type NR_ASN1_MAC_CellGroupConfig_Type. Note: The issue has been exposed following the implementation of MCC commetns to R5s201322 which adds “valueof(p_NR_SYSTEM_CTRL_REQ.Request))” in function f_NR_SS_CommonCellConfig(). This gives a run-time error for cases where MAC_CellGroupConfig or PhysicalCellGroupConfig have been set to “omit” in SpCellConfig. |
| Summary of change | Change the template paramters to use types MAC_CellGroupConfig and PhysicalCellGroupConfig, instead of NR_ASN1_MAC_CellGroupConfig_Type and NR_ASN1_PhysicalCellGroupConfig_Type. Note : This allows “omit” to be passed into the template. For cases where values are passed for this paramters, “R15” needs to be set in the calling function or template. |
| TTCN module | NR_CellCfg_Templates.ttcn |
| MCC160 Comment |  |

Before Change

template (value) NR_SpCellConfig_Type cs_NR_SS_SpCellConfig(template (omit) NR_ServingCellIndex_Type  p_NR_ServingCellIndex,

template (omit) CellGroupId               p_CellGroupId,

template (omit) MAC_CellGroupConfig       p_MAC_CellGroupConfig,

template (omit) PhysicalCellGroupConfig   p_PhysicalCellGroupConfig) :=

{ /* @status    APPROVED (ENDC, NR5GC, NR5GC_IRAT) */

ServingCellIndex := p_NR_ServingCellIndex,

CellGroupConfig := {

CellGroupId := p_CellGroupId,

SCellList   := {},

MAC_CellGroupConfig:= { R15 := p_MAC_CellGroupConfig},

PhysicalCellGroupConfig := { R15 := p_PhysicalCellGroupConfig}

}

};

After Change

template (value) NR_SpCellConfig_Type cs_NR_SS_SpCellConfig(template (omit) NR_ServingCellIndex_Type  p_NR_ServingCellIndex,

template (omit) CellGroupId               p_CellGroupId,

template (omit) NR_ASN1_MAC_CellGroupConfig_Type       p_MAC_CellGroupConfig,

template (omit) NR_ASN1_PhysicalCellGroupConfig_Type   p_PhysicalCellGroupConfig) :=

{ /* @status    APPROVED (ENDC, NR5GC, NR5GC_IRAT) */

ServingCellIndex := p_NR_ServingCellIndex,

CellGroupConfig := {

CellGroupId := p_CellGroupId,

SCellList   := {},

MAC_CellGroupConfig := p_MAC_CellGroupConfig,

PhysicalCellGroupConfig := p_PhysicalCellGroupConfig

}

};

## 2.2 Change 2

| Function name | New function f_NR_SS_SpCellConfig() |
| --- | --- |
| Reason for change | As change 1 |
| Summary of change | Adding wrapper function f_NR_SS_SpCellConfig to handle MAC_CellGroupConfig and PhysicalCellGroupConfig with both values and “omit” |
| TTCN module |  |
| MCC160 Comment |  |

New Function

function f_NR_SS_SpCellConfig(template (omit) NR_ServingCellIndex_Type  p_NR_ServingCellIndex,

template (omit) CellGroupId               p_CellGroupId,

template (omit) MAC_CellGroupConfig       p_MAC_CellGroupConfig,

template (omit) PhysicalCellGroupConfig   p_PhysicalCellGroupConfig) return template (value) NR_SpCellConfig_Type

{

var NR_ASN1_MAC_CellGroupConfig_Type v_MAC_CellGroupConfig := omit;

var NR_ASN1_PhysicalCellGroupConfig_Type v_PhysicalCellGroupConfig := omit;

if (isvalue (p_MAC_CellGroupConfig))

{

v_MAC_CellGroupConfig := { R15 := p_MAC_CellGroupConfig};

}

if (isvalue (p_PhysicalCellGroupConfig))

{

v_PhysicalCellGroupConfig := { R15 := p_PhysicalCellGroupConfig};

}

return cs_NR_SS_SpCellConfig(p_NR_ServingCellIndex, p_CellGroupId, v_MAC_CellGroupConfig, v_PhysicalCellGroupConfig);

}

## 2.3 Change 3

| Function name | e.g. f_NR_ENDC_ReConfigAM_UM |
| --- | --- |
| Reason for change | As change 1 |
| Summary of change | Replace use of template cs_NR_SS_SpCellConfig with call to new function f_NR_SS_SpCellConfig() Note : Similar changes are needed in functions f_NR_ServingCellConfig_Init(),fl_NR_BuildRRCReconfigExistingDRBs(),f_TC_7_1_1_3_2b_ENDC_NR(),fl_TC_8_2_2_1_1_ENDC_NR_TestBody(), and f_TC_7_1_1_3_2b_NR5GC() |
| TTCN module | e.g. NR_CommonFunctions.ttcn |
| MCC160 Comment |  |

Before Change

function f_NR_ENDC_ReConfigAM_UM(NR_CellId_Type p_CellId,

integer p_NoOfNR_AMBearersN,

integer p_NoOfNR_UMBearersM,

template (omit) NR_RadioBearerList_Type p_SS_Drb_ConfigList := omit,

template (omit) NR_RLC_BearerToAddModList_Type p_RLC_BearerConfigList := omit,

template (omit) SRB_ToAddModList p_SRB_ToAddModList := omit,

template (omit) DRB_ToAddModList p_DRB_ToAddModList := omit,

template (omit) SecurityConfig p_SecurityConfig := omit,

template (omit) MAC_CellGroupConfig p_MAC_CellGroupConfig := omit,

NR_RRC_STATE_Type p_RRC_State := RRC_IDLE,

template (omit) PDSCH_ServingCellConfig.nrofHARQ_ProcessesForPDSCH  p_NrofHARQ_ProcessesForPDSCH := omit

) runs on NR_BASE_PTC // @sic R5s201010 sic@

{

.....................

f_NR_SS_CommonRadioBearerConfig(p_CellId, v_SS_Drb_ConfigList);

v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(p_SRB_ToAddModList, v_DRB_ToAddModList, -, v_SecurityConfig);

v_SCGConfig := f_NR_GetSCGConfigDef(p_CellId);

if (isvalue(p_MAC_CellGroupConfig)) {

v_SCGConfig.mac_CellGroupConfig := p_MAC_CellGroupConfig;

f_NR_SS_ServingCellConfig(p_CellId, cs_TimingInfo_Now, cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, p_MAC_CellGroupConfig, omit)));

}

if (isvalue(p_NrofHARQ_ProcessesForPDSCH)) {

v_SCGConfig.spCellConfig.spCellConfigDedicated.pdsch_ServingCellConfig := cs_NR_Pdsch_ServingCellConfigSetup(cs_NR_PDSCH_ServingCellConfigDef(-, -, p_NrofHARQ_ProcessesForPDSCH));

f_NR_SS_CommonCellConfig(p_CellId, cads_NR_PDSCH_Config_REQ(p_CellId, cs_TimingInfo_Now, cs_NR_PDSCH_CellLevelConfigDef(v_SCGConfig.spCellConfig.spCellConfigDedicated.pdsch_ServingCellConfig.setup)));

}

v_SCGConfig.rlc_BearerToAddModList := v_RLC_BearerConfigList;

f_NR_SendRRCReconfigurationContentsToEUTRA(v_RadioBearerConfig, v_SCGConfig, -, -, false, reconfigurationWithSync_CheckRACH);  //@sic R5s190541, R5s200647 sic@

// Get Security info from EUTRA, calculate latest keys and configure SS

// as only need to do this for the first reconfiguration @sic R5s200647, R5s201010 sic@

if (p_RRC_State != RRC_CONNECTED)

{

f_ENDC_GetKeysActivateSecurity(p_CellId, EUTRA);

}

//RRCReconfigurationComplete not yet received from E-UTRA - Wait for it @sic R5s200647 sic@

EUTRA.receive(cmr_EUTRA_NR_OctetData ("Complete")) -> value v_Coordination_MSG_Value;

f_NR_CheckReceived_RRCReconfigurationComplete(valueof(v_Coordination_MSG_Value));

}

After Change

function f_NR_ENDC_ReConfigAM_UM(NR_CellId_Type p_CellId,

integer p_NoOfNR_AMBearersN,

integer p_NoOfNR_UMBearersM,

template (omit) NR_RadioBearerList_Type p_SS_Drb_ConfigList := omit,

template (omit) NR_RLC_BearerToAddModList_Type p_RLC_BearerConfigList := omit,

template (omit) SRB_ToAddModList p_SRB_ToAddModList := omit,

template (omit) DRB_ToAddModList p_DRB_ToAddModList := omit,

template (omit) SecurityConfig p_SecurityConfig := omit,

template (omit) MAC_CellGroupConfig p_MAC_CellGroupConfig := omit,

NR_RRC_STATE_Type p_RRC_State := RRC_IDLE,

template (omit) PDSCH_ServingCellConfig.nrofHARQ_ProcessesForPDSCH  p_NrofHARQ_ProcessesForPDSCH := omit

) runs on NR_BASE_PTC // @sic R5s201010 sic@

{

.............

f_NR_SS_CommonRadioBearerConfig(p_CellId, v_SS_Drb_ConfigList);

v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(p_SRB_ToAddModList, v_DRB_ToAddModList, -, v_SecurityConfig);

v_SCGConfig := f_NR_GetSCGConfigDef(p_CellId);

if (isvalue(p_MAC_CellGroupConfig)) {

v_SCGConfig.mac_CellGroupConfig := p_MAC_CellGroupConfig;

f_NR_SS_ServingCellConfig(p_CellId, cs_TimingInfo_Now, cs_NR_ServingCellConfig_SpCell(f_NR_SS_SpCellConfig(omit, omit, p_MAC_CellGroupConfig, omit)));

}

if (isvalue(p_NrofHARQ_ProcessesForPDSCH)) {

v_SCGConfig.spCellConfig.spCellConfigDedicated.pdsch_ServingCellConfig := cs_NR_Pdsch_ServingCellConfigSetup(cs_NR_PDSCH_ServingCellConfigDef(-, -, p_NrofHARQ_ProcessesForPDSCH));

f_NR_SS_CommonCellConfig(p_CellId, cads_NR_PDSCH_Config_REQ(p_CellId, cs_TimingInfo_Now, cs_NR_PDSCH_CellLevelConfigDef(v_SCGConfig.spCellConfig.spCellConfigDedicated.pdsch_ServingCellConfig.setup)));

}

v_SCGConfig.rlc_BearerToAddModList := v_RLC_BearerConfigList;

f_NR_SendRRCReconfigurationContentsToEUTRA(v_RadioBearerConfig, v_SCGConfig, -, -, false, reconfigurationWithSync_CheckRACH);  //@sic R5s190541, R5s200647 sic@

// Get Security info from EUTRA, calculate latest keys and configure SS

// as only need to do this for the first reconfiguration @sic R5s200647, R5s201010 sic@

if (p_RRC_State != RRC_CONNECTED)

{

f_ENDC_GetKeysActivateSecurity(p_CellId, EUTRA);

}

//RRCReconfigurationComplete not yet received from E-UTRA - Wait for it @sic R5s200647 sic@

EUTRA.receive(cmr_EUTRA_NR_OctetData ("Complete")) -> value v_Coordination_MSG_Value;

f_NR_CheckReceived_RRCReconfigurationComplete(valueof(v_Coordination_MSG_Value));

}