3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210199

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1415 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC test case 7.1.1.10.1 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Datang Linktester |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-12-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add Rel-15 NR5GC test case 7.1.1.10.1 to the NR5GC ATS |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC test case 7.1.1.10.1 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.10.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

3	Branches executed	7

4	Execution Log Files	7

4.1	HiSilicon Balong 5000	7

5	References	7

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 7.1.1.10.1 which is part of the NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Fu Wang
	wangfu@datangmobile.cn

## Verification Test Summary

Test Case: 	7.1.1.10.1

ATS Version:	iwd-TTCN3-B2019-06_D20wk38

System Simulator used:	Datang LinkTester 5G Protocol Conformance Test System ECT9610

UE used:	HiSilicon Balong 5000

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_10_1_NR5GC |
| --- | --- |
| Reason for change | - According to 38.523-1 7.1.1.10.1.3.3, dataInactivityTimer should be set to s10 in preamble. - To match the change 2.5, the UE will be in idle state before postamble. |
| Summary of change | - Set dataInactivityTimer to s10 in preamble. - Change STATE_CONNECTED_3A to STATE_IDLE_1A in postamble. |
| TTCN module | MAC_NR5GC.ttcn |
| MCC160 Comment |  |

Before change

function f_TC_7_1_1_10_1_NR5GC() runs on NR5GC_PTC

{ //DataInactivityTimer expiry

var DRB_Identity v_DRBId;

var template (omit)  NR_RadioBearerList_Type v_SS_Drb_ConfigList;

//Init Cell parameters

f_NR5GC_Init(NR_1);

//Create and configure NR cell1

f_NR_CellConfig_Def(nr_Cell1);

// Get NR capabilities sent during EUTRA preamble

f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeB_ON);

//Retrieve DRB Id of the first PDU session

v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession();

v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, v_DRBId, -, -, -, cs_SDAP_Configuration_None)};

//Configure on SS the DRB No SDAP and RLC/MAC/PDCP normal mode

f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1,

TEST_LOOPModeB_ON,

-,

'06'O,  //p_IpPduDelayTime

-,

-,  //MAC_CellGroupConfig

-,

v_SS_Drb_ConfigList

);

f_NR_TestBody_Set(true);

// @siclog "Step 1-14" siclog@

f_TC_7_1_1_10_1_NR_TestBody(v_DRBId );

f_NR_TestBody_Set(false);

//Postamble

f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A);

};

After change

function f_TC_7_1_1_10_1_NR5GC() runs on NR5GC_PTC

{ //DataInactivityTimer expiry

var DRB_Identity v_DRBId;

var template (omit)  NR_RadioBearerList_Type v_SS_Drb_ConfigList;

//Init Cell parameters

f_NR5GC_Init(NR_1);

//Create and configure NR cell1

f_NR_CellConfig_Def(nr_Cell1);

// Get NR capabilities sent during EUTRA preamble

f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeB_ON);

//Retrieve DRB Id of the first PDU session

v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession();

v_SS_Drb_ConfigList := {f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, v_DRBId, -, -, -, cs_SDAP_Configuration_None)};

//Configure on SS the DRB No SDAP and RLC/MAC/PDCP normal mode

f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1,

TEST_LOOPModeB_ON,

-,

'06'O,  //p_IpPduDelayTime

-,

cs_NR_MAC_CellGroupConfig(-, -, -, -, -, -, cs_NR_DataInactivityTimer_Setup(s10)),  //MAC_CellGroupConfig

-,

v_SS_Drb_ConfigList

);

f_NR_TestBody_Set(true);

// @siclog "Step 1-14" siclog@

f_TC_7_1_1_10_1_NR_TestBody(v_DRBId );

f_NR_TestBody_Set(false);

//Postamble

f_NR_Postamble(nr_Cell1, STATE_IDLE_1A);

};

## Change 2

| Function name | f_TC_7_1_1_10_1_NR_TestBody |
| --- | --- |
| Reason for change | - Step 1 of loop 2 should to wait until the end of loop 1. - In preamble it is said that IP PDU delay shall be set to 6 seconds. - At step 3, UE may receives DL RLC PDU with P field set to 1, then it will send SR to request UL resource for RLC Status PDU during test mode B delay timer 6s running. As a result UE may fail the check in step 4. - Beacause sdap UL header is configured for the UE,sending PDCP data inencryption mode will lead to different PDCP data between uplink and downlink - According to TS 38.331 clause 5.3.8.5 and 5.3.11, upon receiving the expiry of DataInactivityTimer ,UE in CONNECTED state shall go to IDLE state and indicate the release of the RRC connection to upper layers together with the release cause ‘'RRC connection failure’.Acoording to TS 24.501 clause 5.5.1.3.2 case f, when the UE receives an indication of "RRC Connection failure" from the lower layers and does not have signalling pending, UE shall initiate mobility and periodic registration update procedure. Thus, generic test procedure in TS 38.508-1 [4] subclause Table 4.9.5.2.2-1 shall be used but not Table 4.9.4 at step 14 Note： CR R5-206322 Agreed. |
| Summary of change | - Move f_NR_GetNextSendOccasion_DL into loop. - Set t_Watchdog to f_NR_SetTimerToleranceMin(nr_Cell1, l2Timer, 6.0). - Delete f_NR_SS_SystemIndCtrlConfig and SR receiving step. - Using f_IPv4IPv6_IcmpEchoReply to construct sdap data and send. - Add _NR_RRCRelease_Local after step 13, replace f_NR5GC_CheckRRCIdle with f_NR5GC_CheckCampedOnNewCell. |
| TTCN module | MAC_NR5GC.ttcn |
| MCC160 Comment |  |

Before change

function f_TC_7_1_1_10_1_NR_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR5GC_PTC

{

var integer j;

var SubFrameTiming_Type v_TimingDL;

timer t_Watchdog := 10.0;

// Enable SR reception indication

f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(enable));

v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1,100); // Timing of DL

for (j := 0; j < 2; j:= j+1) {

// @siclog "Step 1 - 3, 7-9 " siclog@

v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 6000);

DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_SDUList(nr_Cell1,

p_NR_DRB_Id,

//cs_RlcBearerRouting_NR(nr_Cell1),

cs_TimingInfo_NR(v_TimingDL),

{crs_NR_PDCP_SDU_37B} ));

t_Watchdog.start;

// @siclog "Step 4, 10" siclog@

SYSIND.receive (car_NR_SchedulingReq_IND (nr_Cell1, ?));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4, 10");

// @siclog "Step 5-6, 11-12" siclog@

DRB.receive (car_NR_DRB_COMMON_IND_PDCP_SDUList(nr_Cell1, p_NR_DRB_Id, -, {crs_NR_PDCP_SDU_37B}));

}

//Disable SR reception indication

f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable));

//@siclog "Step 13 " siclog@

f_Delay (10.0);

//@siclog "Step 14" siclog@

//Check: does the test result of generic test procedure in TS 38.508-1 [4] subclause 4.9.4 indicate that the UE is in RRC_IDLE?

f_NR5GC_CheckRRCIdle(nr_Cell1, noRrcConnectionRelease);

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14");

};

After change

function f_TC_7_1_1_10_1_NR_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR5GC_PTC

{

var integer j;

var SubFrameTiming_Type v_TimingDL;

timer t_Watchdog := f_NR_SetTimerToleranceMin(nr_Cell1, l2Timer, 6.0);

var template (value) QosFlow_Identification_Type v_QosFlowId := f_NR_GetQosFlowIdentification_ForDRBId(p_NR_DRB_Id);

var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE());

for (j := 0; j < 2; j:= j+1) {

// @siclog "Step 1 - 3, 7-9 " siclog@

v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1,6000);

DRB.send(cas_NR_DRB_COMMON_REQ_SDAP_SDUList(nr_Cell1, v_QosFlowId, {v_IPData}, cs_TimingInfo_NR(v_TimingDL)));

t_Watchdog.start;

t_Watchdog.timeout;

// @siclog "Step 4, 10" siclog@

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4, 10");

// @siclog "Step 5-6, 11-12" siclog@

DRB.receive (car_NR_DRB_COMMON_IND_SDAP_SDUList(nr_Cell1, v_QosFlowId, {v_IPData}));

}

//@siclog "Step 13 " siclog@

f_Delay (10.0);

f_NR_RRCRelease_Local(nr_Cell1, f_NR_GetNextSendOccasion_DL(nr_Cell1));

//@siclog "Step 14" siclog@

//Check: does the test result of generic test procedure in TS 38.508-1 [4] subclause 4.9.4 indicate that the UE is in RRC_IDLE?

f_NR5GC_CheckCampedOnNewCell(nr_Cell1);

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 14");

};

# Branches executed

NR Band 41,Nea2,Nia2.

# Execution Log Files

## HiSilicon Balong 5000

The HiSilicon Balong 5000 passed this test case on Datang LinkTester 5G Protocol Conformance Test System ECT9610. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test Case Execution log file:
\ TC_7_1_1_10_1_NR5GC_N41.log

- PIXCIT settings used:
\ Pics(HiSilicon).xml

\ Pixit(TC_7_1_1_10_1_NR5GC).xml

# References

| [1] | R5s201523: Supporting information for agreement of NR5GC test case 7.1.1.10.1 in FR1 |
| --- | --- |