3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210203

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1434 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for NR PDCP integrity and ciphering test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-11-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For these test cases the common test case body functions f_TC_7_1_3_2_X_NR_TestBody() and f_TC_7_1_3_3_X_NR_TestBody() have test steps applicable for either EN-DC or NR5GC operation selected by checks on PICS paramters e.g. pc_EN_DC. Since these are UE capabilities it is possible that a UE may support more than 5G deplorment option, e.g. pc_EN_DC set as TRUE does not necessarily mean the test case is executing in EN-DC mode. This can cause the test steps to be executed incorrectly causing test case failure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Since there are no there are no actually common test steps in these test cases, the steps in f_TC_7_1_3_2_X_NR_TestBody() and f_TC_7_1_3_3_X_NR_TestBody() can be moved into the option-specific calling functions, to avoid selecting test steps based on PICS parameter settings. Note : Associated prose CRs R5-205163 and R5-205164 are raised at RAN5#89. Note : Following the changes the common test body functions f_TC_7_1_3_2_X_NR_TestBody() and f_TC_7_1_3_3_X_NR_TestBody() are no longer referenced and can be deleted. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail these test cases due to some steps being executed when they are not applicable for the current operation mode |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.2.1.ENDC, 7.1.3.2.1.NR5GC, 7.1.3.2.2.ENDC, 7.1.3.2.2.NR5GC, 7.1.3.2.3.ENDC. 7.1.3.2.3.NR5GC, 7.1.3.3.1.ENDC, 7.1.3.3.1.NR5GC, 7.1.3.3.2.ENDC, 7.1.3.3.2.NR5GC, 7.1.3.3.3.ENDC, 7.1.3.3.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.523-1 R5-205163, R5-205164 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1 Change 1	4

2.2 Change 2	6

2.3 Change 3	8

2.4 Change 4	9

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2019-06_D20wk38 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## 2.1 Change 1

| Function name | fl_ENDC_PDCP_Integrity() |
| --- | --- |
| Reason for change | For these test cases the common test case body functions f_TC_7_1_3_2_X_NR_TestBody() and f_TC_7_1_3_3_X_NR_TestBody() have test steps applicable for either EN-DC or NR5GC operation selected by checks on PICS paramters e.g. pc_EN_DC. Since these are UE capabilities it is possible that a UE may support more than 5G deplorment option, e.g. pc_EN_DC set as TRUE does not necessarily mean the test case is executing in EN-DC mode. This can cause the test steps to be executed incorrectly causing test case failure. |
| Summary of change | Move implementation of steps 2a1 – 2a4 from f_TC_7_1_3_2_X_NR_TestBody() into this function |
| TTCN module | PDCP_ENDC_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_ENDC_PDCP_Integrity(IntegrityProtAlgorithm p_IntegrityProAlgorithm) runs on ENDC_NR_PTC

{

var ENDC_SRB_DRB_Configuration_Type v_SecondaryConfig:= NR_PDCP_SRB1_2;

var template (value) FreqBandList v_MRDC_FreqList;

var octetstring v_OctetData;

var integer v_EUTRABandInd;

var integer v_NRBandInd;

if (pc_srb3){

v_SecondaryConfig := NR_PDCP_SRB1_2_SRB3;

}

f_ENDC_NR_Init();

//Create and configure NR cell1

f_NR_CellConfig_Def(nr_Cell1);

f_NR_AS_IntegrityAlgorithm_Set(p_IntegrityProAlgorithm);

// Get NR capabilities sent during EUTRA preamble

// @sic R5-195339 sic@

f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_SCG); //Configure DRB2 as normal

f_NR_RRC_PSCellChange_SRB_NR_PDCP(nr_Cell1, v_SecondaryConfig);

f_NR_TestBody_Set(true);

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA);

// Have to receive EUTRA band first @sic R5s190476 change 4.22 sic@

v_OctetData := f_EUTRA_NR_WaitForCoOrd_Data(EUTRA, "EUTRA Band");

v_EUTRABandInd := oct2int(v_OctetData);

v_NRBandInd := f_NR_CellInfo_GetBandIndicator(nr_Cell1);

// Then send both bands under test back to EUTRA

v_MRDC_FreqList := cs_NR_MRDC_FreqList(v_EUTRABandInd, v_NRBandInd);

f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_OctetData (bit2oct(encvalue(v_MRDC_FreqList)), "MRDC_Freq"));

f_TC_7_1_3_2_X_NR_TestBody(tsc_NR_DRB2);

f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger); //@sic R5s190476 sic@ ch 4.6

f_NR_TestBody_Set(false);

//Wait from coordination AFTER the connection is released in E-UTRA

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble");

//Postamble

f_NR_ReleaseAllCells();

}

After Change:

function fl_ENDC_PDCP_Integrity(IntegrityProtAlgorithm p_IntegrityProAlgorithm) runs on ENDC_NR_PTC

{

var ENDC_SRB_DRB_Configuration_Type v_SecondaryConfig:= NR_PDCP_SRB1_2;

var template (value) FreqBandList v_MRDC_FreqList;

var octetstring v_OctetData;

var integer v_EUTRABandInd;

var integer v_NRBandInd;

var template (value) RadioBearerConfig v_RadioBearerConfig;

var NR_SecurityParams_Type v_Security_Params := f_NR_Security_Get();

var IntegrityProtAlgorithm v_IntegrityAlg:= v_Security_Params.AS_Integrity.Algorithm;

var boolean v_DontWaitForComplete:=false;

if (pc_srb3){

v_SecondaryConfig := NR_PDCP_SRB1_2_SRB3;

}

f_ENDC_NR_Init();

//Create and configure NR cell1

f_NR_CellConfig_Def(nr_Cell1);

f_NR_AS_IntegrityAlgorithm_Set(p_IntegrityProAlgorithm);

// Get NR capabilities sent during EUTRA preamble

// @sic R5-195339 sic@

f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_SCG); //Configure DRB2 as normal

f_NR_RRC_PSCellChange_SRB_NR_PDCP(nr_Cell1, v_SecondaryConfig);

f_NR_TestBody_Set(true);

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA);

// Have to receive EUTRA band first @sic R5s190476 change 4.22 sic@

v_OctetData := f_EUTRA_NR_WaitForCoOrd_Data(EUTRA, "EUTRA Band");

v_EUTRABandInd := oct2int(v_OctetData);

v_NRBandInd := f_NR_CellInfo_GetBandIndicator(nr_Cell1);

// Then send both bands under test back to EUTRA

v_MRDC_FreqList := cs_NR_MRDC_FreqList(v_EUTRABandInd, v_NRBandInd);

f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_OctetData (bit2oct(encvalue(v_MRDC_FreqList)), "MRDC_Freq"));

//REMOVED f_TC_7_1_3_2_X_NR_TestBody(tsc_NR_DRB2);

v_RadioBearerConfig:= cs_38508_RadioBearerConfigDef({ cs_NR_SRB_ToAddMod(tsc_SRB3) }, -, -, cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig(nea2, v_IntegrityAlg), secondary));

//@siclog "Step 2a1-2a2" siclog@

//Reconfigure NR MAC sent on SRB3 integrity protected

f_NR_PDCP_SendRRCReconfiguration(nr_Cell1,-, v_RadioBearerConfig);

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2a2");

//@siclog "Step 2a3-a4" siclog@

//SS sends MAC-I Corrupted, configure SS to use different integrity algorithm so that will result in MAC-I failure at UE

f_NR_SS_TriggerMAC_I_Failure(nr_Cell1, tsc_NR_RbId_SRB3);

f_NR_PDCP_SendRRCReconfiguration(nr_Cell1, -, cs_NR_RadioBearer(-,-,-,v_IntegrityAlg), v_DontWaitForComplete);

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA);

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2a4");

f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger); //@sic R5s190476 sic@ ch 4.6

f_NR_TestBody_Set(false);

//Wait from coordination AFTER the connection is released in E-UTRA

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble");

//Postamble

f_NR_ReleaseAllCells();

}

## 2.2 Change 2

| Function name | fl_ENDC_PDCP_Ciphering() |
| --- | --- |
| Reason for change | For these test cases the common test case body functions f_TC_7_1_3_2_X_NR_TestBody() and f_TC_7_1_3_3_X_NR_TestBody() have test steps applicable for either EN-DC or NR5GC operation selected by checks on PICS paramters e.g. pc_EN_DC. Since these are UE capabilities it is possible that a UE may support more than 5G deplorment option, e.g. pc_EN_DC set as TRUE does not necessarily mean the test case is executing in EN-DC mode. This can cause the test steps to be executed incorrectly causing test case failure. |
| Summary of change | Move implementation of steps 2a1 – 2a2 from f_TC_7_1_3_3_X_NR_TestBody() into this function |
| TTCN module | PDCP_ENDC_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_ENDC_PDCP_Ciphering(CipheringAlgorithm p_CipheringAlgorithm) runs on ENDC_NR_PTC

{

f_ENDC_NR_Init();

//Create and configure NR cell1

f_NR_CellConfig_Def(nr_Cell1);

f_NR_AS_CipheringAlgorithm_Set(p_CipheringAlgorithm);

// Get NR capabilities sent during EUTRA preamble

// @sic R5-195340 sic@

f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_SCG); //Configure DRB2 as normal

f_NR_RRC_PSCellChange_SRB_NR_PDCP(nr_Cell1, NR_PDCP_MCG_SRB1_2);

// @sic R5s190918 sic@

f_NR_TestBody_Set(true);

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA);

f_TC_7_1_3_3_X_NR_TestBody(tsc_NR_DRB2);

f_NR_TestBody_Set(false);

f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger);  //@sic R5s190554 sic@ ch 4.15

//Wait from coordination AFTER the connection is released in E-UTRA

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble");

//Postamble

f_NR_ReleaseAllCells();

}

After Change:

function fl_ENDC_PDCP_Ciphering(CipheringAlgorithm p_CipheringAlgorithm) runs on ENDC_NR_PTC

{

var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE());

var DRB_Identity v_DrbId:= p_DRbId;

f_ENDC_NR_Init();

//Create and configure NR cell1

f_NR_CellConfig_Def(nr_Cell1);

f_NR_AS_CipheringAlgorithm_Set(p_CipheringAlgorithm);

// Get NR capabilities sent during EUTRA preamble

// @sic R5-195340 sic@

f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_SCG); //Configure DRB2 as normal

f_NR_RRC_PSCellChange_SRB_NR_PDCP(nr_Cell1, NR_PDCP_MCG_SRB1_2);

// @sic R5s190918 sic@

f_NR_TestBody_Set(true);

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA);

//REMOVED f_TC_7_1_3_3_X_NR_TestBody(tsc_NR_DRB2);

DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_SDUList(nr_Cell1, tsc_NR_DRB2, -, {v_IPData})); //@sic R5s190554 sic@

//@siclog "Step 2a2" siclog@

//Does UE transmit a PDCP Data PDU with SN=K for the first iteration?

DRB.receive(car_NR_DRB_COMMON_IND_PDCP_SDUList_DC(nr_Cell1, tsc_NR_DRB2, cr_RlcBearerRouting_NR(nr_Cell1), -, {v_IPData}));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2a2");

f_NR_TestBody_Set(false);

f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_Trigger);  //@sic R5s190554 sic@ ch 4.15

//Wait from coordination AFTER the connection is released in E-UTRA

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble");

//Postamble

f_NR_ReleaseAllCells();

}

## 2.3 Change 3

| Function name | f_TC_7_1_3_2_X_TestBody_NR |
| --- | --- |
| Reason for change | For these test cases the common test case body functions f_TC_7_1_3_2_X_NR_TestBody() and f_TC_7_1_3_3_X_NR_TestBody() have test steps applicable for either EN-DC or NR5GC operation selected by checks on PICS paramters e.g. pc_EN_DC. Since these are UE capabilities it is possible that a UE may support more than 5G deplorment option, e.g. pc_EN_DC set as TRUE does not necessarily mean the test case is executing in EN-DC mode. This can cause the test steps to be executed incorrectly causing test case failure. |
| Summary of change | - Remove redundant check so that steps 1b1 – 1b2 are always executed - Move implementation of steps 3a1 – 3a2 from f_TC_7_1_3_2_X_NR_TestBody() into this function |
| TTCN module | PDCP_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_7_1_3_2_X_TestBody_NR (DRB_Identity p_RB_Id) runs on NR_BASE_PTC

{

if(pc_EN_DC or pc_NGEN_DC){ //@sic R5s191113 sic@

//@siclog "Step 1a1-1a2" siclog@

//The SS sends EUTRA RRC - Done in EUTRA PTC

}else{

//@siclog "Step 1b1-1b2" siclog@

//The SS sends NR RRC

f_NR_Capability(nr_Cell1);

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 1b2");

}

f_TC_7_1_3_2_X_NR_TestBody(p_RB_Id);

}

After Change:

function f_TC_7_1_3_2_X_TestBody_NR (DRB_Identity p_RB_Id) runs on NR_BASE_PTC

{

var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE());

/*REMOVED  if(pc_EN_DC or pc_NGEN_DC){ //@sic R5s191113 sic@

//@siclog "Step 1a1-1a2" siclog@

//The SS sends EUTRA RRC - Done in EUTRA PTC

}else{ */

//@siclog "Step 1b1-1b2" siclog@

//The SS sends NR RRC

f_NR_Capability(nr_Cell1);

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 1b2");

//REMOVED }

//REMOVED f_TC_7_1_3_2_X_NR_TestBody(p_RB_Id);

//@siclog "Step 3a1" siclog@

DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_SDUList(nr_Cell1, p_RB_Id, -, {v_IPData}));

//@siclog "Step 3a2" siclog@

//Does UE transmit a PDCP Data PDU with SN=K for the first iteration?

DRB.receive(car_NR_DRB_COMMON_IND_PDCP_SDUList_DC(nr_Cell1, p_RB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, {v_IPData}));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3a2"); //@sic R5s200557 sic@

}

## 2.4 Change 4

| Function name | fl_NR5GC_PDCP_Ciphering() |
| --- | --- |
| Reason for change | For these test cases the common test case body functions f_TC_7_1_3_2_X_NR_TestBody() and f_TC_7_1_3_3_X_NR_TestBody() have test steps applicable for either EN-DC or NR5GC operation selected by checks on PICS paramters e.g. pc_EN_DC. Since these are UE capabilities it is possible that a UE may support more than 5G deplorment option, e.g. pc_EN_DC set as TRUE does not necessarily mean the test case is executing in EN-DC mode. This can cause the test steps to be executed incorrectly causing test case failure. |
| Summary of change | Move implementation of steps 3 -4 from f_TC_7_1_3_3_X_NR_TestBody() into this function |
| TTCN module | PDCP_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_NR5GC_PDCP_Ciphering(CipheringAlgorithm p_CipheringAlgorithm) runs on NR5GC_PTC

{

var DRB_Identity v_DRBId;

var template (value) NR_RadioBearer_Type v_SS_Drb_Config;

f_NR5GC_Init_NAS(NR_1);

//Create and configure NR cell1

f_NR_CellConfig_Def(nr_Cell1);

//Initialize PDCP Parameters

f_NR_AS_CipheringAlgorithm_Set(p_CipheringAlgorithm);

//Preamble

f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A, TESTModeA_ON);

v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession();

v_SS_Drb_Config := f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, v_DRBId, -, -, -, cs_SDAP_Configuration_None);//@sic R5s190383 sic@

f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1,

TEST_LOOPModeA_ON,

cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling,

-,

-,

-,

-,

{v_SS_Drb_Config}); //@sic R5s190383 sic@

f_NR_TestBody_Set(true);

//@siclog "Step 1b1-1b2" siclog@

f_NR_Capability(nr_Cell1);

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 1b2");

f_TC_7_1_3_3_X_NR_TestBody(v_DRBId);

f_NR_TestBody_Set(false);

f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1);

//Postamble

f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A);

f_NR_ReleaseAllCells();

}

After Change:

function fl_NR5GC_PDCP_Ciphering(CipheringAlgorithm p_CipheringAlgorithm) runs on NR5GC_PTC

{

var DRB_Identity v_DRBId;

var template (value) NR_RadioBearer_Type v_SS_Drb_Config;

var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE());

f_NR5GC_Init_NAS(NR_1);

//Create and configure NR cell1

f_NR_CellConfig_Def(nr_Cell1);

//Initialize PDCP Parameters

f_NR_AS_CipheringAlgorithm_Set(p_CipheringAlgorithm);

//Preamble

f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A, TESTModeA_ON);

v_DRBId := f_NR_GetDefaultDRB_ForFirstPDUSession();

v_SS_Drb_Config := f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, v_DRBId, -, -, -, cs_SDAP_Configuration_None);//@sic R5s190383 sic@

f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1,

TEST_LOOPModeA_ON,

cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling,

-,

-,

-,

-,

{v_SS_Drb_Config}); //@sic R5s190383 sic@

f_NR_TestBody_Set(true);

//@siclog "Step 1b1-1b2" siclog@

f_NR_Capability(nr_Cell1);

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 1b2");

//REMOVED  f_TC_7_1_3_3_X_NR_TestBody(v_DRBId);

//SS transmits PDCP PDU on MCG DRB ciphered.

DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_SDUList(nr_Cell1, v_DRBId, -, {v_IPData}));//@sic R5s190554 sic@

//@siclog "Step 4" siclog@

//Does UE transmit a PDCP Data PDU with SN=K for the first iteration?

DRB.receive(car_NR_DRB_COMMON_IND_PDCP_SDUList_DC(nr_Cell1, v_DRBId, cr_RlcBearerRouting_NR(nr_Cell1), -, {v_IPData}));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4");

f_NR_TestBody_Set(false);

f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1);

//Postamble

f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A);

f_NR_ReleaseAllCells();

}