3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210204

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1438 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to f_Check_NG_PDUSessionModificationReq in 20wk43 IWD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-11-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The fields EPS_Bearer, PdnIndex and Emergency have been added in type GSM_MobilityInfo_Type. Out of these, EPS_Bearer and PdnIndex are now being set to OMIT as a result of R5s201533 (Change 1). However, the IE Emergency is still uninitialized. This leads to a run time error and needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Initialise Emergency IE in p_GSM_MobilityInfo. Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Runtime errors will occur causing the testcase to fail a conformant UE unfairly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.5.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	3

1.1	Correction to f_Check_NG_PDUSessionModificationReq	3

# Corrections required

## Correction to f_Check_NG_PDUSessionModificationReq

| Function name | f_Check_NG_PDUSessionModifictionReq |
| --- | --- |
| Reason for change | The fields EPS_Bearer, PdnIndex and Emergency have been added in type GSM_MobilityInfo_Type. Out of these, EPS_Bearer and PdnIndex are now being set to OMIT as a result of R5s201533. However, the IE Emergency is still uninitialized. This leads to a run time error and needs to be corrected. |
| Summary of change | Initialise Emergency IE in p_GSM_MobilityInfo. Please see below. |
| TTCN module | Common\NG_NAS\NG_NASTemplateFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> if (ispresent (v_GSMMsg.pdu_Session_Modification_Request.extdProtocolConfigurationOptions)) { p_GSM_MobilityInfo.PCO := v_GSMMsg.pdu_Session_Modification_Request.extdProtocolConfigurationOptions; } else { p_GSM_MobilityInfo.PCO := omit; } p_GSM_MobilityInfo.EPS_Bearer :=omit; //WA#R5s201533 p_GSM_MobilityInfo.PdnIndex := omit; //WA#R5s201533 return v_Result; }

After Change:

<<SKIPPED CODE>> if (ispresent (v_GSMMsg.pdu_Session_Modification_Request.extdProtocolConfigurationOptions)) { p_GSM_MobilityInfo.PCO := v_GSMMsg.pdu_Session_Modification_Request.extdProtocolConfigurationOptions; } else { p_GSM_MobilityInfo.PCO := omit; } p_GSM_MobilityInfo.EPS_Bearer :=omit; //WA#R5s201533 p_GSM_MobilityInfo.PdnIndex := omit; //WA#R5s201533 p_GSM_MobilityInfo.Emergency := omit; //WA#IRAT_Batch2 return v_Result; }