3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210205

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1439 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcases 10.1.6.1 and 10.1.6.2 for 20wk43 IWD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-11-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation of testcase 10.1.6.1, at Step 4 for checking the contents of PDU Session Release REQ, variable v_PDUSessionIdToRelease is passed. Earlier, this variable is assigned like this : var integer v_SessionId := f_NR5GC_MobileInfo_GetNoOfSessions()+1; // @sic R5s201362 sic@ var O1_Type v_PDUSessionIdToRelease := int2oct(v_SessionId, 1); This sets the value for this variable to ‘02’O. However, a UE may not trigger a PDU Session with this Id, as this is only the cid used in AT command. The UEs can map this to any number and use that number as the PDU Session ID in the PDU Session Etablishment REQ. Thus, when the UE sends a PDU Session Release REQ, it fails as the SS expects PDU Sesion Identity as ‘02’O, whereas the UE can send a different value. Same issue is observed in Testcase 10.1.6.2 as well. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Before Step 4 (in 10.1.6.1 and 10.1.6.2), assigned following : v_PDUSessionIdToRelease := v_PDUSessionInfo[1].SessionId; Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail these testcases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.6.1.NR5GC, 10.1.6.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required	4

1.1	Correction to fl_TC_10_1_6_1_Body	4

1.2	Correction to fl_TC_10_1_6_2_Body	6

# Corrections required

## Correction to fl_TC_10_1_6_1_Body

| Function name | Fl_TC_10_1_6_1_Body |
| --- | --- |
| Reason for change | In the current TTCN implementation of testcase 10.1.6.1, at Step 4 for checking the contents of PDU Session Release REQ, variable v_PDUSessionIdToRelease is passed. Earlier, this variable is assigned like this : var integer v_SessionId := f_NR5GC_MobileInfo_GetNoOfSessions()+1; // @sic R5s201362 sic@ var O1_Type v_PDUSessionIdToRelease := int2oct(v_SessionId, 1); This sets the value for this variable to ‘02’O. However, a UE may not trigger a PDU Session with this Id, as this is only the cid used in AT command. The UEs can map this to any number and use that number as the PDU Session ID in the PDU Session Etablishment REQ. Thus, when the UE sends a PDU Session Release REQ, it fails as the SS expects PDU Sesion Identity as ‘02’O, whereas the UE can send a different value. This needs to be addressed. |
| Summary of change | Before Step 4, assigned following : v_PDUSessionIdToRelease := v_PDUSessionInfo[1].SessionId; Please see below. |
| TTCN module | NR5GC\10_1\GSM_Testcases_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> //@siclog "Steps 3" siclog@ //Cause the UE to transmit a PDU SESSION RELEASE REQ f_UT_DeactivateBearer(UT, v_SessionId); // @sic R5s201362 sic@ v_PDUSessionInfo := f_NR5GC_MobileInfo_GetSessionInfoList(); //@siclog "Steps 4" siclog@ // The UE transmits a PDU SESSION RELEASE REQ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered)))) -> value v_ReceivedAsp; // @sic R5s190629 sic@ if (not f_Check_NG_PDUSessionReleaseReq (v_GSM_MobilityInfo, v_PDUSessionInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionIdToRelease)) { // @sic R5s190629, R5s200564 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> //@siclog "Steps 3" siclog@ //Cause the UE to transmit a PDU SESSION RELEASE REQ f_UT_DeactivateBearer(UT, v_SessionId); // @sic R5s201362 sic@ v_PDUSessionInfo := f_NR5GC_MobileInfo_GetSessionInfoList(); v_PDUSessionIdToRelease := v_PDUSessionInfo[1].SessionId; //WA#10_1_6_1 //@siclog "Steps 4" siclog@ // The UE transmits a PDU SESSION RELEASE REQ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking((tsc_SHT_IntegrityProtected_Ciphered)))) -> value v_ReceivedAsp; // @sic R5s190629 sic@ if (not f_Check_NG_PDUSessionReleaseReq (v_GSM_MobilityInfo, v_PDUSessionInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionIdToRelease)) { // @sic R5s190629, R5s200564 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Request Message Failed"); //WA#10_1_6_1 } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } <<SKIPPED CODE>>

## Correction to fl_TC_10_1_6_2_Body

| Function name | Fl_TC_10_1_6_2_Body |
| --- | --- |
| Reason for change | In the current TTCN implementation of testcase 10.1.6.1, at Step 4 for checking the contents of PDU Session Release REQ, variable v_PDUSessionIdToRelease is passed. Earlier, this variable is assigned like this : var integer v_SessionId := f_NR5GC_MobileInfo_GetNoOfSessions()+1; // @sic R5s201362 sic@ var O1_Type v_PDUSessionIdToRelease := int2oct(v_SessionId, 1); This sets the value for this variable to ‘02’O. However, a UE may not trigger a PDU Session with this Id, as this is only the cid used in AT command. The UEs can map this to any number and use that number as the PDU Session ID in the PDU Session Etablishment REQ. Thus, when the UE sends a PDU Session Release REQ, it fails as the SS expects PDU Sesion Identity as ‘02’O, whereas the UE can send a different value. This needs to be addressed. |
| Summary of change | Before Step 4, assigned following : v_PDUSessionIdToRelease := v_PDUSessionInfo[1].SessionId; Please see below. |
| TTCN module | NR5GC\10_1\GSM_Testcases_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> //@siclog "Steps 3" siclog@ //Cause the UE to transmit a PDU SESSION RELEASE REQ f_UT_DeactivateBearer(UT, v_SessionId); // @sic R5s201362 sic@ //@siclog "Steps 4" siclog@ // The UE transmits a PDU SESSION RELEASE REQ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; // @sic R5s190717 sic@ if (not f_Check_NG_PDUSessionReleaseReq (v_GSM_MobilityInfo, f_NR5GC_MobileInfo_GetSessionInfoList(), v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionIdToRelease)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> /@siclog "Steps 2" siclog@ f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1); v_PDUSessionInfo := f_NR5GC_MobileInfo_GetSessionInfoList(); // @sic R5s190717 sic@ v_PDUSessionIdToRelease := v_PDUSessionInfo[1].SessionId; //WA#10_1_6_2 //@siclog "Steps 3" siclog@ //Cause the UE to transmit a PDU SESSION RELEASE REQ f_UT_DeactivateBearer(UT, v_SessionId); // @sic R5s201362 sic@ //@siclog "Steps 4" siclog@ // The UE transmits a PDU SESSION RELEASE REQ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; // @sic R5s190717 sic@ if (not f_Check_NG_PDUSessionReleaseReq (v_GSM_MobilityInfo, f_NR5GC_MobileInfo_GetSessionInfoList(), v_ReceivedAsp.Signalling.Nas[0].Pdu, v_PDUSessionIdToRelease)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Release Request Message Failed"); //WA#10_1_6_2 } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); }<<SKIPPED CODE>>