3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210206

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1441 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NR_InitAS_KeyChaining_SKgNB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-11-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation, the function initialises just the elements AS_Ciphering_S and AS_Integrity_S which are meant for NR-DC. However, the elements AS_Ciphering and AS_Integrity also need to be initialised with the Ciphering and Integrity keys. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Initialised AS_Ciphering and AS_Integrity elements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ciphering and Integrity keys will not be initialised. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.4.1.ENDC, 8.2.2.9.1.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	3

1.1	function f_NR_InitAS_KeyChaining_SKgNB ()	3

# Corrections required

## function f_NR_InitAS_KeyChaining_SKgNB ()

| Function name | f_NR_InitAS_KeyChaining_SKgNB () |
| --- | --- |
| Reason for change | In the current TTCN implementation, the function initialises just the elements AS_Ciphering_S and AS_Integrity_S which are meant for NR-DC. However, the elements AS_Ciphering and AS_Integrity also need to be initialised with the Ciphering and Integrity keys. |
| Summary of change | Initialised AS_Ciphering and AS_Integrity elements |
| TTCN module | Common/NR/NR_SecurityFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR_InitAS_KeyChaining_SKgNB(NR_SecurityParams_Type p_Auth_Params, boolean p_ENDC := true) return NR_SecurityParams_Type { var NR_SecurityParams_Type v_Auth_Params := p_Auth_Params; // Derives KRRCenc key v_Auth_Params.AS_Ciphering_S.KRRCenc := f_EUTRA_NR_Authentication_S69(tsc_RRC_Enc_Alg, f_NR_ConvertCiphAlg2Bitstring(v_Auth_Params.AS_Ciphering.Algorithm), v_Auth_Params.S_KGNB, v_Auth_Params.KDF); // Derives KRRCenc key v_Auth_Params.AS_Ciphering_S.KUPenc := f_EUTRA_NR_Authentication_S69(tsc_UP_Enc_Alg, f_NR_ConvertCiphAlg2Bitstring(v_Auth_Params.AS_Ciphering.Algorithm), v_Auth_Params.S_KGNB, v_Auth_Params.KDF); // Derives KRRCint key v_Auth_Params.AS_Integrity_S.KRRCint := f_EUTRA_NR_Authentication_S69(tsc_RRC_Int_Alg, f_NR_ConvertIntAlg2Bitstring(v_Auth_Params.AS_Integrity.Algorithm), v_Auth_Params.S_KGNB, v_Auth_Params.KDF); if (not p_ENDC) { // Derives KUPint key @sic R5-204475 sic@ v_Auth_Params.AS_Integrity_S.KUPint := f_EUTRA_NR_Authentication_S69(tsc_N_UP_Int_Alg, f_NR_ConvertIntAlg2Bitstring(v_Auth_Params.AS_Integrity.Algorithm), v_Auth_Params.S_KGNB, v_Auth_Params.KDF); } return v_Auth_Params; }

After Change:

function f_NR_InitAS_KeyChaining_SKgNB(NR_SecurityParams_Type p_Auth_Params, boolean p_ENDC := true) return NR_SecurityParams_Type { var NR_SecurityParams_Type v_Auth_Params := p_Auth_Params; // Derives KRRCenc key v_Auth_Params.AS_Ciphering_S.KRRCenc := f_EUTRA_NR_Authentication_S69(tsc_RRC_Enc_Alg, f_NR_ConvertCiphAlg2Bitstring(v_Auth_Params.AS_Ciphering.Algorithm), v_Auth_Params.S_KGNB, v_Auth_Params.KDF); //WA#Derivation of keys for EN-DC v_Auth_Params.AS_Ciphering.KRRCenc := f_EUTRA_NR_Authentication_S69(tsc_RRC_Enc_Alg, f_NR_ConvertCiphAlg2Bitstring(v_Auth_Params.AS_Ciphering.Algorithm), v_Auth_Params.S_KGNB, v_Auth_Params.KDF); // Derives KRRCenc key v_Auth_Params.AS_Ciphering_S.KUPenc := f_EUTRA_NR_Authentication_S69(tsc_UP_Enc_Alg, f_NR_ConvertCiphAlg2Bitstring(v_Auth_Params.AS_Ciphering.Algorithm), v_Auth_Params.S_KGNB, v_Auth_Params.KDF); //WA#Derivation of keys for EN-DC v_Auth_Params.AS_Ciphering.KUPenc := f_EUTRA_NR_Authentication_S69(tsc_UP_Enc_Alg, f_NR_ConvertCiphAlg2Bitstring(v_Auth_Params.AS_Ciphering.Algorithm), v_Auth_Params.S_KGNB, v_Auth_Params.KDF); // Derives KRRCint key v_Auth_Params.AS_Integrity_S.KRRCint := f_EUTRA_NR_Authentication_S69(tsc_RRC_Int_Alg, f_NR_ConvertIntAlg2Bitstring(v_Auth_Params.AS_Integrity.Algorithm), v_Auth_Params.S_KGNB, v_Auth_Params.KDF); //WA#Derivation of keys for EN-DC v_Auth_Params.AS_Integrity.KRRCint := f_EUTRA_NR_Authentication_S69(tsc_RRC_Int_Alg, f_NR_ConvertIntAlg2Bitstring(v_Auth_Params.AS_Integrity.Algorithm), v_Auth_Params.S_KGNB, v_Auth_Params.KDF); if (not p_ENDC) { // Derives KUPint key @sic R5-204475 sic@ v_Auth_Params.AS_Integrity_S.KUPint := f_EUTRA_NR_Authentication_S69(tsc_N_UP_Int_Alg, f_NR_ConvertIntAlg2Bitstring(v_Auth_Params.AS_Integrity.Algorithm), v_Auth_Params.S_KGNB, v_Auth_Params.KDF); //WA#Derivation of keys for EN-DC v_Auth_Params.AS_Integrity.KUPint := f_EUTRA_NR_Authentication_S69(tsc_N_UP_Int_Alg, f_NR_ConvertIntAlg2Bitstring(v_Auth_Params.AS_Integrity.Algorithm), v_Auth_Params.S_KGNB, v_Auth_Params.KDF); } return v_Auth_Params; }