3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210208

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-4 | CR | 0133 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to f_POS_NR_CheckAgnssMultiFreq() |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-11-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) ... Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In function f_POS_NR_CheckAgnssMultiFreq (see change 2 of R5s201403 and R5s201407) “Beidou” satellite system (“bds”) is missing in the list of evaluated GNSS methods in f_POS_NR_CheckAgnssMultiFreq. This could cause the TC to FAIL, since Beidou measurements are not expected because Beidou is not a multifrequency GNSS technology. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In case UE reports Beidou measurements the check for multifrequency signals is ignored. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TC implementation may FAIL a conformant UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | e.g. 9.3.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	2

Change 1: Correction to function f_POS_NR_CheckAgnssMultiFreq	2

# Corrections required

## Change 1: Correction to function f_POS_NR_CheckAgnssMultiFreq

| Function name | f_POS_NR_CheckAgnssMultiFreq() |
| --- | --- |
| Reason for change | In function f_POS_NR_CheckAgnssMultiFreq (see change 2 of R5s201403 and R5s201407) “Beidou” satellite system (“bds”) is missing in the list of evaluated GNSS methods in f_POS_NR_CheckAgnssMultiFreq. This could cause the TC to FAIL, since Beidou measurements are not expected because Beidou is not a multifrequency GNSS technology. |
| Summary of change | In case UE reports Beidou measurements the check for multifrequency signals is ignored. |
| TTCN module | ATS_POS\NR5GC\NR5GC_Positioning_Functions.ttcn |
| MCC160 Comment |  |

Before

<<SKIPPED CODE>>

function f_POS_NR_CheckAgnssMultiFreq(GNSS_MeasurementList p_GNSS_MeasList) runs on NR_BASE_PTC

{ // NOTE: The multifrequencies checked here are those listed in condition MultiFreqAGNSS of Table 7.3.4.2.3.3-11

// @sic R5-198966 sic@

var integer i;

var boolean v_AgpsFound := false;

var boolean v_GlonassFound := false;

var boolean v_GalileoFound := false;

var boolean v_QzssFound := false;

var boolean v_AgpsSignalSupported := pc_A_GPS_L2C or pc_A_GPS_L5;

var boolean v_GlonassSignalSupported := pc_GLONASS_G2 or pc_GLONASS_G3;

var boolean v_GalileoSignalSupported := pc_GALILEO_E5a or pc_GALILEO_E5b or pc_GALILEO_E6 or pc_GALILEO_E5aE5b;

var boolean v_QzssSignalSupported := pc_QZSS_QZS_L1C or pc_QZSS_QZS_L2C or pc_QZSS_QZS_L5;

//Check the measurements reported by the UE

for (i:=0; i<lengthof(p_GNSS_MeasList); i:=i+1) {

select (p_GNSS_MeasList[i].gnss_ID.gnss_id){

case(gps){

v_AgpsFound := true;

if (v_AgpsSignalSupported){

f_POS_NR_CheckSignalsGps(p_GNSS_MeasList[i].gnss_SgnMeasList);

}

}

case(glonass){

v_GlonassFound := true;

if (v_GlonassSignalSupported){

f_POS_NR_CheckSignalsGlonass(p_GNSS_MeasList[i].gnss_SgnMeasList);

}

}

case(galileo){

v_GalileoFound := true;

if (v_GalileoSignalSupported){

f_POS_NR_CheckSignalsGalileo(p_GNSS_MeasList[i].gnss_SgnMeasList);

}

}

case(qzss){

v_QzssFound := true;

if (v_QzssSignalSupported){

f_POS_NR_CheckSignalsQzss(p_GNSS_MeasList[i].gnss_SgnMeasList);

}

}

case else {

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "UE reported Multifrequencies not included in condition MultiFreqAGNSS of Table 7.3.4.2.3.3-11");

}

}

}

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

function f_POS_NR_CheckAgnssMultiFreq(GNSS_MeasurementList p_GNSS_MeasList) runs on NR_BASE_PTC

{ // NOTE: The multifrequencies checked here are those listed in condition MultiFreqAGNSS of Table 7.3.4.2.3.3-11

// @sic R5-198966 sic@

var integer i;

var boolean v_AgpsFound := false;

var boolean v_GlonassFound := false;

var boolean v_GalileoFound := false;

var boolean v_QzssFound := false;

var boolean v_AgpsSignalSupported := pc_A_GPS_L2C or pc_A_GPS_L5;

var boolean v_GlonassSignalSupported := pc_GLONASS_G2 or pc_GLONASS_G3;

var boolean v_GalileoSignalSupported := pc_GALILEO_E5a or pc_GALILEO_E5b or pc_GALILEO_E6 or pc_GALILEO_E5aE5b;

var boolean v_QzssSignalSupported := pc_QZSS_QZS_L1C or pc_QZSS_QZS_L2C or pc_QZSS_QZS_L5;

//Check the measurements reported by the UE

for (i:=0; i<lengthof(p_GNSS_MeasList); i:=i+1) {

select (p_GNSS_MeasList[i].gnss_ID.gnss_id){

case(gps){

v_AgpsFound := true;

if (v_AgpsSignalSupported){

f_POS_NR_CheckSignalsGps(p_GNSS_MeasList[i].gnss_SgnMeasList);

}

}

case(glonass){

v_GlonassFound := true;

if (v_GlonassSignalSupported){

f_POS_NR_CheckSignalsGlonass(p_GNSS_MeasList[i].gnss_SgnMeasList);

}

}

case(galileo){

v_GalileoFound := true;

if (v_GalileoSignalSupported){

f_POS_NR_CheckSignalsGalileo(p_GNSS_MeasList[i].gnss_SgnMeasList);

}

}

case(qzss){

v_QzssFound := true;

if (v_QzssSignalSupported){

f_POS_NR_CheckSignalsQzss(p_GNSS_MeasList[i].gnss_SgnMeasList);

}

}

case(bds){

continue;

}

case else {

f_NR_SetVerdictFailOrInconc (__FILE__, __LINE__, "UE reported Multifrequencies not included in condition MultiFreqAGNSS of Table 7.3.4.2.3.3-11");

}

}

}

<<SKIPPED CODE>>