3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210210

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1445 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of NR5GC test case 9.1.5.1.12 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Datang Linktester |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-11-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1. In this TC, UE will not consider the intra-frequency cells NGC cell E and I as candidates for a maximum of 300 seconds after step 15, which will fail UE at step19. Thus, NGC cell E and I shall be configured to inter-frequency. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. 1. Delete the description ‘except replacing NRf3 with NRf2’ for System Simulator. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE fails NR5GC-IRAT test case 9.1.5.1.12. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.1.12.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2019-06_D20wk38 related to the title of this CR.

Contact:	Fu Wang
	wangfu@datangmobile.cn

# Corrections required

## Change 1

| Function name | f_TC_9_1_5_1_12_NR5GC |
| --- | --- |
| Reason for change | - In this TC, UE will not consider the intra-frequency cells NGC cell E and I as candidates for a maximum of 300 seconds after step 15, which will fail UE at step19. Thus, NGC cell E and I shall be configured to inter-frequency. |
| Summary of change | - The third parameter of f_NR5GC_Init_NAS uses the default value. |
| TTCN module | Initial_Registration_NR5GC.ttcn |
| MCC160 Comment |  |

Before change

function f_TC_9_1_5_1_12_NR5GC() runs on NR5GC_PTC

{

//Initial registration / Rejected / Roaming not allowed in this tracking area

var NR_CellId_Type v_NGC_NASCellC := f_NR5GC_MapNASCell(ngc_CellC, NG_CellsOnDifferentPLMN);

var NR_CellId_Type v_NGC_NASCellE := f_NR5GC_MapNASCell(ngc_CellE, NG_CellsOnDifferentPLMN);

var NR_CellId_Type v_NGC_NASCellI := f_NR5GC_MapNASCell(ngc_CellI, NG_CellsOnDifferentPLMN);

f_NR5GC_Init_NAS(NR_1, NG_CellsOnDifferentPLMN, NR5GC_NAS_ReplaceNRf3_NRf2);

//Create and configure cells

f_NR_CellConfig_Def(v_NGC_NASCellC);

f_NR_CellConfig_Def(v_NGC_NASCellE);

f_NR_CellConfig_Def(v_NGC_NASCellI);

f_NR5GC_Preamble (v_NGC_NASCellE, STATE_OFF_0B);

f_NR_TestBody_Set(true);

fl_TC_9_1_5_1_12_Body();

f_NR_TestBody_Set(false);

// Finish

f_NR_Postamble(v_NGC_NASCellC, STATE_IDLE_1A);

}

After change

function f_TC_9_1_5_1_12_NR5GC() runs on NR5GC_PTC

{

//Initial registration / Rejected / Roaming not allowed in this tracking area

var NR_CellId_Type v_NGC_NASCellC := f_NR5GC_MapNASCell(ngc_CellC, NG_CellsOnDifferentPLMN);

var NR_CellId_Type v_NGC_NASCellE := f_NR5GC_MapNASCell(ngc_CellE, NG_CellsOnDifferentPLMN);

var NR_CellId_Type v_NGC_NASCellI := f_NR5GC_MapNASCell(ngc_CellI, NG_CellsOnDifferentPLMN);

f_NR5GC_Init_NAS(NR_1, NG_CellsOnDifferentPLMN);

//Create and configure cells

f_NR_CellConfig_Def(v_NGC_NASCellC);

f_NR_CellConfig_Def(v_NGC_NASCellE);

f_NR_CellConfig_Def(v_NGC_NASCellI);

f_NR5GC_Preamble (v_NGC_NASCellE, STATE_OFF_0B);

f_NR_TestBody_Set(true);

fl_TC_9_1_5_1_12_Body();

f_NR_TestBody_Set(false);

// Finish

f_NR_Postamble(v_NGC_NASCellC, STATE_IDLE_1A);

}