3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210213

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4555 | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to f_EUTRA_AS_ActivateSecurity_NewAKA() |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_test |  |  |  |  |  | Date: |  |  | 2020-11-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In function f_EUTRA_AS_ActivateSecurity_NewAKA(), EARFCN value is taken from UL_DL_Earfcn.dl_CarrierFreq, which is having a value range up to 65535. So while using frequencies which are above this value ( frequency band greater than 64), TC fails as the EARCN will not match with the actual EARFCN because of which the integrity keys get wrongly generated. This causes the TC to fail. To correct this issue, EARFCN value has mapped to UL_DL_Earfcn.dl_CarrierFreq_r9 if EARFCN value greater than 65535 (frequency band greater than 64) is used. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UL_DL_Earfcn.dl_CarrierFreq_r9 used to get EARCN value if the used EARFCN value greater than 65535 (frequency band greater than 64). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail if the EARFCN value used above 65535 (frequency band greater than 64). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2019-06_D20wk38 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | f_EUTRA_AS_ActivateSecurity_NewAKA() |
| --- | --- |
| Reason for change | In function f_EUTRA_AS_ActivateSecurity_NewAKA(), EARFCN value is taken from UL_DL_Earfcn.dl_CarrierFreq, which is having a value range up to 65535. So while using frequencies which are above this value ( frequency band greater than 64), TC fails as the EARCN will not match with the actual EARFCN because of which the integrity keys get wrongly generated. This causes the TC to fail. To correct this issue, EARFCN value has mapped to UL_DL_Earfcn.dl_CarrierFreq_r9 if EARFCN value greater than 65535 (frequency band greater than 64) is used. |
| Summary of change | UL_DL_Earfcn.dl_CarrierFreq_r9 used to get EARCN value if the used EARFCN value greater than 65535 (frequency band greater than 64). |
| TTCN module | EUTRA_SecuritySteps.ttcn |
| MCC160 Comment |  |

Before change:

<<SKIPPED CODE>>

function f_EUTRA_AS_ActivateSecurity_NewAKA(EUTRA_CellId_Type p_TargetCellId,

EUTRA_SecurityParams_Type p_SecurityParams,

NasCount_Type p_NasCountUL,

template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now,

DRB_IdentityList_Type p_DrbList := tsc_DrbList_Def) runs on EUTRA_PTC

return EUTRA_SecurityParams_Type

{

var EUTRA_SecurityParams_Type v_SecurityParams;

var PhysCellId v_PhysicalCellId := f_EUTRA_CellInfo_GetPhyCellId(p_TargetCellId);

var Frequency_IE_Type v_Frequency_IE_Cell := f_EUTRA_CellInfo_GetFrequencyIEs(p_TargetCellId);

var ARFCN_ValueEUTRA v_EARFCN_DL := v_Frequency_IE_Cell.UL_DL_Earfcn.dl_CarrierFreq;

var boolean v_IsSMC_Procedure := false;   // => Security is (re-)initialised for the non-SECURITY-MODE-COMMAND case

v_SecurityParams := f_EUTRA_Authentication_InitAS(p_SecurityParams,

p_NasCountUL,

v_IsSMC_Procedure,

v_PhysicalCellId,

v_EARFCN_DL); /* Calculate Kenb and KRRCenc, KRRCint, KUPenc */

<<SKIPPED CODE>>

After change:

<<SKIPPED CODE>>

function f_EUTRA_AS_ActivateSecurity_NewAKA(EUTRA_CellId_Type p_TargetCellId,

EUTRA_SecurityParams_Type p_SecurityParams,

NasCount_Type p_NasCountUL,

template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now,

DRB_IdentityList_Type p_DrbList := tsc_DrbList_Def) runs on EUTRA_PTC

return EUTRA_SecurityParams_Type

{

var EUTRA_SecurityParams_Type v_SecurityParams;

var PhysCellId v_PhysicalCellId := f_EUTRA_CellInfo_GetPhyCellId(p_TargetCellId);

var Frequency_IE_Type v_Frequency_IE_Cell := f_EUTRA_CellInfo_GetFrequencyIEs(p_TargetCellId);

// TTCN CHANGES TO MATCH BAND66 BEGIN //

var ARFCN_ValueEUTRA v_EARFCN_DL;

var integer v_BandCell1 := f_EUTRA_CellInfo_GetBand(p_TargetCellId);

if( v_BandCell1 <= 64 ) {

v_EARFCN_DL := v_Frequency_IE_Cell.UL_DL_Earfcn.dl_CarrierFreq;

}

else {

v_EARFCN_DL := v_Frequency_IE_Cell.UL_DL_Earfcn.dl_CarrierFreq_r9;

}

// TTCN CHANGES TO MATCH BAND66 END //

var boolean v_IsSMC_Procedure := false;   // => Security is (re-)initialised for the non-SECURITY-MODE-COMMAND case

v_SecurityParams := f_EUTRA_Authentication_InitAS(p_SecurityParams,

p_NasCountUL,

v_IsSMC_Procedure,

v_PhysicalCellId,

v_EARFCN_DL); /* Calculate Kenb and KRRCenc, KRRCint, KUPenc */<<SKIPPED CODE>>