3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210214

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4556 | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_TC_13_2_1_EUTRA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2020-11-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | During RRC Connection Reconfiguration at step 3-4, dl_CarrierFreq_v9e0 (optional parameter) is used as input parameter in function f_ReturnMeasObjectToAddModList_v9e0. But the expected input parameter of this function is type ARFCN_ValueEUTRA_r9, defined as integer with value from (0..maxEARFCN2). In case the EUTRA Band used is less than 64, dl_CarrierFreq_v9e0 will be not present, this can cause a runtime error as f_ReturnMeasObjectToAddModList_v9e0 is expecting an integer value between 0 and maxEARFCN2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replaced dl_CarrierFreq_v9e0 with dl_CarrierFreq_r9 in function f_ReturnMeasObjectToAddModList_v9e0. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | May cause runtime errors in case the EUTRA band tested is less than 64 (= maxEARFCN). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections	3

1.1	Correction to f_TC_13_2_1_EUTRA	3

# Corrections

## Correction to f_TC_13_2_1_EUTRA

| Function name | f_TC_13_2_1_EUTRA |
| --- | --- |
| Reason for change | In case the EUTRA Band used is less than 64, dl_CarrierFreq_v9e0 will be not present, this can cause a runtime error as f_ReturnMeasObjectToAddModList_v9e0 is expecting an integer value between 0 and maxEARFCN2. |
| Summary of change | Replaced dl_CarrierFreq_v9e0 with dl_CarrierFreq_r9 in function f_ReturnMeasObjectToAddModList_v9e0. |
| TTCN module | 13/MultiLayer_Procedures |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> //@siclog "Step 2" siclog@ /* Check: Does the UE loop back the IP packet on the DRB associated with the default EPS bearer context on Cell 1? */ DRB.receive(car_DRB_COMMON_IND_PDCP_SDUList(eutra_Cell1, tsc_RbId_DRB1, ?, v_PDCP_SDUList_DRB1)); /* @sic R5s120470 sic@ */ f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 13.2.1 Step 2"); //@siclog "Step 3-4" siclog@ //@sic R5s110743 sic@ f_EUTRA_RRCConnectionReconfiguration_MeasConfig(eutra_Cell1, cs_RRCConnectionReconfiguration_13_2_1_Step3(tsc_RRC_TI_Def, v_Frequency_IE_f1.UL_DL_Earfcn.dl_CarrierFreq, v_ChBandDependency_f1.AllowedMeasBandwidth,//@sic R5-171140 sic@ f_EUTRA_SS_InitialiseMeasGapConfig(), //@sic R5-176804 sic@ f_ReturnMeasObjectToAddModList_v9e0(v_Frequency_IE_f1.UL_DL_Earfcn.dl_CarrierFreq_v9e0))); //@siclog "Step 5" siclog@ /* The SS changes Cell 1 and Cell 2 parameters according to the row "T1" in table 13.2.1.3.2-1 */ v_CellPowerList_AtT1 := { cs_CellPower(eutra_Cell1, -91), cs_CellPower(eutra_Cell2, -85) }; <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> //@siclog "Step 2" siclog@ /* Check: Does the UE loop back the IP packet on the DRB associated with the default EPS bearer context on Cell 1? */ DRB.receive(car_DRB_COMMON_IND_PDCP_SDUList(eutra_Cell1, tsc_RbId_DRB1, ?, v_PDCP_SDUList_DRB1)); /* @sic R5s120470 sic@ */ f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Test Case 13.2.1 Step 2"); //@siclog "Step 3-4" siclog@ //@sic R5s110743 sic@ f_EUTRA_RRCConnectionReconfiguration_MeasConfig(eutra_Cell1, cs_RRCConnectionReconfiguration_13_2_1_Step3(tsc_RRC_TI_Def, v_Frequency_IE_f1.UL_DL_Earfcn.dl_CarrierFreq, v_ChBandDependency_f1.AllowedMeasBandwidth,//@sic R5-171140 sic@ f_EUTRA_SS_InitialiseMeasGapConfig(), //@sic R5-176804 sic@ f_ReturnMeasObjectToAddModList_v9e0(v_Frequency_IE_f1.UL_DL_Earfcn.dl_CarrierFreq_r9))); //@siclog "Step 5" siclog@ /* The SS changes Cell 1 and Cell 2 parameters according to the row "T1" in table 13.2.1.3.2-1 */ v_CellPowerList_AtT1 := { cs_CellPower(eutra_Cell1, -91), cs_CellPower(eutra_Cell2, -85) }; <<SKIPPED CODE>>