3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210216

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4558 | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to LTE-GERAN test cases using Measurement reports |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2020-11-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | During execution of different LTE-GERAN test cases a couple of issues were found: - cr_MeasurementReport_Geran_1Entry is defeined as: cr_MeasurementReport(p_MeasId, ?, ?, cr_MeasResultNeighCells_measResultListGERAN_1Entry(*, *, p_PhysCellId, omit, ? )); Where cr_MeasResultNeighCells_measResultListGERAN_1Entry two first input parameters are defined as mandatory , cr_MeasurementReport_Geran_1Entry uses “*” AnyValueOrOmit. - In template cr_MeasResultNeighCells_measResultListGERAN_1Entry 4th input parameter, MeasResultGERAN.cgi_Info p_Cgi_Info, is declared as mandatory. According to ASN.1 definition it should be OPTIONAL: cgi-Info SEQUENCE { cellGlobalId CellGlobalIdGERAN, routingAreaCode BIT STRING (SIZE (8)) OPTIONAL |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Used “?” AnyValue as input paramter instead of “*” AnyValueOrOmit. - Declared 4th input parameter of template cr_MeasResultNeighCells_measResultListGERAN_1Entry as template (omit) as MeasResultGERAN.cgi-Info is an OPTIONAL field. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may FAIL the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Legacy LTE-GERAN Test cases where Measurement reports are used. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections	4

1.1	Correction to template cr_MeasurementReport_Geran_1Entry	4

1.2	Correction to template cr_MeasResultNeighCells_measResultListGERAN_1Entry	4

# Corrections

## Correction to template cr_MeasurementReport_Geran_1Entry

| Template name | cr_MeasurementReport_Geran_1Entry |
| --- | --- |
| Reason for change | MeasResultNeighCells_measResultListGERAN_1Entry two first input parameters are defined as mandatory, cr_MeasurementReport_Geran_1Entry uses “*” AnyValueOrOmit. |
| Summary of change | 1. Used “?” AnyValue as input paramter instead of “*” AnyValueOrOmit. |
| TTCN module | Common/EUTRA/EUTRA_Measurement_Templates.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> template (present) UL_DCCH_Message cr_MeasurementReport_Geran_1Entry(template integer p_MeasId, template PhysCellIdGERAN p_PhysCellId) := /* @status APPROVED (LTE_A_IRAT, LTE_IRAT) */ cr_MeasurementReport(p_MeasId, ?, ?, cr_MeasResultNeighCells_measResultListGERAN_1Entry(*, *, p_PhysCellId, omit, ? )); <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (present) UL_DCCH_Message cr_MeasurementReport_Geran_1Entry(template integer p_MeasId, template PhysCellIdGERAN p_PhysCellId) := /* @status APPROVED (LTE_A_IRAT, LTE_IRAT) */ cr_MeasurementReport(p_MeasId, ?, ?, cr_MeasResultNeighCells_measResultListGERAN_1Entry(?, ?, p_PhysCellId, omit, ? )); <<SKIPPED CODE>>

## Correction to template cr_MeasResultNeighCells_measResultListGERAN_1Entry

| Template name | cr_MeasResultNeighCells_measResultListGERAN_1Entry |
| --- | --- |
| Reason for change | cr_MeasResultNeighCells_measResultListGERAN_1Entry 4th input parameter MeasResultGERAN.cgi_Info p_Cgi_Info, is declared as mandatory. According to ASN.1 definition it should be OPTIONAL: |
| Summary of change | Declared 4th input parameter of template cr_MeasResultNeighCells_measResultListGERAN_1Entry as template (omit) as MeasResultGERAN.cgi-Info is an OPTIONAL field. |
| TTCN module | Common/EUTRA/EUTRA_Measurement_Templates.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> template (present) MeasResults.measResultNeighCells cr_MeasResultNeighCells_measResultListGERAN_1Entry(template integer p_ARFCN, template BandIndicatorGERAN p_BandIndicator, template PhysCellIdGERAN p_PhysCellId, template MeasResultGERAN.cgi_Info p_Cgi_Info, template RSSI_Range_Type p_RSSI) := <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (present) MeasResults.measResultNeighCells cr_MeasResultNeighCells_measResultListGERAN_1Entry(template integer p_ARFCN, template BandIndicatorGERAN p_BandIndicator, template PhysCellIdGERAN p_PhysCellId, template (omit) MeasResultGERAN.cgi_Info p_Cgi_Info, template RSSI_Range_Type p_RSSI) := <<SKIPPED CODE>>