3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210218

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4559 | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE TC 8.6.2.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI10_Test |  |  |  |  |  | Date: |  |  | 2020-11-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | During test case 8.6.2.6 initialization , following declaration is done: “var LogMeasReport_LogMeasAvailable_Type v_LogMeasReport_LogMeasAvailable”. Being ASN.1 definition as follows logMeasAvailable-r10 ENUMERATED {true} OPTIONAL, In this case, it is possible that omit is assigned to v_LogMeasReport_LogMeasAvailable as logMeasAvailable-r10 is defined as OPTIONAL. This will provoke a runtime error as omit will be assigned to a non-template variable, v_LogMeasReport_LogMeasAvailable. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Declared v_LogMeasReport_LogMeasAvailable as template (omit), this way omit can be assignated to it, as logMeasAvailable-r10 is defined as OPTIONAL field. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may FAIL the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections	3

1.1	Correction to function f_TC_8_6_2_6_EUTRA	3

# Corrections

## Correction to function f_TC_8_6_2_6_EUTRA

| Function name | Correction to function f_TC_8_6_2_6_EUTRA |
| --- | --- |
| Reason for change | It is possible that omit can be assigned v_LogMeasReport_LogMeasAvailable as logMeasAvailable-r10 is defined as OPTIONAL. This will provoke a runtime error as omit will be assigned to a non-template variable, v_LogMeasReport_LogMeasAvailable. |
| Summary of change | Declared v_LogMeasReport_LogMeasAvailable as template (omit), this way omit can be assignated to it, as logMeasAvailable-r10 is defined as OPTIONAL field. |
| TTCN module | LTE_A_R10_R11/8_6/ RRC_MDT_Logged.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> function f_TC_8_6_2_6_EUTRA() runs on EUTRA_PTC { //Logged MDT / Release of logged MDT measurement configuration / Expire of duration timer var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def; var EUTRA_CellInfo_Type v_CellInfo_Cell1; var CellIdentity v_CellIdentity_Cell1; var PLMN_Identity v_PLMN_Identity_Cell1; var T_ReselectionEutran_Type v_T_Reselection := 7; var SRB_COMMON_IND v_SRB_COMMON_IND; var LogMeasReport_LogMeasAvailable_Type v_LogMeasReport_LogMeasAvailable; var B48_Type v_AbsoluteTimeInfo; var NAS_MSG_Indication_Type v_NasInd; //@sic R5s130566 sic@ <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_TC_8_6_2_6_EUTRA() runs on EUTRA_PTC { //Logged MDT / Release of logged MDT measurement configuration / Expire of duration timer var RRC_TransactionIdentifier v_RRC_TI := tsc_RRC_TI_Def; var EUTRA_CellInfo_Type v_CellInfo_Cell1; var CellIdentity v_CellIdentity_Cell1; var PLMN_Identity v_PLMN_Identity_Cell1; var T_ReselectionEutran_Type v_T_Reselection := 7; var SRB_COMMON_IND v_SRB_COMMON_IND; var template (omit) LogMeasReport_LogMeasAvailable_Type v_LogMeasReport_LogMeasAvailable; var B48_Type v_AbsoluteTimeInfo; var NAS_MSG_Indication_Type v_NasInd; //@sic R5s130566 sic@ <<SKIPPED CODE>>