3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210222

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1451 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR PDCP integrity test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-12-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - RLC configuration on SS side is missing for SRB3 bearer. - In case of pc_SRB3 the test body of 7.1.3.2.x in step 2a1 is configuring duplicit SRB3 addition instead of 38.523-1 Table 7.1.3.2.1.3.3-2, which is reconfiguration of periodicBSR timer sent on SRB3.. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added SRB3 RLC configuration of SS in case of pc_SRB3 - Changed the content of RRCreconfiguration in step 2a1 so it corresponds to 38.523-1 Table 7.1.3.2.1.3.3-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE capable of SRB3 will fail the test. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.2.1.ENDC, 7.1.3.2.2.ENDC, 7.1.3.2.3.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1 Change 1	3

2.2 Change 2	5

# Overview

This document lists all the essencial changes needed to correct problems in the TTCN implementation of test case 7.1.3.1.2 which is part of the NR5GC test suite.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

# 2.1 Change 1

| Function name | fl_ENDC_PDCP_Integrity |
| --- | --- |
| Reason for change | RLC configuration on SS side is missing for SRB3 bearer in case of pc_SRB3. |
| Summary of change | Added SRB3 RLC configuration of SS in case of pc_SRB3 |
| TTCN module | 7_1_3\PDCP_ENDC_NR.ttcn |
| MCC160 Comment |  |

Before change:

function fl_ENDC_PDCP_Integrity(IntegrityProtAlgorithm p_IntegrityProAlgorithm) runs on ENDC_NR_PTC

{

var ENDC_SRB_DRB_Configuration_Type v_SecondaryConfig:= NR_PDCP_SRB1_2;

//var template (value) FreqBandList v_MRDC_FreqList; // Anritsu TTCN WA for FreqBandList

var template (value) FreqBandListWrapper v_MRDC_FreqList; // Anritsu TTCN WA for FreqBandList

var octetstring v_OctetData;

var integer v_EUTRABandInd;

var integer v_NRBandInd;

if (pc_srb3){

v_SecondaryConfig := NR_PDCP_SRB1_2_SRB3;

}

f_ENDC_NR_Init();

//Create and configure NR cell1

f_NR_CellConfig_Def(nr_Cell1);

f_NR_AS_IntegrityAlgorithm_Set(p_IntegrityProAlgorithm);

// Get NR capabilities sent during EUTRA preamble

// @sic R5-195339 sic@

f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_SCG); //Configure DRB2 as normal

f_NR_RRC_PSCellChange_SRB_NR_PDCP(nr_Cell1, v_SecondaryConfig);

f_NR_TestBody_Set(true);

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA);

// Have to receive EUTRA band first @sic R5s190476 change 4.22 sic@

v_OctetData := f_EUTRA_NR_WaitForCoOrd_Data(EUTRA, "EUTRA Band");

v_EUTRABandInd := oct2int(v_OctetData);

v_NRBandInd := f_NR_CellInfo_GetBandIndicator(nr_Cell1);

// Then send both bands under test back to EUTRA

//v_MRDC_FreqList := cs_NR_MRDC_FreqList(v_EUTRABandInd, v_NRBandInd); // Anritsu TTCN WA for FreqBandList

v_MRDC_FreqList := cs_MRDC_FreqListWrapper(v_EUTRABandInd, v_NRBandInd); // Anritsu TTCN WA for FreqBandList

f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_OctetData (bit2oct(encvalue(v_MRDC_FreqList)), "MRDC_Freq"));

f_TC_7_1_3_2_X_NR_TestBody(tsc_NR_DRB2);

f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger); //@sic R5s190476 sic@ ch 4.6

f_NR_TestBody_Set(false);

//Wait from coordination AFTER the connection is released in E-UTRA

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble");

//Postamble

f_NR_ReleaseAllCells();

}

After change:

function fl_ENDC_PDCP_Integrity(IntegrityProtAlgorithm p_IntegrityProAlgorithm) runs on ENDC_NR_PTC

{

var ENDC_SRB_DRB_Configuration_Type v_SecondaryConfig:= NR_PDCP_SRB1_2;

//var template (value) FreqBandList v_MRDC_FreqList; // Anritsu TTCN WA for FreqBandList

var template (value) FreqBandListWrapper v_MRDC_FreqList; // Anritsu TTCN WA for FreqBandList

var octetstring v_OctetData;

var integer v_EUTRABandInd;

var integer v_NRBandInd;

var template (value) NR_RadioBearerList_Type v_RadioBearerList;

if (pc_srb3){

v_SecondaryConfig := NR_PDCP_SRB1_2_SRB3;

}

f_ENDC_NR_Init();

//Create and configure NR cell1

f_NR_CellConfig_Def(nr_Cell1);

f_NR_AS_IntegrityAlgorithm_Set(p_IntegrityProAlgorithm);

// Get NR capabilities sent during EUTRA preamble

// @sic R5-195339 sic@

f_NR_ENDC_PreambleOnEUTRA(nr_Cell1, MCG_SCG); //Configure DRB2 as normal

f_NR_RRC_PSCellChange_SRB_NR_PDCP(nr_Cell1, v_SecondaryConfig);

if (pc_srb3){

v_RadioBearerList := { cs_NR_SS_RadioBearer_Config(tsc_NR_RbId_SRB3, cs_NR_PDCP_Configuration_RBTerminating(cs_NR_PDCP_RbConfig_ParamsSRB), cs_NR_RlcBearerConfig_SRB1SRB2SRB3(3,1)) };

f_NR_SS_CommonRadioBearerConfig(nr_Cell1, v_RadioBearerList);

}

f_NR_TestBody_Set(true);

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA);

// Have to receive EUTRA band first @sic R5s190476 change 4.22 sic@

v_OctetData := f_EUTRA_NR_WaitForCoOrd_Data(EUTRA, "EUTRA Band");

v_EUTRABandInd := oct2int(v_OctetData);

v_NRBandInd := f_NR_CellInfo_GetBandIndicator(nr_Cell1);

// Then send both bands under test back to EUTRA

//v_MRDC_FreqList := cs_NR_MRDC_FreqList(v_EUTRABandInd, v_NRBandInd); // Anritsu TTCN WA for FreqBandList

v_MRDC_FreqList := cs_MRDC_FreqListWrapper(v_EUTRABandInd, v_NRBandInd); // Anritsu TTCN WA for FreqBandList

f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_OctetData (bit2oct(encvalue(v_MRDC_FreqList)), "MRDC_Freq"));

f_TC_7_1_3_2_X_NR_TestBody(tsc_NR_DRB2);

f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger); //@sic R5s190476 sic@ ch 4.6

f_NR_TestBody_Set(false);

//Wait from coordination AFTER the connection is released in E-UTRA

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble");

//Postamble

f_NR_ReleaseAllCells();

}

# 2.2 Change 2

| Function name | f_TC_7_1_3_2_X_NR_TestBody |
| --- | --- |
| Reason for change | In case of pc_SRB3 the test body of 7.1.3.2.x in step 2a1 is configuring duplicit SRB3 addition instead of 38.523-1 Table 7.1.3.2.1.3.3-2, which is reconfiguration of periodicBSR timer sent on SRB3. |
| Summary of change | Changed the content of RRCreconfiguration in step 2a1 so it corresponds to 38.523-1 Table 7.1.3.2.1.3.3-2. |
| TTCN module | NR_TC_Common\7_1_3\PDCP_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before change:

function f_TC_7_1_3_2_X_NR_TestBody(DRB_Identity p_RB_Id) runs on NR_BASE_PTC

{

var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE());

var template (value) RadioBearerConfig v_RadioBearerConfig;

var NR_SecurityParams_Type v_Security_Params := f_NR_Security_Get();

var IntegrityProtAlgorithm v_IntegrityAlg:= v_Security_Params.AS_Integrity.Algorithm;

var boolean v_DontWaitForComplete:=false;

if ( (pc_EN_DC or pc_NGEN_DC) and pc_srb3)

{

v_RadioBearerConfig:= cs_38508_RadioBearerConfigDef({ cs_NR_SRB_ToAddMod(tsc_SRB3) }, -, -, cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig(nea2, v_IntegrityAlg), secondary));

//@siclog "Step 2a1-2a2" siclog@

//Reconfigure NR MAC sent on SRB3 integrity protected

f_NR_PDCP_SendRRCReconfiguration(nr_Cell1,-, v_RadioBearerConfig);

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2a2");

//@siclog "Step 2a3-a4" siclog@

//SS sends MAC-I Corrupted, configure SS to use different integrity algorithm so that will result in MAC-I failure at UE

f_NR_SS_TriggerMAC_I_Failure(nr_Cell1, tsc_NR_RbId_SRB3);

f_NR_PDCP_SendRRCReconfiguration(nr_Cell1, -, cs_NR_RadioBearer(-,-,-,v_IntegrityAlg), v_DontWaitForComplete);

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA);

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2a4");

}

if (not pc_EN_DC){

//@siclog "Step 3a1" siclog@

DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_SDUList(nr_Cell1, p_RB_Id, -, {v_IPData}));

//@siclog "Step 3a2" siclog@

//Does UE transmit a PDCP Data PDU with SN=K for the first iteration?

DRB.receive(car_NR_DRB_COMMON_IND_PDCP_SDUList_DC(nr_Cell1, p_RB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, {v_IPData}));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3a2"); //@sic R5s200557 sic@

}

}

After change:

function f_TC_7_1_3_2_X_NR_TestBody(DRB_Identity p_RB_Id) runs on NR_BASE_PTC

{

var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE());

var template (value) RadioBearerConfig v_RadioBearerConfig;

var NR_SecurityParams_Type v_Security_Params := f_NR_Security_Get();

var IntegrityProtAlgorithm v_IntegrityAlg:= v_Security_Params.AS_Integrity.Algorithm;

var boolean v_DontWaitForComplete:=false;

var template (value) CellGroupConfig v_SCGConfig;

if ( (pc_EN_DC or pc_NGEN_DC) and pc_srb3)

{

//REMOVED v_RadioBearerConfig:= cs_38508_RadioBearerConfigDef({ cs_NR_SRB_ToAddMod(tsc_SRB3) }, -, -, cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig(nea2, v_IntegrityAlg), secondary));

//@siclog "Step 2a1-2a2" siclog@

//Reconfigure NR MAC sent on SRB3 integrity protected

v_SCGConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_SCG, -, -, cs_NR_MAC_CellGroupConfig(-, -, cs_NR_BSR_ConfigDef(sf10)));

SRB.send(cas_NR_SRB3_RrcPdu_REQ(nr_Cell1, -, cs_38508_RRCReconfiguration(tsc_NR_RRC_TI_Def, -, bit2oct(encvalue(v_SCGConfig)))));

SRB.receive(car_NR_SRB3_RrcPdu_IND(nr_Cell1, cr_38508_RRCReconfigurationComplete));

//REMOVED        f_NR_PDCP_SendRRCReconfiguration(nr_Cell1,-, v_RadioBearerConfig);

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2a2");

//@siclog "Step 2a3-a4" siclog@

//SS sends MAC-I Corrupted, configure SS to use different integrity algorithm so that will result in MAC-I failure at UE

f_NR_SS_TriggerMAC_I_Failure(nr_Cell1, tsc_NR_RbId_SRB3);

f_NR_PDCP_SendRRCReconfiguration(nr_Cell1, -, cs_NR_RadioBearer(-,-,-,v_IntegrityAlg), v_DontWaitForComplete);

f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA);

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2a4");

}

if (not pc_EN_DC){

//@siclog "Step 3a1" siclog@

DRB.send(cas_NR_DRB_COMMON_REQ_PDCP_SDUList(nr_Cell1, p_RB_Id, -, {v_IPData}));

//@siclog "Step 3a2" siclog@

//Does UE transmit a PDCP Data PDU with SN=K for the first iteration?

DRB.receive(car_NR_DRB_COMMON_IND_PDCP_SDUList_DC(nr_Cell1, p_RB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, {v_IPData}));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3a2"); //@sic R5s200557 sic@

}

}