3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210223

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4563 | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to eMTC testcase 7.3.5.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test, LTE_MTCe2_L1-UEConTest |  |  |  |  |  | Date: |  |  | 2020-12-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - For Step5, sending UL grant with more than 56 bits in RAR will cause the UE to send DRB data with RRC Reconfiguration Complete, which results in TC failure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - For eMTC test, enable UplinkGrantCE_Mode to initialize the TBS of 56 bits. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.5.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## Change 1

| Function name | f_EUTRA_7_3_5_3_TestBody() |
| --- | --- |
| Reason for change | For Step5, sending UL grant with more than 56 bits in RAR will cause the UE to send DRB data with RRC Reconfiguration Complete, which results in TC failure. |
| Summary of change | For eMTC test, enable UplinkGrantCE_Mode to initialize the TBS of 56 bits. |
| TTCN module | EUTRA_RRCSteps.ttcn |
| MCC160 Comment |  |

Before Change

function f_EUTRA_7_3_5_3_TestBody(inout PDCP_SS_State_Type p_PDCP_Rec)

runs on EUTRA_PTC

{

……

//@siclog "Step 5" siclog@

// The UE transmits a  RRCConnectionReconfigurationComplete message.

v_RRCConnectionReconfiguration_v890_IEs := f_EUTRA_CellInfo_GetReconfig_BR(eutra_Cell2); //@sic R5s180630 sic@

f_EUTRA_508RRC_IntraLTE_HO_InterCell(eutra_Cell1,   // @sic R5s100430 change 2; R5s100524 sic@

eutra_Cell2,

{cs_DRB_ConfigAM_PDCP_Mode, cs_DRB2_ConfigUM_PDCP_Mode(p_PDCP_Rec.PDCP_SN_Size) }, // @sic R5-168359 sic@

tsc_C_RNTI_Def3,

-,

-,

-,

cs_UL_GrantConfig_Stop,

n32, //@sic R5s190596 sic@

-,

-,

-,

v_RRCConnectionReconfiguration_v890_IEs); // @sic R5s170095, R5s180630 sic@

//@siclog "Step 5A" siclog@

// The SS assigns additional UL grant to allow the UE to send a PDCP status report.

f_EUTRA_ULGrantConfiguration_Common(eutra_Cell2, // @sic R5-168359 sic@

cs_TimingInfo_Now,

f_EUTRA_DciUlInfo_Get(eutra_Cell2), // @sic R5s170009, R5s170809 sic@

cs_UplinkTimeAlignment_Synch_Auto_Def,

cs_UL_GrantConfig_OneTime);

…….

} // f_EUTRA_7_3_5_3_TestBody

After change

function f_EUTRA_7_3_5_3_TestBody(inout PDCP_SS_State_Type p_PDCP_Rec)

runs on EUTRA_PTC

{

……

//@siclog "Step 5" siclog@

// The UE transmits a  RRCConnectionReconfigurationComplete message.

v_RRCConnectionReconfiguration_v890_IEs := f_EUTRA_CellInfo_GetReconfig_BR(eutra_Cell2); //@sic R5s180630 sic@

f_EUTRA_508RRC_IntraLTE_HO_InterCell(eutra_Cell1,   // @sic R5s100430 change 2; R5s100524 sic@

eutra_Cell2,

{cs_DRB_ConfigAM_PDCP_Mode, cs_DRB2_ConfigUM_PDCP_Mode(p_PDCP_Rec.PDCP_SN_Size) }, // @sic R5-168359 sic@

tsc_C_RNTI_Def3,

-,

-,

-,

cs_UL_GrantConfig_Stop,

n32, //@sic R5s190596 sic@

-,

-,

true,

v_RRCConnectionReconfiguration_v890_IEs); // @sic R5s170095, R5s180630 sic@

//@siclog "Step 5A" siclog@

// The SS assigns additional UL grant to allow the UE to send a PDCP status report.

f_EUTRA_ULGrantConfiguration_Common(eutra_Cell2, // @sic R5-168359 sic@

cs_TimingInfo_Now,

f_EUTRA_DciUlInfo_Get(eutra_Cell2), // @sic R5s170009, R5s170809 sic@

cs_UplinkTimeAlignment_Synch_Auto_Def,

cs_UL_GrantConfig_OneTime);

…….

} // f_EUTRA_7_3_5_3_TestBody