3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210226

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1453 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC test case 8.1.4.2.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-12-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - From TS 33.501 ch. 6.9.2.1.1: NOTE 2: Since the AMF does not send the NH value to gNB/ng-eNB at the initial connection setup, the NH value associated with the NCC value one cannot be used in the next Xn handover or the next intra-gNB/intra-ng-eNB-CU handover, for the next Xn handover or the next intra-gNB-CU/intra-ng-eNB handover the horizontal key derivation (see Figure 6.9.2.1.1-1) will apply. From above, NCC = 1 is not valid according to TS 33.501. NCC = 2 should be used after NCC = 0. - pc_IP_Ping should be TRUE if v_IdleUpdatedState will be set to TESTMode_OFF - If p_TestModeState is set to TEST_LOOPModeB_ON, Close UE Test loop procedure should be triggered - During the inter-RAT HO from EUTRA to NR, NR SRB2 is already established. PDU Session Modification Request after the HO can be sent using SRB2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Increment NCC by 1 if current NCC is calculated as NCC=1 - Corrected the logic when setting v_IdleUpdatedState to TESTMode_OFF - Add TEST_LOOPModeB_ON in the condition to trigger Close UE Test loop procedure - Add option to receive PDU Session Modification Request via SRB2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.2.2.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2019-06_D20wk38 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | fl_TC_8_1_4_2_2_1_NR5GC_EUTRA_TestBody |
| --- | --- |
| Reason for change | From TS 33.501 ch. 6.9.2.1.1: NOTE 2: Since the AMF does not send the NH value to gNB/ng-eNB at the initial connection setup, the NH value associated with the NCC value one cannot be used in the next Xn handover or the next intra-gNB/intra-ng-eNB-CU handover, for the next Xn handover or the next intra-gNB-CU/intra-ng-eNB handover the horizontal key derivation (see Figure 6.9.2.1.1-1) will apply. From above, NCC = 1 is not valid according to TS 33.501. NCC = 2 should be used after NCC = 0. |
| Summary of change | Increment NCC by 1 if current NCC is calculated as NCC=1 |
| TTCN module | IRAT_Handover_NR5GC_EUTRA |
| MCC160 Comment |  |

Before change:

function fl_TC_8_1_4_2_2_1_NR5GC_EUTRA_TestBody() runs on EUTRA_5GS_PTC

{

var octetstring v_NR_HOCmd;

var EUTRA_SecurityParams_Type v_SecurityParams := f_EUTRA_Security_Get();

// @sic R5s201078 sic@ as per 33.401 7.2.8.4.3:

// the source MME shall increase its locally kept NCC value by one and compute a fresh NH using A.4

v_SecurityParams.NH := f_EUTRA_NB_Authentication_S12(v_SecurityParams.KDF,

v_SecurityParams.KASME,

v_SecurityParams.NH);

v_SecurityParams.NCC := (v_SecurityParams.NCC + 1) mod 4;

f_EUTRA_Security_Set (v_SecurityParams);

//@siclog "Step 1" siclog@

// Send latest parameters to NR

f_EUTRA38_Send_AuthParametersToNR();

f_EUTRA38_SendPduSessionInfoListToNR();

// Wait for NR to send back encoded RRC Reconfiguration message

v_NR_HOCmd := f_EUTRA_NR_WaitForCoOrd_Data(NR, "HO Cmd");

SRB.send( cas_SRB1_RrcPdu_REQ( eutra_Cell1, cs_TimingInfo_Now, cs_MobilityFromEutraCommand_Handover( tsc_RRC_TI_Def,

false,

nr,

v_NR_HOCmd,

omit)));

// UE now moved to NR

f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Postamble");

}

After change:

function fl_TC_8_1_4_2_2_1_NR5GC_EUTRA_TestBody() runs on EUTRA_5GS_PTC

{

var octetstring v_NR_HOCmd;

var EUTRA_SecurityParams_Type v_SecurityParams := f_EUTRA_Security_Get();

// @sic R5s201078 sic@ as per 33.401 7.2.8.4.3:

// the source MME shall increase its locally kept NCC value by one and compute a fresh NH using A.4

v_SecurityParams.NH := f_EUTRA_NB_Authentication_S12(v_SecurityParams.KDF,

v_SecurityParams.KASME,

v_SecurityParams.NH);

v_SecurityParams.NCC := (v_SecurityParams.NCC + 1) mod 4;

if (v_SecurityParams.NCC == 1){

v_SecurityParams.NCC := (v_SecurityParams.NCC + 1) mod 4;

}

f_EUTRA_Security_Set (v_SecurityParams);

//@siclog "Step 1" siclog@

// Send latest parameters to NR

f_EUTRA38_Send_AuthParametersToNR();

f_EUTRA38_SendPduSessionInfoListToNR();

// Wait for NR to send back encoded RRC Reconfiguration message

v_NR_HOCmd := f_EUTRA_NR_WaitForCoOrd_Data(NR, "HO Cmd");

SRB.send( cas_SRB1_RrcPdu_REQ( eutra_Cell1, cs_TimingInfo_Now, cs_MobilityFromEutraCommand_Handover( tsc_RRC_TI_Def,

false,

nr,

v_NR_HOCmd,

omit)));

// UE now moved to NR

f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Postamble");

}

## 2.2 Change 2

| Function name | f_EUTRA38_NR5GC_Preamble |  |
| --- | --- | --- |
| Reason for change | - pc_IP_Ping should be TRUE if v_IdleUpdatedState will be set to TESTMode_OFF - If p_TestModeState is set to TEST_LOOPModeB_ON, Close UE Test loop procedure should be triggered |  |
| Summary of change | - Corrected the logic when setting v_IdleUpdatedState to TESTMode_OFF - Add TEST_LOOPModeB_ON in the condition to trigger Close UE Test loop procedure |  |
| TTCN module | EUTRA38_NR5GCProcedures |  |
| MCC160 Comment |  |  |

Before change:

function f_EUTRA38_NR5GC_Preamble(EUTRA_CellId_Type p_CellId,

NR_TESTMODE_STATE_Type p_TestModeState,

EUTRA_RRC_STATE_Type p_RRC_State,

RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease,

template (omit) EPS_NetworkFeatureSupport p_NtwkFeatSupport := cs_EPS_NwkFtSup_NR,

template (omit) PLMN_List p_EquivalentPlmnList := omit) runs on EUTRA_Multi5GS_PTC

{

var NR_TESTMODE_STATE_Type v_IdleUpdatedState := p_TestModeState;

var AbsoluteCellPower_Type v_AbsoluteCellPowerLevel := tsc_ServingCellRS_EPRE;

select (p_TestModeState) { // @sic R5-204399 sic@

case (PING_Or_TESTModeB_ON, PING_Or_TEST_LOOPModeB_ON){

if (not pc_IP_Ping) { // don't need to activate test mode if using Ping

v_IdleUpdatedState := TESTMode_OFF;

}

}

}

// Store the value of NetworkFeatureSupport to be used throughout the test

f_EUTRA_MobileInfo_SetNetworkFeatureSupport (p_NtwkFeatSupport);

f_EUTRA38_MultiPDN_InitialRegistration(p_CellId, // @sic R5-204399 sic@

v_IdleUpdatedState,

p_ReleaseConnection,

p_EquivalentPlmnList,

v_AbsoluteCellPowerLevel);

if (p_RRC_State == RRC_CONNECTED) {

f_EUTRA38_MultiPDN_RbEst_Def(p_CellId);

if (p_TestModeState == PING_Or_TEST_LOOPModeB_ON and not pc_IP_Ping) { // @sic R%s200887 sic@

f_EUTRA_CloseUE_TestLoopModeB (p_CellId, '00'O);

}

}

}

After change:

function f_EUTRA38_NR5GC_Preamble(EUTRA_CellId_Type p_CellId,

NR_TESTMODE_STATE_Type p_TestModeState,

EUTRA_RRC_STATE_Type p_RRC_State,

RRCConnectionReleaseRequired_Type p_ReleaseConnection := rrcConnectionRelease,

template (omit) EPS_NetworkFeatureSupport p_NtwkFeatSupport := cs_EPS_NwkFtSup_NR,

template (omit) PLMN_List p_EquivalentPlmnList := omit) runs on EUTRA_Multi5GS_PTC

{

var NR_TESTMODE_STATE_Type v_IdleUpdatedState := p_TestModeState;

var AbsoluteCellPower_Type v_AbsoluteCellPowerLevel := tsc_ServingCellRS_EPRE;

select (p_TestModeState) { // @sic R5-204399 sic@

case (PING_Or_TESTModeB_ON, PING_Or_TEST_LOOPModeB_ON){

if (pc_IP_Ping) { // don't need to activate test mode if using Ping

v_IdleUpdatedState := TESTMode_OFF;

}

}

}

// Store the value of NetworkFeatureSupport to be used throughout the test

f_EUTRA_MobileInfo_SetNetworkFeatureSupport (p_NtwkFeatSupport);

f_EUTRA38_MultiPDN_InitialRegistration(p_CellId, // @sic R5-204399 sic@

v_IdleUpdatedState,

p_ReleaseConnection,

p_EquivalentPlmnList,

v_AbsoluteCellPowerLevel);

if (p_RRC_State == RRC_CONNECTED) {

f_EUTRA38_MultiPDN_RbEst_Def(p_CellId);

if (p_TestModeState == TEST_LOOPModeB_ON or (p_TestModeState == PING_Or_TEST_LOOPModeB_ON and not pc_IP_Ping)) { // @sic R%s200887 sic@

f_EUTRA_CloseUE_TestLoopModeB (p_CellId, '00'O);

}

}

}

## 2.3 Change 3

| Function name | f_NR5GC_IRAT_S1ToN1_SessionModification |  |
| --- | --- | --- |
| Reason for change | During the inter-RAT HO from EUTRA to NR, NR SRB2 is already established. PDU Session Modification Request after the HO can be sent using SRB2. |  |
| Summary of change | Add option to receive PDU Session Modification Request via SRB2. |  |
| TTCN module | NR5GC_IRAT_CommonFunctions |  |
| MCC160 Comment |  |  |

Before change:

function f_NR5GC_IRAT_S1ToN1_SessionModification(NR_CellId_Type p_CellId,

EUTRA_NR_PduSessionInfoList_Type p_PduSessionsFromLTE,

boolean p_FailWhenK_LessN := true,

integer p_ExpectedNumberOfNewPDUSessions := (pc_noOf_PDUsSameConnection + pc_noOf_PDUsNewConnection)) runs on NR5GC_PTC return boolean  //Anritsu TTCN CR R5s201413

{

var template (value) PDUSessionInfoList_Type v_FromLTEList;

var NG_NAS_UL_PduList_Type v_PduSessionModificationRequestList := {};

var IntegerList_Type v_PDU_SessionIdList := {};

var GSM_MobilityInfo_Type v_GSM_MobilityInfo;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var NG_NAS_UL_Pdu_Type v_NG_NAS_UL_Pdu;

var NG_NAS_UL_Message_Type v_GSMMsg;

var integer v_PDU_SessionId;

var template (value) RadioBearerConfig v_RadioBearerConfig;

var template(value) CellGroupConfig v_CellGroupConfig;

var template (value) NR_RadioBearerList_Type v_RadioBearerList;  // list of DRBs to be configured at the SS (SRB2 alredy configured in SS at cell creation/RRC release procedure => not needed here)

var template (value) SDAP_ConfigurationList_Type v_SDAP_ConfigurationList;

var boolean v_ReregisterIMS := false;

var template (value) NG_NAS_DL_Pdu_Type v_NG_NAS_DL_Pdu;

var NR_DRBInfoList_Type v_NewDRBInfoList;

var integer N := p_ExpectedNumberOfNewPDUSessions;  // number of PDU SESSION MODIFICATION REQUEST messages being expected

var integer K := 0;   // number of PDU SESSION MODIFICATION REQUEST messages already processed (i.e. the corresponding PDU SESSION MODIFICATION COMMAND and PDU SESSION MODIFICATION COMPLETE have been sent/received)

var integer L := 0;   // number of PDU SESSION MODIFICATION REQUEST messages being received

var boolean v_WaitForComplete := false;

var template (present) NG_UE_SM_Cap v_NG_UE_SM_Cap := ?;                         // according to 38.508-1 Table 4.5A.2C.2.3-1

var template (present) IntegrityProtMaxDataRate v_IntegrityProtMaxDataRate := ?; // according to 38.508-1 Table 4.5A.2C.2.3-1

var integer i;

timer t_Wait := 10.0; // @sic R5-204330 sic@

// Build the GSMMobilityInfo for all received PDUs, but don't store it (some may be established, rather than modified)

for (i := 0; i < N; i := i + 1) {

v_FromLTEList[i] := cs_GSM_MobilityInfo(int2oct(p_PduSessionsFromLTE[i].PDU_SessionId, 1), -, p_PduSessionsFromLTE[i].IsIMS); // @sic R5s200564 sic@

}

t_Wait.start;  // @sic R5-204330 sic@

while (K < N) {

alt { // @sic R5-204330 sic@

[] SRB.receive(car_NR_SRB_NasPdu_IND(p_CellId, tsc_NR_RbId_SRB1, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp      //Anritsu TTCN CR R5s201413

{

v_NG_NAS_UL_Pdu := v_ReceivedAsp.Signalling.Nas[0].Pdu;

v_GSMMsg := v_NG_NAS_UL_Pdu.PiggybackedPduList[0].Msg;

if (ischosen(v_GSMMsg.pdu_Session_Modification_Request)) {

t_Wait.stop;

v_PduSessionModificationRequestList[L] := v_NG_NAS_UL_Pdu;

L := L + 1;

if (L > N) { // @sic R5-204330 sic@

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "L > N");

}

}

else if (v_WaitForComplete and ischosen(v_GSMMsg.pdu_Session_Modification_Complete)) {

if (not f_Check_NG_PDUSessionModificationComplete(v_NG_NAS_UL_Pdu, v_GSM_MobilityInfo.SessionId, v_GSM_MobilityInfo.PTI)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Invalid PDU SESSION MODIFICATION COMPLETE");

}

v_WaitForComplete := false;

K := K + 1;

}

else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Unexpected NAS PDU");

}

if (not v_WaitForComplete and (K < L)) {   // => process the next PDU SESSION MODIFICATION REQUEST

v_NG_NAS_UL_Pdu := v_PduSessionModificationRequestList[K];

if (not f_Check_NG_PDUSessionModificationReq(v_GSM_MobilityInfo, valueof(v_FromLTEList), v_NG_NAS_UL_Pdu, -, -, v_NG_UE_SM_Cap, -, -, -, v_IntegrityProtMaxDataRate, -, -, -, -, -, cr_NG_Request_Type('101'B))) {     // v_GSM_MobilityInfo is out parameter of f_Check_NG_PDUSessionModificationReq

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Invalid PDU SESSION MODIFICATION REQUEST");

}

// ok, this PDU is to be modified, so can now store it in our list

v_PDU_SessionId := oct2int(v_GSM_MobilityInfo.SessionId); // @sic R5s200564 sic@

if (f_IntegerList_Search(v_PDU_SessionIdList, v_PDU_SessionId) < 0) {

v_PDU_SessionIdList[K] := v_PDU_SessionId;

} else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "There has been a PDU SESSION MODIFICATION REQUEST with the same session id already");

}

f_NR5GC_MobileInfo_AddPDUSession(v_GSM_MobilityInfo); //Anritsu TTCN CR R5s201413

for (i := 0; i < N; i := i + 1) {

if (p_PduSessionsFromLTE[i].PDU_SessionId == oct2int(v_GSM_MobilityInfo.SessionId)) {

f_IP_ChangeDrbMappingNR(IP, p_CellId, cs_IP_QosFlowId(p_PduSessionsFromLTE[i].PDU_SessionId, p_PduSessionsFromLTE[i].QFI), p_PduSessionsFromLTE[i].PdnIndex);

v_ReregisterIMS := v_ReregisterIMS or p_PduSessionsFromLTE[i].IsIMS;

break;

}

}

/*  As DRBs are not configured, and IMS Re-registration is not specified in the prose, then this is not needed

if (v_ReregisterIMS) {

// Tell IMS to start waiting for the Register message.

f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_ReregistrationStart(NR));

}*/

v_NG_NAS_DL_Pdu := f_Get_NG_PDUSessionModificationCommand(v_GSM_MobilityInfo.SessionId, v_GSM_MobilityInfo);

SRB.send(cas_NR_SRB_NasPdu_REQ(p_CellId, -, -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_NG_NAS_DL_Pdu)));

v_WaitForComplete := true;

t_Wait.start;  // restart timer to wait for next Request

}

}

[] t_Wait.timeout {

if (p_FailWhenK_LessN and (K < N)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Not enough PDU Session Modification Request messages");

} else if (not p_FailWhenK_LessN) { // New message not received in time, so just end the loop  @sic R5-204330 sic@

break;

}

}

}

}

// Timer will have been started again in loop, but loop is now finished, so need to stop timer

t_Wait.stop;

// now build and store global DRB list - as we're not piggybacking the Mod. Cmd with a Reconfig,

// we can build the whole list in one go, rather than doing it one by one within the loop

f_BuildRRCReconfigParamsNewDRBs (v_RadioBearerConfig,

v_CellGroupConfig,

v_RadioBearerList,

v_SDAP_ConfigurationList,

p_CellId,

f_NR5GC_MobileInfo_GetSessionInfoList(),

SRB1);

//  @sic R5-204330 sic@

v_NewDRBInfoList := f_BuildDRBNewDRBList (valueof(v_RadioBearerConfig.drb_ToAddModList), valueof(v_RadioBearerList), valueof(v_CellGroupConfig)); //  @sic R5-196745 sic@

// Store updated DRB information, but don't configure these DRBs

f_NR5GC_MobileInfo_SetDRBInfoList(v_NewDRBInfoList);

// calling function needs to wait for IMS to finish

// FFS EUTRA38_DRB....this will need IMS (and other?) DRBs to be configured - which they currently aren't

return v_ReregisterIMS;

}

After change:

function f_NR5GC_IRAT_S1ToN1_SessionModification(NR_CellId_Type p_CellId,

EUTRA_NR_PduSessionInfoList_Type p_PduSessionsFromLTE,

boolean p_FailWhenK_LessN := true,

integer p_ExpectedNumberOfNewPDUSessions := (pc_noOf_PDUsSameConnection + pc_noOf_PDUsNewConnection),

NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB1) runs on NR5GC_PTC return boolean  //Anritsu TTCN CR R5s201413

{

var template (value) PDUSessionInfoList_Type v_FromLTEList;

var NG_NAS_UL_PduList_Type v_PduSessionModificationRequestList := {};

var IntegerList_Type v_PDU_SessionIdList := {};

var GSM_MobilityInfo_Type v_GSM_MobilityInfo;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var NG_NAS_UL_Pdu_Type v_NG_NAS_UL_Pdu;

var NG_NAS_UL_Message_Type v_GSMMsg;

var integer v_PDU_SessionId;

var template (value) RadioBearerConfig v_RadioBearerConfig;

var template(value) CellGroupConfig v_CellGroupConfig;

var template (value) NR_RadioBearerList_Type v_RadioBearerList;  // list of DRBs to be configured at the SS (SRB2 alredy configured in SS at cell creation/RRC release procedure => not needed here)

var template (value) SDAP_ConfigurationList_Type v_SDAP_ConfigurationList;

var boolean v_ReregisterIMS := false;

var template (value) NG_NAS_DL_Pdu_Type v_NG_NAS_DL_Pdu;

var NR_DRBInfoList_Type v_NewDRBInfoList;

var integer N := p_ExpectedNumberOfNewPDUSessions;  // number of PDU SESSION MODIFICATION REQUEST messages being expected

var integer K := 0;   // number of PDU SESSION MODIFICATION REQUEST messages already processed (i.e. the corresponding PDU SESSION MODIFICATION COMMAND and PDU SESSION MODIFICATION COMPLETE have been sent/received)

var integer L := 0;   // number of PDU SESSION MODIFICATION REQUEST messages being received

var boolean v_WaitForComplete := false;

var template (present) NG_UE_SM_Cap v_NG_UE_SM_Cap := ?;                         // according to 38.508-1 Table 4.5A.2C.2.3-1

var template (present) IntegrityProtMaxDataRate v_IntegrityProtMaxDataRate := ?; // according to 38.508-1 Table 4.5A.2C.2.3-1

var integer i;

var SRBs_AlreadyConfigured v_SRBs := SRB1;

timer t_Wait := 10.0; // @sic R5-204330 sic@

// Build the GSMMobilityInfo for all received PDUs, but don't store it (some may be established, rather than modified)

for (i := 0; i < N; i := i + 1) {

v_FromLTEList[i] := cs_GSM_MobilityInfo(int2oct(p_PduSessionsFromLTE[i].PDU_SessionId, 1), -, p_PduSessionsFromLTE[i].IsIMS); // @sic R5s200564 sic@

}

t_Wait.start;  // @sic R5-204330 sic@

while (K < N) {

alt { // @sic R5-204330 sic@

[] SRB.receive(car_NR_SRB_NasPdu_IND(p_CellId, p_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp      //Anritsu TTCN CR R5s201413

{

v_NG_NAS_UL_Pdu := v_ReceivedAsp.Signalling.Nas[0].Pdu;

v_GSMMsg := v_NG_NAS_UL_Pdu.PiggybackedPduList[0].Msg;

if (ischosen(v_GSMMsg.pdu_Session_Modification_Request)) {

t_Wait.stop;

v_PduSessionModificationRequestList[L] := v_NG_NAS_UL_Pdu;

L := L + 1;

if (L > N) { // @sic R5-204330 sic@

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "L > N");

}

}

else if (v_WaitForComplete and ischosen(v_GSMMsg.pdu_Session_Modification_Complete)) {

if (not f_Check_NG_PDUSessionModificationComplete(v_NG_NAS_UL_Pdu, v_GSM_MobilityInfo.SessionId, v_GSM_MobilityInfo.PTI)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Invalid PDU SESSION MODIFICATION COMPLETE");

}

v_WaitForComplete := false;

K := K + 1;

}

else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Unexpected NAS PDU");

}

if (not v_WaitForComplete and (K < L)) {   // => process the next PDU SESSION MODIFICATION REQUEST

v_NG_NAS_UL_Pdu := v_PduSessionModificationRequestList[K];

if (not f_Check_NG_PDUSessionModificationReq(v_GSM_MobilityInfo, valueof(v_FromLTEList), v_NG_NAS_UL_Pdu, -, -, v_NG_UE_SM_Cap, -, -, -, v_IntegrityProtMaxDataRate, -, -, -, -, -, cr_NG_Request_Type('101'B))) {     // v_GSM_MobilityInfo is out parameter of f_Check_NG_PDUSessionModificationReq

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Invalid PDU SESSION MODIFICATION REQUEST");

}

// ok, this PDU is to be modified, so can now store it in our list

v_PDU_SessionId := oct2int(v_GSM_MobilityInfo.SessionId); // @sic R5s200564 sic@

if (f_IntegerList_Search(v_PDU_SessionIdList, v_PDU_SessionId) < 0) {

v_PDU_SessionIdList[K] := v_PDU_SessionId;

} else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "There has been a PDU SESSION MODIFICATION REQUEST with the same session id already");

}

f_NR5GC_MobileInfo_AddPDUSession(v_GSM_MobilityInfo); //Anritsu TTCN CR R5s201413

for (i := 0; i < N; i := i + 1) {

if (p_PduSessionsFromLTE[i].PDU_SessionId == oct2int(v_GSM_MobilityInfo.SessionId)) {

f_IP_ChangeDrbMappingNR(IP, p_CellId, cs_IP_QosFlowId(p_PduSessionsFromLTE[i].PDU_SessionId, p_PduSessionsFromLTE[i].QFI), p_PduSessionsFromLTE[i].PdnIndex);

v_ReregisterIMS := v_ReregisterIMS or p_PduSessionsFromLTE[i].IsIMS;

break;

}

}

/*  As DRBs are not configured, and IMS Re-registration is not specified in the prose, then this is not needed

if (v_ReregisterIMS) {

// Tell IMS to start waiting for the Register message.

f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_ReregistrationStart(NR));

}*/

v_NG_NAS_DL_Pdu := f_Get_NG_PDUSessionModificationCommand(v_GSM_MobilityInfo.SessionId, v_GSM_MobilityInfo);

SRB.send(cas_NR_SRB_NasPdu_REQ(p_CellId, -, -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_NG_NAS_DL_Pdu)));

v_WaitForComplete := true;

t_Wait.start;  // restart timer to wait for next Request

}

}

[] t_Wait.timeout {

if (p_FailWhenK_LessN and (K < N)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Not enough PDU Session Modification Request messages");

} else if (not p_FailWhenK_LessN) { // New message not received in time, so just end the loop  @sic R5-204330 sic@

break;

}

}

}

}

// Timer will have been started again in loop, but loop is now finished, so need to stop timer

t_Wait.stop;

// now build and store global DRB list - as we're not piggybacking the Mod. Cmd with a Reconfig,

// we can build the whole list in one go, rather than doing it one by one within the loop

if (p_SRB == tsc_NR_RbId_SRB2){

v_SRBs := SRB1_2;

}

f_BuildRRCReconfigParamsNewDRBs (v_RadioBearerConfig,

v_CellGroupConfig,

v_RadioBearerList,

v_SDAP_ConfigurationList,

p_CellId,

f_NR5GC_MobileInfo_GetSessionInfoList(),

v_SRBs);

//  @sic R5-204330 sic@

v_NewDRBInfoList := f_BuildDRBNewDRBList (valueof(v_RadioBearerConfig.drb_ToAddModList), valueof(v_RadioBearerList), valueof(v_CellGroupConfig)); //  @sic R5-196745 sic@

// Store updated DRB information, but don't configure these DRBs

f_NR5GC_MobileInfo_SetDRBInfoList(v_NewDRBInfoList);

// calling function needs to wait for IMS to finish

// FFS EUTRA38_DRB....this will need IMS (and other?) DRBs to be configured - which they currently aren't

return v_ReregisterIMS;

}

## 2.4 Change 4

| Function name | fl_TC_8_1_4_2_2_1_TestBody |  |
| --- | --- | --- |
| Reason for change | During the inter-RAT HO from EUTRA to NR, NR SRB2 is already established. PDU Session Modification Request after the HO can be sent using SRB2. |  |
| Summary of change | Receive PDU Session Modification Request via SRB2 |  |
| TTCN module | IRAT_Handover_NR5GC_NR |  |
| MCC160 Comment |  |  |

Before change:

function fl_TC_8_1_4_2_2_1_TestBody() runs on NR5GC_PTC

{

var EUTRA_NR_CoOrd_Security_Type v_SecurityRx := f_EUTRA_NR_WaitForCoOrd_Security (EUTRA);

var EUTRA_NR_PduSessionInfoList_Type v_PduSessionInfoList;

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var TriggerOrTimeout_Type v_TriggerOrTimeout;

var PDUSessionInfoList_Type v_PDUSessionInfoList;

var template (value) QosFlow_Identification_Type v_QosFlowId;

var NR_DRB_COMMON_IND v_NR_DRB_COMMON_IND;

var octetstring v_SduTX;

var octetstring v_SduRX;

timer t_ReregisterIMS := 10.0;

//@siclog "Step 0" siclog@

//@sic R5-198922 sic@

f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2);

f_Delay (5.0);

//@siclog "Steps 1-2" siclog@

// derive 5G parameters from values received from EUTRA

f_NR5GC_MappedContextFromS1_InitNAS (nr_Cell1, v_SecurityRx, true);

v_PduSessionInfoList := f_EUTRA_NR_WaitForCoOrd_PduSessionInfoList(EUTRA);

// this also sends octetstring to EUTRA and waits for the Complete message

f_NR5GC_508RRC_InterRAT_HO_ToNR(nr_Cell1, v_PduSessionInfoList, v_SecurityRx); // @sic R5s201078 sic@

f_NR_PreliminaryPass( __FILE__, __LINE__, "Step 2" );

//@siclog "Steps 3-5" siclog@

SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected))) -> value v_ReceivedAsp; // @sic R5s201078 sic@

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, Mobility)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell1, v_GMM_MobilityInfo, -, -, noRrcConnectionRelease, -, tsc_NR_RbId_SRB2);

// Procedure for UE-requested PDU session modification according to 38.508 Table 4.9.14.2.2-1

f_NR5GC_IRAT_S1ToN1_SessionModification(nr_Cell1, v_PduSessionInfoList);

//  Deal with potential IMS Re-Registration

if (f_NR_GetDefaultDRB_ForIMSPDUSession() > -1) { // There is an IMS session

v_TriggerOrTimeout := f_IMS_IPCAN_WaitForTriggerOrTimeout(IMS[tsc_Index_IMS1], t_ReregisterIMS);

select (v_TriggerOrTimeout) {

case (TRIGGER) {                                          // => IMS Re-Registration has been started ...

f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]);        //    ... wait for IMS Re-Registration being finished

}

case (TIMEOUT) {

f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_ReregistrationStop);

}

}

}

// @siclog "Step 7 Void" siclog@

// @siclog "Steps 8-9" siclog@

// The SS sends one IP Packet to the UE on the default DRB associated with the first PDU session.

v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList();

if (isvalue(v_PDUSessionInfoList)) { // @sic R5s200517 sic@

// @sic R5-204453 sic@

v_SduTX := f_LoopbackModeBorPing_GetDataTX(f_NR5GC_GetPdnIndex(valueof(v_PDUSessionInfoList[0])), defaultBearer, 1, false);

v_QosFlowId := f_NR_GetQosFlowIdentification_ForDRBId(f_NR_GetDefaultDRB_InDRBList (f_NR_GetDRBList_ForGivenPDUSession(oct2int((v_PDUSessionInfoList[0].SessionId)))));

DRB.send(cas_NR_DRB_COMMON_REQ_SDAP_SDUList(nr_Cell1, v_QosFlowId, { v_SduTX }));

DRB.receive(car_NR_DRB_COMMON_IND_SDAP_SDUList(nr_Cell1, v_QosFlowId, { ? })) -> value v_NR_DRB_COMMON_IND;

v_SduRX := v_NR_DRB_COMMON_IND.U_Plane.SlotData.PduSduList.SdapSdu[0];

if (v_SduTX == v_SduRX) {

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 9");

} else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Failed IP check");

}

} else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "No PDU Session");

}

}

After change:

function fl_TC_8_1_4_2_2_1_TestBody() runs on NR5GC_PTC

{

var EUTRA_NR_CoOrd_Security_Type v_SecurityRx := f_EUTRA_NR_WaitForCoOrd_Security (EUTRA);

var EUTRA_NR_PduSessionInfoList_Type v_PduSessionInfoList;

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var TriggerOrTimeout_Type v_TriggerOrTimeout;

var PDUSessionInfoList_Type v_PDUSessionInfoList;

var template (value) QosFlow_Identification_Type v_QosFlowId;

var NR_DRB_COMMON_IND v_NR_DRB_COMMON_IND;

var octetstring v_SduTX;

var octetstring v_SduRX;

timer t_ReregisterIMS := 10.0;

//@siclog "Step 0" siclog@

//@sic R5-198922 sic@

f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2);

f_Delay (5.0);

//@siclog "Steps 1-2" siclog@

// derive 5G parameters from values received from EUTRA

f_NR5GC_MappedContextFromS1_InitNAS (nr_Cell1, v_SecurityRx, true);

v_PduSessionInfoList := f_EUTRA_NR_WaitForCoOrd_PduSessionInfoList(EUTRA);

// this also sends octetstring to EUTRA and waits for the Complete message

f_NR5GC_508RRC_InterRAT_HO_ToNR(nr_Cell1, v_PduSessionInfoList, v_SecurityRx); // @sic R5s201078 sic@

f_NR_PreliminaryPass( __FILE__, __LINE__, "Step 2" );

//@siclog "Steps 3-5" siclog@

SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected))) -> value v_ReceivedAsp; // @sic R5s201078 sic@

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, Mobility)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell1, v_GMM_MobilityInfo, -, -, noRrcConnectionRelease, -, tsc_NR_RbId_SRB2);

// Procedure for UE-requested PDU session modification according to 38.508 Table 4.9.14.2.2-1

f_NR5GC_IRAT_S1ToN1_SessionModification(nr_Cell1, v_PduSessionInfoList, -, -, tsc_NR_RbId_SRB2);

//  Deal with potential IMS Re-Registration

if (f_NR_GetDefaultDRB_ForIMSPDUSession() > -1) { // There is an IMS session

v_TriggerOrTimeout := f_IMS_IPCAN_WaitForTriggerOrTimeout(IMS[tsc_Index_IMS1], t_ReregisterIMS);

select (v_TriggerOrTimeout) {

case (TRIGGER) {                                          // => IMS Re-Registration has been started ...

f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]);        //    ... wait for IMS Re-Registration being finished

}

case (TIMEOUT) {

f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_ReregistrationStop);

}

}

}

// @siclog "Step 7 Void" siclog@

// @siclog "Steps 8-9" siclog@

// The SS sends one IP Packet to the UE on the default DRB associated with the first PDU session.

v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList();

if (isvalue(v_PDUSessionInfoList)) { // @sic R5s200517 sic@

// @sic R5-204453 sic@

v_SduTX := f_LoopbackModeBorPing_GetDataTX(f_NR5GC_GetPdnIndex(valueof(v_PDUSessionInfoList[0])), defaultBearer, 1, false);

v_QosFlowId := f_NR_GetQosFlowIdentification_ForDRBId(f_NR_GetDefaultDRB_InDRBList (f_NR_GetDRBList_ForGivenPDUSession(oct2int((v_PDUSessionInfoList[0].SessionId)))));

DRB.send(cas_NR_DRB_COMMON_REQ_SDAP_SDUList(nr_Cell1, v_QosFlowId, { v_SduTX }));

DRB.receive(car_NR_DRB_COMMON_IND_SDAP_SDUList(nr_Cell1, v_QosFlowId, { ? })) -> value v_NR_DRB_COMMON_IND;

v_SduRX := v_NR_DRB_COMMON_IND.U_Plane.SlotData.PduSduList.SdapSdu[0];

if (v_SduTX == v_SduRX) {

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 9");

} else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Failed IP check");

}

} else {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "No PDU Session");

}

}