3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210227

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4567 | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE test case 8.2.4.13a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI9_Test |  |  |  |  |  | Date: |  |  | 2020-02-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | At Step 8, while Cell 1 EARFCN is configured as above 65535 (band greater than 64), RRC Connection reconfiguration fails as message content doesn’t match with the 3GPP specs. This issue eventually leads to the test case failure as the RRCConnection reconfiguration is getting failed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | v_Frequency_IE_Cell1.UL_DL_Earfcn.dl_CarrierFreq_r9 and v_Frequency_IE_Cell28.UL_DL_Earfcn.dl_CarrierFreq_r9 are interchanged in the function f_EUTRA_RRCConnectionReconfiguration_MeasConfig() which is generating the RRCConnection reconfiguration message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case if used EARFCN greater than 65535 (band greater than B64). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.4.13a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2019-06_D20wk38 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | f_EUTRA_RRCConnectionReconfiguration_MeasConfig() |
| --- | --- |
| Reason for change | At Step 8, while Cell 1 EARFCN is configured as above 65535 (band greater than 64), RRC Connection reconfiguration fails as message content doesn’t match with the 3GPP specs. This issue eventually leads to the test case failure as the RRCConnection reconfiguration is getting failed. |
| Summary of change | v_Frequency_IE_Cell1.UL_DL_Earfcn.dl_CarrierFreq_r9 and v_Frequency_IE_Cell28.UL_DL_Earfcn.dl_CarrierFreq_r9 are interchanged in the function f_EUTRA_RRCConnectionReconfiguration_MeasConfig() which is generating the RRCConnection reconfiguration message. |
| TTCN module | RRC_Handover.ttcn |
| MCC160 Comment |  |

Before change:

<<SKIPPED CODE>>

f_EUTRA_RRCConnectionReconfiguration_MeasConfig (eutra_Cell28, //@sic R5s130649 sic@

cs_508_RRCConnectionReconfiguration_ConditionMEAS (v_RRC_TI,

cs_Measurement_Config_A3_InterFreq_8_2_4_1Xa_Step8(  //@sic R5-133693 sic@

v_Frequency_IE_Cell1.UL_DL_Earfcn.dl_CarrierFreq,

v_ChBandDependency_Cell1.AllowedMeasBandwidth,

v_Frequency_IE_Cell28.UL_DL_Earfcn.dl_CarrierFreq,

v_ChBandDependency_Cell28.AllowedMeasBandwidth,

cs_508_MeasGapConfig_GP1,

f_ReturnMeasObjectToAddModList_2Entries_v9e0(                                                                                                            v_Frequency_IE_Cell1.UL_DL_Earfcn.dl_CarrierFreq_r9,

v_Frequency_IE_Cell28.UL_DL_Earfcn.dl_CarrierFreq_r9)))); // @sic R5-171995 sic@

<<SKIPPED CODE>>

After change:

<<SKIPPED CODE>>

f_EUTRA_RRCConnectionReconfiguration_MeasConfig (eutra_Cell28, //@sic R5s130649 sic@

cs_508_RRCConnectionReconfiguration_ConditionMEAS (v_RRC_TI,

cs_Measurement_Config_A3_InterFreq_8_2_4_1Xa_Step8(  //@sic R5-133693 sic@

v_Frequency_IE_Cell1.UL_DL_Earfcn.dl_CarrierFreq,

v_ChBandDependency_Cell1.AllowedMeasBandwidth,

v_Frequency_IE_Cell28.UL_DL_Earfcn.dl_CarrierFreq,

v_ChBandDependency_Cell28.AllowedMeasBandwidth,

cs_508_MeasGapConfig_GP1,

f_ReturnMeasObjectToAddModList_2Entries_v9e0(//TTCN Change by Anritsu

v_Frequency_IE_Cell28.UL_DL_Earfcn.dl_CarrierFreq_r9,

v_Frequency_IE_Cell1.UL_DL_Earfcn.dl_CarrierFreq_r9)))); // @sic R5-171995 sic@

<<SKIPPED CODE>>