3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210229

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4569 | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to MAC test case 7.1.6.5 for CAT-M1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, LTE_extDRX-UEConTest |  |  |  |  |  | Date: |  |  | 2020-12-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - The UL grant configuration (SF4) can conflict with Scheduling request (SF5) in case of with FDD_TypeB_HalfDuplex. Proposed change modifies the UL grant configuration to SF8 to include guard subframe between UL and DL. Reference CR R5s180475. - CAT-M1 Status PDU is expected after 6 subframes as 2 subframe difference is the schedule wait between DCI and DL-SCH (Ref 36.213 cl. 7.1.11 PDSCH subframe assignment for BL/CE UE) - PUSCH(BSR) is expected after 4 subframes from Grant Schedule but in cat-m1 case ‘f_AckNack_TimingInfo’ adds 6 subframes to timing as part of change 2. Therefore, 2 subframes are subtracted at reception of BSR |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The UL grant configuration (SF4) can conflict with Scheduling request (SF5) in case of with FDD_TypeB_HalfDuplex. Proposed change modifies the UL grant configuration to SF8 to include guard subframe between UL and DL. Reference CR R5s180475. - CAT-M1 Status PDU is expected after 6 subframes as 2 subframe difference is the schedule wait between DCI and DL-SCH (Ref 36.213 cl. 7.1.11 PDSCH subframe assignment for BL/CE UE) - PUSCH(BSR) is expected after 4 subframes from Grant Schedule but in cat-m1 case ‘f_AckNack_TimingInfo’ adds 6 subframes to timing as part of change 2. Therefore, 2 subframes are subtracted at reception of BSR |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | conformant Cat-M UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.6.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2019-06_D20wk38 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | function f_TC_7_1_6_5_EUTRA() |
| --- | --- |
| Reason for change | The UL grant configuration (SF4) can be in conflict with Scheduling request (SF5) in case of with FDD_TypeB_HalfDuplex. Proposed change modifies the UL grant configuration to SF8 to include guard subframe between UL and DL. Reference CR R5s180475 |
| Summary of change | Modified UL grant configuration to SF8 for cat-M |
| TTCN module | LTE_A_PRO\7_1\MAC_eDRX.ttcn |
| MCC160 Comment |  |

Before change

....

module MAC_eDRX {

import from CommonDefs all;

import from CommonAspDefs all;

import from Common4G5G_Templates all;

import from EUTRA_CellInfo all;

import from EUTRA_CellInfoInit all;

import from EUTRA_ConfigurationSteps all;

import from EUTRA_CommonProcedures all;

import from EUTRA_CommonDefs all;

import from EUTRA_NB_CommonDefs all;

import from EUTRA_NB_ASP_Templates all;

import from EUTRA_CellCfg_Templates all;

import from EUTRA_RRC_ASN1_Definitions language "ASN.1:2002" all with {encode "UNALIGNED_PER_OctetAligned"};

import from EUTRA_RRC_Templates all;

import from EUTRA_Component all;

import from EUTRA_L2_CommonTemplates all;

import from EUTRA_L2_CommonFunctions all;

import from EUTRA_MAC_Common all;

import from EUTRA_LoopBack all;

import from EUTRA_RRCSteps all;

…

function f_TC_7_1_6_5_EUTRA ( ) runs on EUTRA_PTC

{ /* eDRX operation / Long cycle  configured / Parameters configured by RRC */

const integer tsc_LongDRXCycle := 5120;

const integer tsc_DrxStartOffset := 4; //@sic R5s110698 sic@

var SubFrameTiming_Type v_Timing1 := valueof(cs_SubFrameTiming(0, 0, 0));

var EUTRA_FDD_TDD_Mode_Type v_EUTRA_FDD_TDD_Mode;

///////////////////////////////////////////////////////////////////////////////////

//      PREAMBLE

//////////////////////////////////////////////////////////////////////////////////

f_EUTRA_Init(c1);

v_EUTRA_FDD_TDD_Mode := f_EUTRA_CellInfo_GetFDD_TDD (eutra_Cell1);

f_EUTRA_CellConfig_Def (eutra_Cell1);

f_EUTRA_Preamble(eutra_Cell1, STATE2A_TESTLOOP_ModeA);

//f_EUTRA_SS_CellConfig_DciDlInfo_NoRetransmission(eutra_Cell1);

// creates cell and performs registration

//@sic R5-104796 R5-106675 DRX support on SS mandated sic@

f_EUTRA_LoopBackActivation_State4_Def( eutra_Cell1, cs_UE_TestLoopModeA_LB_Setup_1DRB(0, tsc_DRB1) );

///////////////////////////////////////////////////////////////////////////////////

//      Test Body

//////////////////////////////////////////////////////////////////////////////////

f_EUTRA_TestBody_Set(true);

f_EUTRA_ULGrantConfiguration_Common(eutra_Cell1,

cs_TimingInfo_SubFrame(4),

cs_DciInfo_CcchDcchDtchUL_Explicit(4, 4), // Imcs = Itbs =4, Nprb Results in TB size of 328 selected to be neither too big nor too small

cs_UplinkTimeAlignment_Synch_Auto_Def,                  //@sic R5s110618 sic@

cs_UL_GrantConfig_ContPeriodic(5));       // periodic grant every 5ms (80% of 6ms the Inactivity Timer)

....

After change

....

/******************************************************************************/

// @copyright   Copyright Notification

//   No part may be reproduced except as authorized by written permission.

//   The copyright and the foregoing restriction extend to reproduction in all media.

//   (c) 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

//   All rights reserved.

// @version: IWD_20wk38

// $Date: 2019-12-17 18:27:35 +0100 (Tue, 17 Dec 2019) $

// $Rev: 26774 $

/******************************************************************************/

module MAC_eDRX {

import from CommonDefs all;

import from CommonAspDefs all;

import from Common4G5G_Templates all;

import from EUTRA_CellInfo all;

import from EUTRA_CellInfoInit all;

import from EUTRA_ConfigurationSteps all;

import from EUTRA_CommonProcedures all;

import from EUTRA_CommonDefs all;

import from EUTRA_NB_CommonDefs all;

import from EUTRA_NB_ASP_Templates all;

import from EUTRA_CellCfg_Templates all;

import from EUTRA_RRC_ASN1_Definitions language "ASN.1:2002" all with {encode "UNALIGNED_PER_OctetAligned"};

import from EUTRA_RRC_Templates all;

import from EUTRA_Component all;

import from EUTRA_L2_CommonTemplates all;

import from EUTRA_L2_CommonFunctions all;

import from EUTRA_MAC_Common all;

import from EUTRA_LoopBack all;

import from EUTRA_RRCSteps all;

import from Parameters all; //Anritsu 7.1.6.5

…

function f_TC_7_1_6_5_EUTRA ( ) runs on EUTRA_PTC

{ /* eDRX operation / Long cycle  configured / Parameters configured by RRC */

const integer tsc_LongDRXCycle := 5120;

const integer tsc_DrxStartOffset := 4; //@sic R5s110698 sic@

var SubFrameTiming_Type v_Timing1 := valueof(cs_SubFrameTiming(0, 0, 0));

var EUTRA_FDD_TDD_Mode_Type v_EUTRA_FDD_TDD_Mode;

///////////////////////////////////////////////////////////////////////////////////

//      PREAMBLE

//////////////////////////////////////////////////////////////////////////////////

f_EUTRA_Init(c1);

v_EUTRA_FDD_TDD_Mode := f_EUTRA_CellInfo_GetFDD_TDD (eutra_Cell1);

f_EUTRA_CellConfig_Def (eutra_Cell1);

f_EUTRA_Preamble(eutra_Cell1, STATE2A_TESTLOOP_ModeA);

//f_EUTRA_SS_CellConfig_DciDlInfo_NoRetransmission(eutra_Cell1);

// creates cell and performs registration

//@sic R5-104796 R5-106675 DRX support on SS mandated sic@

f_EUTRA_LoopBackActivation_State4_Def( eutra_Cell1, cs_UE_TestLoopModeA_LB_Setup_1DRB(0, tsc_DRB1) );

///////////////////////////////////////////////////////////////////////////////////

//      Test Body

//////////////////////////////////////////////////////////////////////////////////

f_EUTRA_TestBody_Set(true);

if (pc_ue_CategoryDL_M1) {

f_EUTRA_ULGrantConfiguration_Common(eutra_Cell1,

cs_TimingInfo_SubFrame(8), //Anritsu 7.1.6.5

cs_DciInfo_CcchDcchDtchUL_Explicit(4, 4),

cs_UplinkTimeAlignment_Synch_Auto_Def,    //@sic R5s110618 sic@

cs_UL_GrantConfig_ContPeriodic(5)); }

else{

f_EUTRA_ULGrantConfiguration_Common(eutra_Cell1,

cs_TimingInfo_SubFrame(4),

cs_DciInfo_CcchDcchDtchUL_Explicit(4, 4),

cs_UplinkTimeAlignment_Synch_Auto_Def,    //@sic R5s110618 sic@

cs_UL_GrantConfig_ContPeriodic(5)); }

…

## 2.2 Change 2

| Function names | function f_AckNack_TimingInfo, function f_GetSubFrameNumForULHARQ |
| --- | --- |
| Reason for change | CAT-M1 Status PDU is expected after 6 subframes as 2 subframe difference is the schedule wait between DCI and DL-SCH (Ref 36.213 cl. 7.1.11 PDSCH subframe assignment for BL/CE UE) |
| Summary of change | Modified HARQ ack/nack timing to 6 subframes for cat-M |
| TTCN module | Common\EUTRA\ EUTRA_MAC_Common.ttcn |
| MCC160 Comment |  |

Before change:

....

function f_AckNack_TimingInfo(SubFrameTiming_Type p_Timing,

EUTRA_FDD_TDD_Mode_Type p_EUTRA_FDD_TDD_Mode) return SubFrameTiming_Type

{

var integer v_SubframeNumber := f_GetSubFrameNumForULHARQ(p_Timing.Subframe.Number, p_EUTRA_FDD_TDD_Mode);

var integer v_FrameNumber := p_Timing.SFN.Number;

var integer v_HSFN := p_Timing.HSFN.Number; //@sic R5s201260 sic@

if (v_SubframeNumber  < 4) {

v_HSFN := (p_Timing.HSFN.Number + (v_FrameNumber + 1)/1024) mod 1024

v_FrameNumber := (v_FrameNumber + 1) mod 1024;

}

return valueof(cs_SubFrameTiming(v_FrameNumber, v_SubframeNumber, v_HSFN));

}

//----------------------------------------------------------------------------

/*

* @desc      Get the subframe number for UE to send ACK/NACK.

* @param     p_SubFrameNum

* @param     p_EUTRA_FDD_TDD  .. mode: FDD or TDD

* @return    integer

* @status    APPROVED (LTE, LTE_A_PRO)

*/

function f_GetSubFrameNumForULHARQ(integer p_SubFrameNum,

EUTRA_FDD_TDD_Mode_Type p_EUTRA_FDD_TDD) return integer

{

var integer v_SubFrameNum := p_SubFrameNum;

if (p_EUTRA_FDD_TDD == FDD)

{

v_SubFrameNum := (v_SubFrameNum+ 4)mod 10;

}

else if (p_EUTRA_FDD_TDD == TDD) // caters only for default UL DL Mode of 1.

{

select (p_SubFrameNum)

{

case (0) {

v_SubFrameNum := 7;

}

case (1) {

v_SubFrameNum := 7;

}

case (4) {

v_SubFrameNum := 8;

}

case (5) {

v_SubFrameNum := 2;

}

case (6) {

v_SubFrameNum := 2;

}

case (9) {

v_SubFrameNum := 3;

}

case else {

FatalError(__FILE__, __LINE__, "Incorrect DL subframe number.");

}

}

}

return v_SubFrameNum;

};

…..

After change:

function f_AckNack_TimingInfo(SubFrameTiming_Type p_Timing,

EUTRA_FDD_TDD_Mode_Type p_EUTRA_FDD_TDD_Mode) return SubFrameTiming_Type

{

var integer v_SubframeNumber := f_GetSubFrameNumForULHARQ(p_Timing.Subframe.Number, p_EUTRA_FDD_TDD_Mode);

var integer v_FrameNumber := p_Timing.SFN.Number;

var integer v_HSFN := p_Timing.HSFN.Number; //@sic R5s201260 sic@

if (pc_ue_CategoryDL_M1){

if (v_SubframeNumber  < 6) {    //anritsu 7.1.6.5 HARQ timing change 1

v_HSFN := (p_Timing.HSFN.Number + (v_FrameNumber + 1)/1024) mod 1024;

v_FrameNumber := (v_FrameNumber + 1) mod 1024;

}}

else{

if (v_SubframeNumber  < 4) {

v_HSFN := (p_Timing.HSFN.Number + (v_FrameNumber + 1)/1024) mod 1024;

v_FrameNumber := (v_FrameNumber + 1) mod 1024;

}

}

return valueof(cs_SubFrameTiming(v_FrameNumber, v_SubframeNumber, v_HSFN));

}

//----------------------------------------------------------------------------

/*

* @desc      Get the subframe number for UE to send ACK/NACK.

* @param     p_SubFrameNum

* @param     p_EUTRA_FDD_TDD  .. mode: FDD or TDD

* @return    integer

* @status    APPROVED (LTE, LTE_A_PRO)

*/

function f_GetSubFrameNumForULHARQ(integer p_SubFrameNum,

EUTRA_FDD_TDD_Mode_Type p_EUTRA_FDD_TDD) return integer

{

var integer v_SubFrameNum := p_SubFrameNum;

if (p_EUTRA_FDD_TDD == FDD)

{

if (pc_ue_CategoryDL_M1)

{

v_SubFrameNum := (v_SubFrameNum+ 6)mod 10;    //anritsu 7.1.6.5 change 2

}

else

{

v_SubFrameNum := (v_SubFrameNum+ 4)mod 10;

}

}

else if (p_EUTRA_FDD_TDD == TDD) // caters only for default UL DL Mode of 1.

{

select (p_SubFrameNum)

{

case (0) {

v_SubFrameNum := 7;

}

case (1) {

v_SubFrameNum := 7;

}

case (4) {

v_SubFrameNum := 8;

}

case (5) {

v_SubFrameNum := 2;

}

case (6) {

v_SubFrameNum := 2;

}

case (9) {

v_SubFrameNum := 3;

}

case else {

FatalError(__FILE__, __LINE__, "Incorrect DL subframe number.");

}

}

}

return v_SubFrameNum;

};

……

## 2.3 Change 3

| Function name | function f_TC_7_1_6_Steps() |
| --- | --- |
| Reason for change | PUSCH(BSR) is expected after 4 subframes from Grant Schedule but in cat-m1 case ‘f_AckNack_TimingInfo’ adds 6 subframes to timing as part of change 2. Therefore, 2 subframes are subtracted at reception of BSR |
| Summary of change | PUSCH(BSR) Indication received at 4 subframes instead of 6 |
| TTCN module | Common\EUTRA\ EUTRA_MAC_Common.ttcn |
| MCC160 Comment |  |

Before change:

....

function f_TC_7_1_6_Steps(integer p_LongDRXCycle, //@sic R5s170931 sic@

float p_WatchDog := 5.0)runs on EUTRA_PTC

{

….

//@siclog "Step 15" siclog@

// Config SS to report Reception of Buffer status report

// f_SS_ConfigBufferStatusReportIndMode(eutra_Cell1,tsc_L1Mac_IndicationMode_Enable);

//@sic R5s120634 sic@

if (v_EUTRA_FDD_TDD_Mode == FDD) {

v_Timing1 := f_GetNextSendOccasion_7_1_6_X(eutra_Cell1, v_EUTRA_FDD_TDD_Mode, v_LongDRXCycle, tsc_DrxStartOffset, tsc_OnDurationTimer - 1);

} else { // if TDD

v_Timing1 := f_GetNextSendOccasion_7_1_6_X(eutra_Cell1, v_EUTRA_FDD_TDD_Mode, v_LongDRXCycle, tsc_DrxStartOffset, tsc_OnDurationTimer - 2);

}

f_SS_ConfigBufferStatusReportIndMode(eutra_Cell1, tsc_L1Mac_IndicationMode_Enable, cs_TimingInfo_EUTRA_NB(v_Timing1));

v_Timing1.Subframe.Number := f_GetSubframeForULGrant(v_Timing1.Subframe.Number, v_EUTRA_FDD_TDD_Mode); //@sic R5s120352 sic@

// UL Grant of 16 bits

//f_EUTRA_OneULGrantTransmission(eutra_Cell1, cs_TimingInfo_EUTRA_NB(v_Timing1.SFN.Number, v_Timing1.Subframe.Number), 0, 1);

f_EUTRA_ULGrantConfiguration_Common(eutra_Cell1,

cs_TimingInfo_EUTRA_NB(v_Timing1),

cs_DciInfo_CcchDcchDtchUL_Explicit(0, 1),

cs_UplinkTimeAlignment_Synch_None,

cs_UL_GrantConfig_OneTime);

//@siclog "Step 17" siclog@

select (v_EUTRA_FDD_TDD_Mode) {

case (FDD) {

v_SubframeOffset := 8 * (tsc_MaxHARQTx - 1);

}

case (TDD) {

v_SubframeOffset := 10 * (tsc_MaxHARQTx - 1);

}

}

v_Timing2 := f_EUTRA_TimingInfoAdd(v_Timing1, v_SubframeOffset);

v_NormalSF := f_DRX_NormalSF(v_Timing2.Subframe.Number, 0, v_EUTRA_FDD_TDD_Mode);      //@sic R5s120352 sic@

v_Timing2 := f_EUTRA_TimingInfoAdd(v_Timing2, v_NormalSF);

v_EncodedRlcPdu := f_RLC_AMD_1PDU_Encvalue(5, tsc_NoPoll, f_PDCP_PDU_Encvalue(cs_PDCP_PDU_LongSN(cs_PDCP_DataPdu_LongSN(tsc_B12_SQN5, crs_PDCP_SDU_17B))));//@sic R5s110552 sic@

DRB.send(cas_DRB_COMMOM_REQ_MAC_PDU(eutra_Cell1,

tsc_RbId_DRB1,

cs_TimingInfo_EUTRA_NB(v_Timing2),

cs_MAC_PDUList_1(cs_MAC_PDU_NoPadding_RLCPDU_LCID(tsc_LchId_DRB1, v_EncodedRlcPdu)),

cs_HarqProcessAssignment_Automatic));

// Preconfig SS for preventing UE to enter DRX

// @sic R5s100520 sic@

v_Timing3 := f_EUTRA_TimingInfoAdd(v_Timing2, 5);

// @sic R5s100520 to pre-configure SS to stop report reception of BSR 2 subframes after BSR is received sic@

f_SS_ConfigBufferStatusReportIndMode(eutra_Cell1, tsc_L1Mac_IndicationMode_Disable, cs_TimingInfo_EUTRA_NB(v_Timing3));

t_Watchdog.start;

//@siclog "Step 16" siclog@

// Check: Does the UE transmit a Buffer Status Report on the UL indicating an empty buffer?

// Get the time when UE sent BSR

// Note the function not suitable for finding the UL Data timing with respect to Grant, but still result in same value

v_Timing1 := f_AckNack_TimingInfo(v_Timing1, v_EUTRA_FDD_TDD_Mode);

SYSIND.receive(car_BSR_IND(eutra_Cell1,

cr_TimingInfo(v_Timing1.SFN.Number, v_Timing1.Subframe.Number),

cr_ShortBSR_Ind(?, ?)));

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 16");

t_Watchdog.stop;

//@siclog "Step 18" siclog@

// Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 17?

v_Timing2 := f_AckNack_TimingInfo(v_Timing2, v_EUTRA_FDD_TDD_Mode);

t_Watchdog.start;

SYSIND.receive(car_UL_HARQ_IND(eutra_Cell1,

cr_TimingInfo(v_Timing2.SFN.Number, v_Timing2.Subframe.Number), ack));

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 18");

t_Watchdog.stop;

}

} //end of group DRX_Functions

…

After change:

....

function f_TC_7_1_6_Steps(integer p_LongDRXCycle, //@sic R5s170931 sic@

float p_WatchDog := 5.0)runs on EUTRA_PTC

{

….

//@siclog "Step 15" siclog@

// Config SS to report Reception of Buffer status report

// f_SS_ConfigBufferStatusReportIndMode(eutra_Cell1,tsc_L1Mac_IndicationMode_Enable);

//@sic R5s120634 sic@

if (v_EUTRA_FDD_TDD_Mode == FDD) {

v_Timing1 := f_GetNextSendOccasion_7_1_6_X(eutra_Cell1, v_EUTRA_FDD_TDD_Mode, v_LongDRXCycle, tsc_DrxStartOffset, tsc_OnDurationTimer - 1);

} else { // if TDD

v_Timing1 := f_GetNextSendOccasion_7_1_6_X(eutra_Cell1, v_EUTRA_FDD_TDD_Mode, v_LongDRXCycle, tsc_DrxStartOffset, tsc_OnDurationTimer - 2);

}

f_SS_ConfigBufferStatusReportIndMode(eutra_Cell1, tsc_L1Mac_IndicationMode_Enable, cs_TimingInfo_EUTRA_NB(v_Timing1));

v_Timing1.Subframe.Number := f_GetSubframeForULGrant(v_Timing1.Subframe.Number, v_EUTRA_FDD_TDD_Mode); //@sic R5s120352 sic@

// UL Grant of 16 bits

//f_EUTRA_OneULGrantTransmission(eutra_Cell1, cs_TimingInfo_EUTRA_NB(v_Timing1.SFN.Number, v_Timing1.Subframe.Number), 0, 1);

f_EUTRA_ULGrantConfiguration_Common(eutra_Cell1,

cs_TimingInfo_EUTRA_NB(v_Timing1),

cs_DciInfo_CcchDcchDtchUL_Explicit(0, 1),

cs_UplinkTimeAlignment_Synch_None,

cs_UL_GrantConfig_OneTime);

//@siclog "Step 17" siclog@

select (v_EUTRA_FDD_TDD_Mode) {

case (FDD) {

v_SubframeOffset := 8 * (tsc_MaxHARQTx - 1);

}

case (TDD) {

v_SubframeOffset := 10 * (tsc_MaxHARQTx - 1);

}

}

v_Timing2 := f_EUTRA_TimingInfoAdd(v_Timing1, v_SubframeOffset);

v_NormalSF := f_DRX_NormalSF(v_Timing2.Subframe.Number, 0, v_EUTRA_FDD_TDD_Mode);      //@sic R5s120352 sic@

v_Timing2 := f_EUTRA_TimingInfoAdd(v_Timing2, v_NormalSF);

v_EncodedRlcPdu := f_RLC_AMD_1PDU_Encvalue(5, tsc_NoPoll, f_PDCP_PDU_Encvalue(cs_PDCP_PDU_LongSN(cs_PDCP_DataPdu_LongSN(tsc_B12_SQN5, crs_PDCP_SDU_17B))));//@sic R5s110552 sic@

DRB.send(cas_DRB_COMMOM_REQ_MAC_PDU(eutra_Cell1,

tsc_RbId_DRB1,

cs_TimingInfo_EUTRA_NB(v_Timing2),

cs_MAC_PDUList_1(cs_MAC_PDU_NoPadding_RLCPDU_LCID(tsc_LchId_DRB1, v_EncodedRlcPdu)),

cs_HarqProcessAssignment_Automatic));

// Preconfig SS for preventing UE to enter DRX

// @sic R5s100520 sic@

v_Timing3 := f_EUTRA_TimingInfoAdd(v_Timing2, 5);

// @sic R5s100520 to pre-configure SS to stop report reception of BSR 2 subframes after BSR is received sic@

f_SS_ConfigBufferStatusReportIndMode(eutra_Cell1, tsc_L1Mac_IndicationMode_Disable, cs_TimingInfo_EUTRA_NB(v_Timing3));

t_Watchdog.start;

//@siclog "Step 16" siclog@

// Check: Does the UE transmit a Buffer Status Report on the UL indicating an empty buffer?

// Get the time when UE sent BSR

// Note the function not suitable for finding the UL Data timing with respect to Grant, but still result in same value

v_Timing1 := f_AckNack_TimingInfo(v_Timing1, v_EUTRA_FDD_TDD_Mode);

if (pc_ue_CategoryDL_M1)

{

SYSIND.receive(car_BSR_IND(eutra_Cell1,

cr_TimingInfo(v_Timing1.SFN.Number, v_Timing1.Subframe.Number-2),//Anritsu 7.1.6.5 change 3

cr_ShortBSR_Ind(?, ?)));

}

else

{

SYSIND.receive(car_BSR_IND(eutra_Cell1,

cr_TimingInfo(v_Timing1.SFN.Number, v_Timing1.Subframe.Number),

cr_ShortBSR_Ind(?, ?)));

}

//@siclog "Step 18" siclog@

// Check: Does the UE transmit a HARQ ACK for the DL MAC PDU in Step 17?

v_Timing2 := f_AckNack_TimingInfo(v_Timing2, v_EUTRA_FDD_TDD_Mode);

t_Watchdog.start;

SYSIND.receive(car_UL_HARQ_IND(eutra_Cell1,

cr_TimingInfo(v_Timing2.SFN.Number, v_Timing2.Subframe.Number), ack));

f_EUTRA_PreliminaryPass(__FILE__, __LINE__, "Step 18");

t_Watchdog.stop;

}

} //end of group DRX_Functions

…