3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210233

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4568 | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE PLMN selection testcase 6.1.1.1b |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2020-12-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As in R5s201565, the t_GuardTimer for TC 6.1.1.1b should also be updated to 17000 seconds to comply with 36.523-1 Table 6.1.1.1b.3.2-2 (Step 20 & 32). The t_GuardTimer of 9600s is not enough for PLMN selection in steps 20 & 32 for conformant CAT-M1 UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changing t_GuardTimer from 9600 to 17100. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.1b |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2019-06_D20wk38 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

| Function name | testcase TC_6_1_1_1b() |
| --- | --- |
| Reason for change | As in R5s201565, the t_GuardTimer for TC 6.1.1.1b should also be updated to 17000 seconds to comply with 36.523-1 Table 6.1.1.1b.3.2-2 (Step 20 & 32). The t_GuardTimer of 9600s is not enough for PLMN selection in steps 20 & 32 for conformant CAT-M1 UE. |
| Summary of change | Changing t_GuardTimer from 9600 to 17100. |
| TTCN module | LTE_Testsuite.ttcn |
| MCC160 Comment |  |

Before Change

testcase TC_6_1_1_1b() runs on MTC_LTE system SYSTEM_LTE {

// @purpose

//   PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode / Single Frequency operation

var EUTRA_PTC        v_EUTRA      := null;

var IMS_PTC          v_IMS1       := null;

var IMS_PTC          v_IMS2       := null;

timer t_GuardTimer := int2float(9600);

v_EUTRA := EUTRA_PTC.create alive;

f_MTC_ConnectPTCs_LTE(system, v_EUTRA, v_IMS1, v_IMS2);

v_EUTRA.start(f_TC_6_1_1_1b_EUTRA());

t_GuardTimer.start;

f_MTC_MainLoop_LTE(t_GuardTimer);

}

After change

testcase TC_6_1_1_1b() runs on MTC_LTE system SYSTEM_LTE {

// @purpose

//   PLMN selection of RPLMN, HPLMN/EHPLMN, UPLMN and OPLMN / Automatic mode / Single Frequency operation

var EUTRA_PTC        v_EUTRA      := null;

var IMS_PTC          v_IMS1       := null;

var IMS_PTC          v_IMS2       := null;

timer t_GuardTimer := int2float(17100);

v_EUTRA := EUTRA_PTC.create alive;

f_MTC_ConnectPTCs_LTE(system, v_EUTRA, v_IMS1, v_IMS2);

v_EUTRA.start(f_TC_6_1_1_1b_EUTRA());

t_GuardTimer.start;

f_MTC_MainLoop_LTE(t_GuardTimer);

}