3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210235

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1581 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function fl_TC_9_1_1_3_6_Body |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation of this function, a local default is activated for testcase 9.1.1.3 at Step 10 by calling : v_MyDefaultVar := activate (a_IgnoreAuthenticationFailure (v_CellId)); This is then deactivated after the for loop, just after Step 15 for testcase 9_1_1_3 and after Step 10 for 9_1_1_6 : deactivate (v_MyDefaultVar); // @sic R5-202655 sic@ This is a problem. The TTCN tries to deactivate a default which was never started for testcase 9_1_1_6. This causes a run-time error Another issue is also observed during execution of 9_1_1_3 . After Step 15 the UE is expected to Register on Cell B. The UE correctly camps on it as well, but the rrcSetupComplete with piggybacked Registration REQ is captured in the local default that is started earlier in the testcase and is still operational. This also needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Implemented an alternative solution to both the issues mentioned above. Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The testcase fail because of a run-time error |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.1.3.NR5GC, 9.1.1.6.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required	4

1.1	function fl_TC_9_1_1_3_6_Body	4

# Corrections required

## function fl_TC_9_1_1_3_6_Body

| Function name | fl_TC_9_1_1_3_6_Body |
| --- | --- |
| Reason for change | In the current TTCN implementation of this function, a local default is activated for testcase 9.1.1.3 at Step 10 by calling : v_MyDefaultVar := activate (a_IgnoreAuthenticationFailure (v_CellId)); This is then deactivated after the for loop, just after Step 15 for testcase 9_1_1_3 and after Step 10 for 9_1_1_6 : deactivate (v_MyDefaultVar); // @sic R5-202655 sic@ This is a problem. The TTCN tries to deactivate a default which was never started for testcase 9_1_1_6. This causes a run-time error Another issue is also observed during execution of 9_1_1_3 . After Step 15 the UE is expected to Register on Cell B. The UE correctly camps on it as well, but the rrcSetupComplete with piggybacked Registration REQ is captured in the local default that is started earlier in the testcase and is still operational. This also needs to be corrected. |
| Summary of change | Implemented an alternative solution to both the issues mentioned above. Please see below. |
| TTCN module | ATS_NR5GC/9_1_1/Authentication_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> v_MsgToSend.authentication_Request.ngNasKeySetId.nasKeySetId := v_KSI_InUse; for (i := 0; i < 3; i := i+1) { if(p_UseEAP){ //@siclog "TC 9_1_1_3 Step 6/8/10" siclog@ }else{ //@siclog "TC 9_1_1_6 Step 2/4/6" siclog@ } if ((i == 2)){ // @sic R5-200616 sic@ if(p_UseEAP){ //@siclog "TC 9_1_1_3 Step 10" siclog@ v_MyDefaultVar := activate (a_IgnoreAuthenticationFailure (v_CellId)); // @sic R5-202655 sic@ f_NR_SetCellPower (v_NGC_NASCellB, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR2); } else { // @sic R5-201084 sic@ //@siclog "TC 9_1_1_6 Step 5" siclog@ } } SRB.send(cas_NR_SRB_NasPdu_REQ(v_CellId, // @sic R5-199098 sic@ tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(v_SecurityHeader, v_MsgToSend))); if (i == 2) { // don't wait for 3rd failure msg break; } if(p_UseEAP){ //@siclog "Step 7/9" siclog@ // This is for testcase 9_1_1_3 } else { //@siclog "Step 2/4" siclog@ // This is for testcase 9_1_1_6 } SRB.receive(car_NR_SRB_NasPdu_IND(v_CellId, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(v_SecurityHeader))) -> value v_ReceivedAsp; // @sic R5-199098 sic@ if (match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_NG_AUTHENTICATION_FAILURE(cr_GMM_GSM_Cause(omit, '01000111'B)))) { // ngKSI already in use f_NR_PreliminaryPass(__FILE__, __LINE__, "Step "&int2str((2*i)+7)); } } if(p_UseEAP){ //@siclog "Step 12 - 15" siclog@ // This is for 9_1_1_3 } else { // @sic R5-201084 sic@ //@siclog "Step 6A" siclog@ // This is for testcase 9_1_1_6 f_NR_CheckNoRRCSetupReq (v_NGC_NASCellA, 30.0); //@siclog "Step 6B" siclog@ f_NR_SetCellPower (v_NGC_NASCellB, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR2); //@siclog "Step 7 - 10" siclog@ //This is for Testcase 9_1_1_6 // @sic R5-199098 sic@ } v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellB, tsc_NR_RRC_TI_Def, ?); deactivate (v_MyDefaultVar); // @sic R5-202655 sic@ if (f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { // @sic R5s200237 sic@ if(p_UseEAP){ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); } } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> // now update with "old" KSI value v_MsgToSend.authentication_Request.ngNasKeySetId.nasKeySetId := v_KSI_InUse; for (i := 0; i < 3; i := i+1) { if(p_UseEAP){ //@siclog "TC 9_1_1_3 Step 6/8/10" siclog@ }else{ //@siclog "TC 9_1_1_6 Step 2/4/6" siclog@ } if ((i == 2)){ // @sic R5-200616 sic@ if(p_UseEAP){ //@siclog "TC 9_1_1_3 Step 10" siclog@ v_MyDefaultVar := activate (a_IgnoreAuthenticationFailure (v_CellId)); // @sic R5-202655 sic@ f_NR_SetCellPower (v_NGC_NASCellB, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR2); } else { // @sic R5-201084 sic@ //@siclog "TC 9_1_1_6 Step 5" siclog@ } } SRB.send(cas_NR_SRB_NasPdu_REQ(v_CellId, // @sic R5-199098 sic@ tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(v_SecurityHeader, v_MsgToSend))); if (i == 2) { // don't wait for 3rd failure msg break; } if(p_UseEAP){ //@siclog "Step 7/9" siclog@ // This is for testcase 9_1_1_3 } else { //@siclog "Step 2/4" siclog@ // This is for testcase 9_1_1_6 } SRB.receive(car_NR_SRB_NasPdu_IND(v_CellId, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(v_SecurityHeader))) -> value v_ReceivedAsp; // @sic R5-199098 sic@ if (match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_NG_AUTHENTICATION_FAILURE(cr_GMM_GSM_Cause(omit, '01000111'B)))) { // ngKSI already in use f_NR_PreliminaryPass(__FILE__, __LINE__, "Step "&int2str((2*i)+7)); } } if(p_UseEAP){ //@siclog "Step 12 - 15" siclog@ // This is for 9_1_1_3 } else { // @sic R5-201084 sic@ //@siclog "Step 6A" siclog@ // This is for testcase 9_1_1_6 f_NR_CheckNoRRCSetupReq (v_NGC_NASCellA, 30.0); //@siclog "Step 6B" siclog@ f_NR_SetCellPower (v_NGC_NASCellB, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); f_NR_SetCellPower (v_NGC_NASCellA, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR1, tsc_NR_SuitableIntraFreqCellSSS_EPRE_FR2); //@siclog "Step 7 - 10" siclog@ //This is for Testcase 9_1_1_6 // @sic R5-199098 sic@ } //WA#9_1_1_3 change Start : //v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (v_NGC_NASCellB, tsc_NR_RRC_TI_Def, ?); f_NR_RRC_SetupRequest_Def(v_NGC_NASCellB, ?, ?); if(p_UseEAP){ deactivate (v_MyDefaultVar); // @sic R5-202655 sic@ } f_NR_RRCSetup_Def (v_NGC_NASCellB); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(v_NGC_NASCellB, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?,- , *), tsc_SHT_IntegrityProtected); //deactivate (v_MyDefaultVar); // @sic R5-202655 sic@ //WA#9_1_1_3 Change stop if (f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { // @sic R5s200237 sic@ if(p_UseEAP){ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); } else { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 10"); } } <<SKIPPED CODE>>