3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210239

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1584 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC test case 10.1.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | If pc_noOf_PDUsNewConnection >0 then in Preamble part UE uses DNN “ABCC” for Registration. But in TC 10.1.1.2, 2nd PDU is also configured with DNN “ABCC”. As UE can’t activate two PDUs with same DNN TC is failing in step 0A. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Instead of DNN “ABCC” use DNN "APN-1" for 2nd PDU. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testcase will fail a conformant UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2019-06_D20wk50 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | fl_TC_10_1_1_2_Body() |
| --- | --- |
| Reason for change | If pc_noOf_PDUsNewConnection >0 then in Preamble part UE uses DNN “ABCC” for Registration. But in TC 10.1.1.2, 2nd PDU is also configured with DNN “ABCC”. As UE can’t activate two PDUs with same DNN TC is failing in step 0A. |
| Summary of change | Instead of DNN “ABCC” use DNN "APN-1" for 2nd PDU. |
| TTCN module | GSM_TestCases_NR5GC.ttcn |
| MCC160 Comment |  |

Before Change:

function fl_TC_10_1_1_2_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)); var PDUSessionInfoList_Type v_PDUSessionInfo; var O1_Type v_PDUSessionIdToRelease; var NG_NAS_SecurityParams_Type v_SecurityParams := f_NR5GC_Security_Get(); var PDUSessionInfoList_Type v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var template (value) NG_NAS_DL_Pdu_Type v_MsgToSend; var template DNN v_DNN := cs_DNN(-, f_DomainName_Encode(tsc_RandomAPN)); //Anritsu TTCN CR R5s210148 v_GSM_MobilityInfo := v_PDUSessionInfoList[0]; //@siclog "Steps 0A - 0B" siclog@ f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_RandomAPN); // @sic R5-203785, R5s201230 sic@ f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1, -, -, -, v_DNN, -, v_DNN); // @sic R5-203785 sic@ //Anritsu TTCN CR R5s210148 v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); v_GSM_MobilityInfo := v_PDUSessionInfoList[1]; //@siclog "Steps 1 - 2" siclog@ //Send PDU Session Authentication Command and receive PDU Session Authentication Complete f_PDUSessionAuthentication( v_NGC_NASCellA, v_SecurityParams, v_GSM_MobilityInfo, v_PLMN, tsc_NR_RbId_SRB2); // @sic R5s20956 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); //@siclog "Step 3" siclog@ @sic R5-200214 sic@ //SS transmits PDU Session Authentication result v_MsgToSend := f_Get_PDUSessionAuthenticationResult(v_GSM_MobilityInfo, cs_EAP_Message(bit2oct(encvalue(cs_EAP_Success)))); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, - , -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Steps 4 - 5" siclog@ //Send PDU Session Authentication Command and receive PDU Session Authentication Complete f_PDUSessionAuthentication( v_NGC_NASCellA, v_SecurityParams, v_GSM_MobilityInfo, v_PLMN, tsc_NR_RbId_SRB2); // @sic R5s20956 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); v_PDUSessionInfo := f_NR5GC_MobileInfo_GetSessionInfoList(); v_PDUSessionIdToRelease := v_PDUSessionInfo[f_NR5GC_MobileInfo_GetNoOfSessions()-1].SessionId; //@siclog "Steps 6 - 17" siclog@ //VOIDED // @sic R5-203785 sic@ //@siclog "Steps 18 - 19" siclog@ f_NR5GC_PDUSessionRelease(v_NGC_NASCellA, // @sic R5-206354 sic@ v_PDUSessionIdToRelease, cs_GMM_GSM_Cause (omit, '00011101'B), //5GSM cause User Authentication or Authorization failed cs_GPRS_Timer2_3_IEI('37'O, '101'B, '00000'B), -, -, cs_EAP_Message(bit2oct(encvalue(cs_EAP_Failure)))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 19"); }

After Change:

function fl_TC_10_1_1_2_Body() runs on NR5GC_PTC { var NR_CellId_Type v_NGC_NASCellA := f_NR5GC_MapNASCell(ngc_CellA, NG_AllCellsOnSamePLMN); var NAS_PlmnId v_PLMN := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (v_NGC_NASCellA)); var PDUSessionInfoList_Type v_PDUSessionInfo; var O1_Type v_PDUSessionIdToRelease; var NG_NAS_SecurityParams_Type v_SecurityParams := f_NR5GC_Security_Get(); var PDUSessionInfoList_Type v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var template (value) NG_NAS_DL_Pdu_Type v_MsgToSend; var template DNN v_DNN := cs_DNN(-, f_DomainName_Encode(tsc_APN1_String)); //Anritsu TTCN CR R5s210148 v_GSM_MobilityInfo := v_PDUSessionInfoList[0]; //@siclog "Steps 0A - 0B" siclog@ f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_APN1_String); // @sic R5-203785, R5s201230 sic@ f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, 1, -, -, -, v_DNN, -, v_DNN); // @sic R5-203785 sic@ //Anritsu TTCN CR R5s210148 v_PDUSessionInfoList := f_NR5GC_MobileInfo_GetSessionInfoList(); v_GSM_MobilityInfo := v_PDUSessionInfoList[1]; //@siclog "Steps 1 - 2" siclog@ //Send PDU Session Authentication Command and receive PDU Session Authentication Complete f_PDUSessionAuthentication( v_NGC_NASCellA, v_SecurityParams, v_GSM_MobilityInfo, v_PLMN, tsc_NR_RbId_SRB2); // @sic R5s20956 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); //@siclog "Step 3" siclog@ @sic R5-200214 sic@ //SS transmits PDU Session Authentication result v_MsgToSend := f_Get_PDUSessionAuthenticationResult(v_GSM_MobilityInfo, cs_EAP_Message(bit2oct(encvalue(cs_EAP_Success)))); SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, - , -, cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); //@siclog "Steps 4 - 5" siclog@ //Send PDU Session Authentication Command and receive PDU Session Authentication Complete f_PDUSessionAuthentication( v_NGC_NASCellA, v_SecurityParams, v_GSM_MobilityInfo, v_PLMN, tsc_NR_RbId_SRB2); // @sic R5s20956 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 5"); v_PDUSessionInfo := f_NR5GC_MobileInfo_GetSessionInfoList(); v_PDUSessionIdToRelease := v_PDUSessionInfo[f_NR5GC_MobileInfo_GetNoOfSessions()-1].SessionId; //@siclog "Steps 6 - 17" siclog@ //VOIDED // @sic R5-203785 sic@ //@siclog "Steps 18 - 19" siclog@ f_NR5GC_PDUSessionRelease(v_NGC_NASCellA, // @sic R5-206354 sic@ v_PDUSessionIdToRelease, cs_GMM_GSM_Cause (omit, '00011101'B), //5GSM cause User Authentication or Authorization failed cs_GPRS_Timer2_3_IEI('37'O, '101'B, '00000'B), -, -, cs_EAP_Message(bit2oct(encvalue(cs_EAP_Failure)))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 19"); }