3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210242

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1587 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR common function f_NR_InitDL_BWPs() |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For some NR bands (n28, n39, n40), configuration of DL BWP and LocationAndBandwidth can be incorrect because SSB subcarrier spacing is used which can be different from the specific carrier subcarrier spacing as configured in MIB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Use specific carrier subcarrier spacing for f_NR_InitLocationAndBandwidth() and cs_NR_DownlinkBWP_List_Def instead of SSB subcarrier spacing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | f_NR_CellIfno_Init() |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_NR_InitDL_BWPs |
| --- | --- |
| Reason for change | For some NR bands (n28, n39, n40), configuration of DL BWP and LocationAndBandwidth can be incorrect because SSB subcarrier spacing is used which can be different from the specific carrier subcarrier spacing as configured in MIB. |
| Summary of change | Use specific carrier subcarrier spacing for f_NR_InitLocationAndBandwidth() and cs_NR_DownlinkBWP_List_Def instead of SSB subcarrier spacing. |
| TTCN module | NR_CellInfoInit.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR_InitDL_BWPs(PhysCellId p_PhysCellId, NR_FrequencyBand_Type p_NR_FrequencyBand, SSB_Index p_SSB_Index ) return template (value) NR_DownlinkBWP_List_Type { /* @sic R5-190786: p_PhysCellId sic@ */ var template (value) NR_DownlinkBWP_List_Type v_DL_BWPs; var template (value) DMRS_DownlinkConfig v_DMRS_DownlinkConfig; var template (value) NR_SearchSpace_NrofCandidates_Type v_NrofCandidates; //@sic R5-201655 RP-201138 sic@ var template (value) PDSCH_TimeDomainResourceAllocationList v_PDSCH_TimeDomainResourceAllocationList; var template (omit) PDSCH_Config.mcs_Table v_Mcs_Table := omit; // @sic R5-190670: qam64 per default independent from FR1/FR2 sic@ var BWP.locationAndBandwidth v_LocationAndBandwidth := f_NR_InitLocationAndBandwidth(p_NR_FrequencyBand.FrequencyBandIndicator, p_NR_FrequencyBand.SSB_SubcarrierSpacing, p_NR_FrequencyBand.ChannelBandwidth); //Same SCS used for all channels); if (f_NR_IsBandFR1(p_NR_FrequencyBand.FrequencyBandIndicator)) { //FR1 v_DMRS_DownlinkConfig := cs_38508_DMRS_DownlinkConfig_FR1; v_PDSCH_TimeDomainResourceAllocationList := cs_38508_PDSCH_TimeDomainResourceAllocationList_FR1; v_NrofCandidates := cs_38508_NrofCandidates_Def; } else { //FR2 v_DMRS_DownlinkConfig := cs_38508_DMRS_DownlinkConfig_FR2; v_PDSCH_TimeDomainResourceAllocationList := cs_38508_PDSCH_TimeDomainResourceAllocationList_FR2; v_NrofCandidates := cds_38508_NrofCandidates_FR2 //@sic R5s200898 RP-201138 sic@ } v_DL_BWPs := cs_NR_DownlinkBWP_List_Def(p_NR_FrequencyBand.SSB_SubcarrierSpacing, p_PhysCellId, v_LocationAndBandwidth, v_DMRS_DownlinkConfig, v_NrofCandidates, v_PDSCH_TimeDomainResourceAllocationList, v_Mcs_Table, p_NR_FrequencyBand.Coreset0_OffsetRBs, p_SSB_Index ); return v_DL_BWPs; }

After Change:

function f_NR_InitDL_BWPs(PhysCellId p_PhysCellId, NR_FrequencyBand_Type p_NR_FrequencyBand, SSB_Index p_SSB_Index ) return template (value) NR_DownlinkBWP_List_Type { /* @sic R5-190786: p_PhysCellId sic@ */ var template (value) NR_DownlinkBWP_List_Type v_DL_BWPs; var template (value) DMRS_DownlinkConfig v_DMRS_DownlinkConfig; var template (value) NR_SearchSpace_NrofCandidates_Type v_NrofCandidates; //@sic R5-201655 RP-201138 sic@ var template (value) PDSCH_TimeDomainResourceAllocationList v_PDSCH_TimeDomainResourceAllocationList; var template (omit) PDSCH_Config.mcs_Table v_Mcs_Table := omit; // @sic R5-190670: qam64 per default independent from FR1/FR2 sic@ var BWP.locationAndBandwidth v_LocationAndBandwidth := f_NR_InitLocationAndBandwidth(p_NR_FrequencyBand.FrequencyBandIndicator, p_NR_FrequencyBand.FrequencyInfoDL.scs_SpecificCarrierList[0].subcarrierSpacing, p_NR_FrequencyBand.ChannelBandwidth); //Same SCS used for all channels); if (f_NR_IsBandFR1(p_NR_FrequencyBand.FrequencyBandIndicator)) { //FR1 v_DMRS_DownlinkConfig := cs_38508_DMRS_DownlinkConfig_FR1; v_PDSCH_TimeDomainResourceAllocationList := cs_38508_PDSCH_TimeDomainResourceAllocationList_FR1; v_NrofCandidates := cs_38508_NrofCandidates_Def; } else { //FR2 v_DMRS_DownlinkConfig := cs_38508_DMRS_DownlinkConfig_FR2; v_PDSCH_TimeDomainResourceAllocationList := cs_38508_PDSCH_TimeDomainResourceAllocationList_FR2; v_NrofCandidates := cds_38508_NrofCandidates_FR2 //@sic R5s200898 RP-201138 sic@ } v_DL_BWPs := cs_NR_DownlinkBWP_List_Def(p_NR_FrequencyBand.FrequencyInfoDL.scs_SpecificCarrierList[0].subcarrierSpacing, p_PhysCellId, v_LocationAndBandwidth, v_DMRS_DownlinkConfig, v_NrofCandidates, v_PDSCH_TimeDomainResourceAllocationList, v_Mcs_Table, p_NR_FrequencyBand.Coreset0_OffsetRBs, p_SSB_Index ); return v_DL_BWPs; }