3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210247

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0658 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS 5GS test case 6.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK, Anritsu Lts, Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-12-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add IMS 5GS test case 6.7 to the IMS/NR5GC ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to IMS 5GS test case 6.7 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

3	Branches Executed	6

4	Execution Log Files	6

4.1	Qualcomm SM8350	6

4.2	HiSilicon Balong 5000	6

5	References	7

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 6.7 which is part of the IMS NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

Narendra Kalajasti

Narendra.kalahasti@anritsu.com

Hans Rohnert

Hans.rohnert@rohde-schwarz.com

## Verification Test Summary

Test Case: 	6.7.NR5GC

ATS Version:	iwd-TTCN3-B2019-06_D20wk38

System Simulator used:	Keysight S8704A Protocol Conformance Toolset, Anritsu Protocol Conformance Test System ME7834NR and Rohde & Schwarz 5G Protocol Conformance Test platform

UE used:	Qualcomm SM8350 and HiSilicon Balong 5000

Verification Status:	PASS

# Corrections required

## 2.1 Change 1

| Function name | f_TC_6_7_NR5GC_IMS1 |
| --- | --- |
| Reason for change | Following CT1 response to LS R5-205041 (C1-206661), test case is changed at RAN5#89 |
| Summary of change | Removing TP3 and the modified corresponding test steps 7-8 to not send an invalid MAC for the 2nd register. Added steps to complete the IMS registration procedure Note : Changes are corresponding to R5-206386 agreed at RAN5#89 |
| TTCN module | IMS_NR5GC_RegistrationTestcases.ttcn |
| MCC160 Comment |  |

Before Change:

function f_TC_6_7_NR5GC_IMS1() runs on IMS_PTC

{

var charstring v_InvalidNonce := f_IMS_Authentication_GenerateInvalidNonce(macError);    // Nonce with invalid MAC

var IMS_DATA_REQ v_IMS_REGISTER_REQ;

var template (value) IMS_RoutingInfo_Type v_RoutingInfoDL;

var REGISTER_Request v_RegisterReq;

var charstring v_CallId;

var integer i;

var SecurityClientParamsList_Type v_SecurityClientParamsList;

f_IMS_CC_Preamble(IPCAN_SignallingOnly, IMS_NULL, IMS_Security);

f_IMS_TestBody_Set(true);

// @siclog "Step 1" siclog@

f_IMS_CC_StartSignalling(IPCAN_InitialRegistration);

// @siclog "Step 2" siclog@

v_IMS_REGISTER_REQ := f_IMS_REGISTER_InitialRequest();

v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ.RoutingInfo);

v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register;

v_CallId := v_RegisterReq.msgHeader.callId.callid;

v_SecurityClientParamsList[0] := f_IMS_Register_GetSecurityClientParams(v_RegisterReq);

for (i:=0; i<3; i:=i+1) {

f_IMS_Register_SecurityInit(v_RegisterReq, -, false);

// @siclog "Step 3/5/7" siclog@

IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine401, f_IMS_RegisterResponse_401_MessageHeaderTX(v_RegisterReq, v_InvalidNonce))));

if (i < 2) {

// @siclog "Step 4/6" siclog@

v_IMS_REGISTER_REQ := f_IMS_REGISTER_RequestWithSpecificAuth(v_CallId, cr_Authorization_InvalidMAC);

v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register;

v_SecurityClientParamsList := f_IMS_Authentication_CheckSecurityClientParams(v_SecurityClientParamsList, v_RegisterReq);

f_IMS_Register_CheckAutsNotPresent(v_RegisterReq);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step " & int2str(4 + 2*i));

}

}

// @siclog "Step 8"

f_IMS_CheckUnexpectedRegisterRequest(10.0);

f_IMS_TestBody_Set(false);

f_IMS_CC_Postamble(IPCAN_InitialRegistration);

}

After Change:

function f_TC_6_7_NR5GC_IMS1() runs on IMS_PTC

{

var charstring v_InvalidNonce := f_IMS_Authentication_GenerateInvalidNonce(macError);    // Nonce with invalid MAC

var IMS_DATA_REQ v_IMS_REGISTER_REQ;

var template (value) IMS_RoutingInfo_Type v_RoutingInfoDL;

var REGISTER_Request v_RegisterReq;

var charstring v_CallId;

var integer i;

var SecurityClientParamsList_Type v_SecurityClientParamsList;

f_IMS_CC_Preamble(IPCAN_SignallingOnly, IMS_NULL, IMS_Security);

f_IMS_TestBody_Set(true);

// @siclog "Step 1" siclog@

f_IMS_CC_StartSignalling(IPCAN_InitialRegistration);

// @siclog "Step 2" siclog@

v_IMS_REGISTER_REQ := f_IMS_REGISTER_InitialRequest();

v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ.RoutingInfo);

v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register;

v_CallId := v_RegisterReq.msgHeader.callId.callid;

v_SecurityClientParamsList[0] := f_IMS_Register_GetSecurityClientParams(v_RegisterReq);

for (i:=0; i<2; i:=i+1) {

f_IMS_Register_SecurityInit(v_RegisterReq, -, false);

// @siclog "Step 3/5" siclog@

IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine401, f_IMS_RegisterResponse_401_MessageHeaderTX(v_RegisterReq, v_InvalidNonce))));  // 6.7 - Change 1

//REMOVED if (i < 2) {

// @siclog "Step 4/6" siclog@

v_IMS_REGISTER_REQ := f_IMS_REGISTER_RequestWithSpecificAuth(v_CallId, cr_Authorization_InvalidMAC);

v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register;

v_SecurityClientParamsList := f_IMS_Authentication_CheckSecurityClientParams(v_SecurityClientParamsList, v_RegisterReq);

f_IMS_Register_CheckAutsNotPresent(v_RegisterReq);

f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step " & int2str(4 + 2*i));

//REMOVED }

}

// @siclog "Step 8"

//REMOVED f_IMS_CheckUnexpectedRegisterRequest(10.0);

f_IMS_Register_SecurityInit(v_RegisterReq);

IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine401, f_IMS_RegisterResponse_401_MessageHeaderTX(v_RegisterReq))));

v_IMS_REGISTER_REQ := f_IMS_REGISTER_SubsequentRequest();

v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ.RoutingInfo);

v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register;

IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine200, f_IMS_RegisterResponse_200_MessageHeaderTX(v_RegisterReq))));

f_IMS_Register_SubscribeNotify(v_IMS_REGISTER_REQ, tsc_IMS_PublishDuringSubscribeForRegEvent);

f_IMS_TestBody_Set(false);

f_IMS_CC_Postamble(IPCAN_InitialRegistration);

}

# Branches Executed

The test case was executed on EN-DC band combinations DC_3A_n41 and DC_3A_n78

# Execution Log Files

## Qualcomm SM8350

The Qualcomm SM8350 UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Keysight/TC_6_7_LOG.html

- PICS/PIXIT parameter file:

Keysight/TC_6_7_PIXIT.html

The Qualcomm SM8350 UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu/LOGS.xml

- PICS/PIXIT parameter file:

Anritsu/PICS_PIXIT.xml

The Qualcomm SM8350 UE passed this test case on Rohde & Schwarz 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Rohde&Schwarz/qualcomm/ttcn_tli.log

## HiSilicon Balong 5000

The HiSilicon Balong 5000 UE passed this test case on Rohde & Schwarz 5G Protocol Conformance Test platform. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Rohde&Schwarz/Hisilicon/ttcn_tli.log

# References

| [1] | R5s210005 : Supporting information for addition of IMS 5GS test case 6.7 |
| --- | --- |