3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210253

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1589 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NR5GC_CellInfo_SetSIB3_start |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation of f_NR5GC_CellInfo_SetSIB3_start, […].intraFreqBlackCellList[p_Index].start_ is initialized. According to ASN.1 definition: PCI-Range ::= SEQUENCE { start PhysCellId, range ENUMERATED {n4, n8, n12, n16, n24, n32, n48, n64, n84, n96, n128, n168, n252, n504, n1008,spare1} OPTIONAL -- Need S } However, the range IE is not initialized. This will cause later an error as range field, being an OPTIONAL field, is not initialized properly, test case will fail later as inside function f_NR_CellInfo_Set operation valueof will be called ending the test case due to a runtime error. […].intraFreqBlackCellList[p_Index].range must be initialized. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added initilization of OPTIONAL field […].intraFreqBlackCellList[p_Index].range. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case(s) may end with a runtime error. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.17.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required	4

1.1	Function f_NR5GC_CellInfo_SetSIB3_start	4

# Corrections required

## Function f_NR5GC_CellInfo_SetSIB3_start

| Function name | f_NR5GC_CellInfo_SetSIB3_start |
| --- | --- |
| Reason for change | range field, being an OPTIONAL field, is not initialized properly, test case will fail later as inside function f_NR_CellInfo_Set operation valueof will be called, ending the test case due to a runtime error. |
| Summary of change | Added initilization of OPTIONAL field […].intraFreqBlackCellList[p_Index].range. |
| TTCN module | Common/NR/NR_CellInfo.ttcn |
| MCC160 Comment |  |

Before Change:

<<SKIPPED CODE>> function f_NR5GC_CellInfo_SetSIB3_start(NR_CellId_Type p_NR_CellId, integer p_Index, PhysCellId p_Start ) runs on NR_BASE_PTC { var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); select (v_NR_CellInfo.Sysinfo.NR_SysinfoCombination) { case (NR_3, NR_5) { v_NR_CellInfo.Sysinfo.BcchInfo.SIs[1].message_.c1.systemInformation.criticalExtensions.systemInformation.sib_TypeAndInfo[0].sib3.intraFreqBlackCellList[p_Index].start_ := valueof(p_Start); f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); } case else { FatalError(__FILE__, __LINE__, "Invalid System Combination"); } } } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_NR5GC_CellInfo_SetSIB3_start(NR_CellId_Type p_NR_CellId, integer p_Index, PhysCellId p_Start ) runs on NR_BASE_PTC { var NR_CellInfo_Type v_NR_CellInfo := f_NR_CellInfo_Get(p_NR_CellId); select (v_NR_CellInfo.Sysinfo.NR_SysinfoCombination) { case (NR_3, NR_5) { v_NR_CellInfo.Sysinfo.BcchInfo.SIs[1].message_.c1.systemInformation.criticalExtensions.systemInformation.sib_TypeAndInfo[0].sib3.intraFreqBlackCellList[p_Index].start_ := valueof(p_Start); v_NR_CellInfo.Sysinfo.BcchInfo.SIs[1].message_.c1.systemInformation.criticalExtensions.systemInformation.sib_TypeAndInfo[0].sib3.intraFreqBlackCellList[p_Index].range := omit; f_NR_CellInfo_Set(p_NR_CellId, v_NR_CellInfo); } case else { FatalError(__FILE__, __LINE__, "Invalid System Combination"); } } } <<SKIPPED CODE>>