3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210259

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1476 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR RLC test case 7.1.2.3.8 in EN-DC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| s |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-12-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Current TTCN setting is expecting receive RACH preamble indicator before RRC Reconfiguration Complete message. This is not required for RLC testcases like 7.1.2.3.8. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Disable waiting for RACH preamble indicator before Reconfiguration Complete message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Testcase will fail a conformant UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.3.8.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	7

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_NR_ENDC_ReConfigAM_UM |
| --- | --- |
| Reason for change | Current TTCN setting is expecting receive RACH preamble indicator before RRC Reconfiguration Complete message. This is not required for RLC testcases like 7.1.2.3.8. |
| Summary of change | Disable waiting for RACH preamble indicator before Reconfiguration Complete message. |
| TTCN module | NR_CommonFunctions.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR_ENDC_ReConfigAM_UM(NR_CellId_Type p_NR_CellId, integer p_NoOfNR_AMBearersN, integer p_NoOfNR_UMBearersM, template (omit) NR_RadioBearerList_Type p_SS_Drb_ConfigList := omit, template (omit) NR_RLC_BearerToAddModList_Type p_RLC_BearerConfigList := omit, template (omit) SRB_ToAddModList p_SRB_ToAddModList := omit, template (omit) DRB_ToAddModList p_DRB_ToAddModList := omit, template (omit) SecurityConfig p_SecurityConfig := omit, template (omit) MAC_CellGroupConfig p_MAC_CellGroupConfig := omit, NR_RRC_STATE_Type p_RRC_State := RRC_IDLE, template (omit) PDSCH_ServingCellConfig.nrofHARQ_ProcessesForPDSCH p_NrofHARQ_ProcessesForPDSCH := omit, boolean p_ReconfigRLConly := false // @sic R5s201514, R5-205671 sic@ ) runs on NR_BASE_PTC // @sic R5s201010 sic@ { // @sic R5-203533: added p_NrofHARQ_ProcessesForPDSCH sic@ var DRB_Identity v_DrbId := tsc_NR_DRB2; // for ENDC only var template (value) CellGroupConfig v_SCGConfig; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var template (value) DRB_ToAddModList v_DRB_ToAddModList; var template (value) SecurityConfig v_SecurityConfig; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var template (value) RLC_Config v_RLC_Config; var EUTRA_NR_Coordination_MSG v_Coordination_MSG_Value; var integer i; var boolean v_IsFR1:= f_NR_CellInfo_GetIsFR1(p_NR_CellId); var template (omit) PDCP_Config.drb.statusReportRequired v_StatusReportRequired := true_; for (i:=0; i < p_NoOfNR_AMBearersN + p_NoOfNR_UMBearersM; i:=i+1) { if (i < p_NoOfNR_AMBearersN) { v_SS_Drb_ConfigList[i] := f_NR_GetRadioBearerConfig_FullAM(p_NR_CellId, v_DrbId); v_RLC_Config := cs_38508_RLC_Config_AM_DRB(f_NR_UL_AM_RLC(v_IsFR1), f_NR_DL_AM_RLC(v_IsFR1)); } else { v_SS_Drb_ConfigList[i] := f_NR_GetRadioBearerConfig_FullUM(p_NR_CellId, v_DrbId); //ENDC function: Sdap=omit v_RLC_Config := cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)); v_StatusReportRequired := omit; } v_RLC_BearerConfigList[i] := cs_38508_RLC_BearerConfig_DRB(v_DrbId, v_RLC_Config, cs_38508_LogicalChannelConfig_DRB); v_DRB_ToAddModList[i] := cs_38508_DRB_ToAddMod(v_DrbId, cs_38508_PDCP_Config (-, -, -, -, v_StatusReportRequired)); // @sic R5s200696, R5s200998 sic@ v_DrbId := v_DrbId + 1; } v_SecurityConfig := cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig (f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()), secondary); if (isvalue(p_SS_Drb_ConfigList)) { v_SS_Drb_ConfigList := valueof(p_SS_Drb_ConfigList); } if (isvalue(p_RLC_BearerConfigList)) { v_RLC_BearerConfigList := valueof(p_RLC_BearerConfigList); } if (isvalue(p_DRB_ToAddModList)) { v_DRB_ToAddModList := valueof(p_DRB_ToAddModList); } if (isvalue(p_SecurityConfig)) { v_SecurityConfig := valueof(p_SecurityConfig); } f_NR_SS_CommonRadioBearerConfig(p_NR_CellId, v_SS_Drb_ConfigList); v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(p_SRB_ToAddModList, v_DRB_ToAddModList, -, v_SecurityConfig); v_SCGConfig := f_NR_GetSCGConfigDef(p_NR_CellId); if (p_ReconfigRLConly == false) { if (isvalue(p_MAC_CellGroupConfig)) { v_SCGConfig.mac_CellGroupConfig := p_MAC_CellGroupConfig; f_NR_SS_ServingCellConfig(p_NR_CellId, cs_TimingInfo_Now, cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, p_MAC_CellGroupConfig, omit))); } if (isvalue(p_NrofHARQ_ProcessesForPDSCH)) { v_SCGConfig.spCellConfig.spCellConfigDedicated.pdsch_ServingCellConfig := cs_NR_Pdsch_ServingCellConfigSetup(cs_NR_PDSCH_ServingCellConfigDef(-, -, p_NrofHARQ_ProcessesForPDSCH)); f_NR_SS_CommonCellConfig(p_NR_CellId, cads_NR_PDSCH_Config_REQ(p_NR_CellId, cs_TimingInfo_Now, cs_NR_PDSCH_CellLevelConfigDef(v_SCGConfig.spCellConfig.spCellConfigDedicated.pdsch_ServingCellConfig.setup))); } } else { v_SCGConfig.mac_CellGroupConfig := omit; v_SCGConfig.physicalCellGroupConfig := omit; v_SCGConfig.spCellConfig := omit; } v_SCGConfig.rlc_BearerToAddModList := v_RLC_BearerConfigList; f_NR_SendRRCReconfigurationContentsToEUTRA(v_RadioBearerConfig, v_SCGConfig, -, -, false, reconfigurationWithSync_CheckRACH); //@sic R5s190541, R5s200647 sic@ // Get Security info from EUTRA, calculate latest keys and configure SS // as only need to do this for the first reconfiguration @sic R5s200647, R5s201010 sic@ if (p_RRC_State != RRC_CONNECTED) { f_ENDC_GetKeysActivateSecurity(p_NR_CellId, EUTRA); } //RRCReconfigurationComplete not yet received from E-UTRA - Wait for it @sic R5s200647 sic@ EUTRA.receive(cmr_EUTRA_NR_OctetData ("Complete")) -> value v_Coordination_MSG_Value; f_NR_CheckReceived_RRCReconfigurationComplete(valueof(v_Coordination_MSG_Value)); }

After Change:

function f_NR_ENDC_ReConfigAM_UM(NR_CellId_Type p_NR_CellId, integer p_NoOfNR_AMBearersN, integer p_NoOfNR_UMBearersM, template (omit) NR_RadioBearerList_Type p_SS_Drb_ConfigList := omit, template (omit) NR_RLC_BearerToAddModList_Type p_RLC_BearerConfigList := omit, template (omit) SRB_ToAddModList p_SRB_ToAddModList := omit, template (omit) DRB_ToAddModList p_DRB_ToAddModList := omit, template (omit) SecurityConfig p_SecurityConfig := omit, template (omit) MAC_CellGroupConfig p_MAC_CellGroupConfig := omit, NR_RRC_STATE_Type p_RRC_State := RRC_IDLE, template (omit) PDSCH_ServingCellConfig.nrofHARQ_ProcessesForPDSCH p_NrofHARQ_ProcessesForPDSCH := omit, boolean p_ReconfigRLConly := false, // @sic R5s201514, R5-205671 sic@ ReconfigurationWithSyncAvailable_Type p_WaitForRachPreamble := reconfigurationWithSync_CheckRACH ) runs on NR_BASE_PTC // @sic R5s201010 sic@ { // @sic R5-203533: added p_NrofHARQ_ProcessesForPDSCH sic@ var DRB_Identity v_DrbId := tsc_NR_DRB2; // for ENDC only var template (value) CellGroupConfig v_SCGConfig; var template (value) RadioBearerConfig v_RadioBearerConfig; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var template (value) DRB_ToAddModList v_DRB_ToAddModList; var template (value) SecurityConfig v_SecurityConfig; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var template (value) RLC_Config v_RLC_Config; var EUTRA_NR_Coordination_MSG v_Coordination_MSG_Value; var integer i; var boolean v_IsFR1:= f_NR_CellInfo_GetIsFR1(p_NR_CellId); var template (omit) PDCP_Config.drb.statusReportRequired v_StatusReportRequired := true_; for (i:=0; i < p_NoOfNR_AMBearersN + p_NoOfNR_UMBearersM; i:=i+1) { if (i < p_NoOfNR_AMBearersN) { v_SS_Drb_ConfigList[i] := f_NR_GetRadioBearerConfig_FullAM(p_NR_CellId, v_DrbId); v_RLC_Config := cs_38508_RLC_Config_AM_DRB(f_NR_UL_AM_RLC(v_IsFR1), f_NR_DL_AM_RLC(v_IsFR1)); } else { v_SS_Drb_ConfigList[i] := f_NR_GetRadioBearerConfig_FullUM(p_NR_CellId, v_DrbId); //ENDC function: Sdap=omit v_RLC_Config := cs_38508_RLC_Config_UM_DRB(f_NR_UL_UM_RLC(), f_NR_DL_UM_RLC(v_IsFR1)); v_StatusReportRequired := omit; } v_RLC_BearerConfigList[i] := cs_38508_RLC_BearerConfig_DRB(v_DrbId, v_RLC_Config, cs_38508_LogicalChannelConfig_DRB); v_DRB_ToAddModList[i] := cs_38508_DRB_ToAddMod(v_DrbId, cs_38508_PDCP_Config (-, -, -, -, v_StatusReportRequired)); // @sic R5s200696, R5s200998 sic@ v_DrbId := v_DrbId + 1; } v_SecurityConfig := cs_38508_SecurityConfig(cs_38508_SecurityAlgorithmConfig (f_NR_AS_CipheringAlgorithm_Get(), f_NR_AS_IntegrityAlgorithm_Get()), secondary); if (isvalue(p_SS_Drb_ConfigList)) { v_SS_Drb_ConfigList := valueof(p_SS_Drb_ConfigList); } if (isvalue(p_RLC_BearerConfigList)) { v_RLC_BearerConfigList := valueof(p_RLC_BearerConfigList); } if (isvalue(p_DRB_ToAddModList)) { v_DRB_ToAddModList := valueof(p_DRB_ToAddModList); } if (isvalue(p_SecurityConfig)) { v_SecurityConfig := valueof(p_SecurityConfig); } f_NR_SS_CommonRadioBearerConfig(p_NR_CellId, v_SS_Drb_ConfigList); v_RadioBearerConfig := cs_38508_RadioBearerConfigDef(p_SRB_ToAddModList, v_DRB_ToAddModList, -, v_SecurityConfig); v_SCGConfig := f_NR_GetSCGConfigDef(p_NR_CellId); if (p_ReconfigRLConly == false) { if (isvalue(p_MAC_CellGroupConfig)) { v_SCGConfig.mac_CellGroupConfig := p_MAC_CellGroupConfig; f_NR_SS_ServingCellConfig(p_NR_CellId, cs_TimingInfo_Now, cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(omit, omit, p_MAC_CellGroupConfig, omit))); } if (isvalue(p_NrofHARQ_ProcessesForPDSCH)) { v_SCGConfig.spCellConfig.spCellConfigDedicated.pdsch_ServingCellConfig := cs_NR_Pdsch_ServingCellConfigSetup(cs_NR_PDSCH_ServingCellConfigDef(-, -, p_NrofHARQ_ProcessesForPDSCH)); f_NR_SS_CommonCellConfig(p_NR_CellId, cads_NR_PDSCH_Config_REQ(p_NR_CellId, cs_TimingInfo_Now, cs_NR_PDSCH_CellLevelConfigDef(v_SCGConfig.spCellConfig.spCellConfigDedicated.pdsch_ServingCellConfig.setup))); } } else { v_SCGConfig.mac_CellGroupConfig := omit; v_SCGConfig.physicalCellGroupConfig := omit; v_SCGConfig.spCellConfig := omit; } v_SCGConfig.rlc_BearerToAddModList := v_RLC_BearerConfigList; f_NR_SendRRCReconfigurationContentsToEUTRA(v_RadioBearerConfig, v_SCGConfig, -, -, false, p_WaitForRachPreamble); //@sic R5s190541, R5s200647 sic@ // Get Security info from EUTRA, calculate latest keys and configure SS // as only need to do this for the first reconfiguration @sic R5s200647, R5s201010 sic@ if (p_RRC_State != RRC_CONNECTED) { f_ENDC_GetKeysActivateSecurity(p_NR_CellId, EUTRA); } //RRCReconfigurationComplete not yet received from E-UTRA - Wait for it @sic R5s200647 sic@ EUTRA.receive(cmr_EUTRA_NR_OctetData ("Complete")) -> value v_Coordination_MSG_Value; f_NR_CheckReceived_RRCReconfigurationComplete(valueof(v_Coordination_MSG_Value)); }

## Change 2

| Function name | f_NR_RLC_Reconfiguration |
| --- | --- |
| Reason for change | Current TTCN setting is expecting receive RACH preamble indicator before RRC Reconfiguration Complete message. This is not required for RLC testcases like 7.1.2.3.8. |
| Summary of change | Disable waiting for RACH preamble indicator before Reconfiguration Complete message. |
| TTCN module | RLC_TC_Common_NR.ttcn |
| MCC160 Comment |  |

Before Change:

function f_NR_RLC_Reconfiguration(NR_CellId_Type p_NR_CellId, integer p_NoOfNR_AMBearersN, integer p_NoOfNR_UMBearersM, template (value) NR_RLC_BearerToAddModList_Type p_RLC_BearerConfigList, template (omit) NR_RadioBearerList_Type p_SS_Drb_ConfigList := omit) runs on NR_BASE_PTC { if (f_GetTestcaseAttrib_ENDC(testcasename())) { //ENDC tc f_NR_ENDC_ReConfigAM_UM(p_NR_CellId, p_NoOfNR_AMBearersN, p_NoOfNR_UMBearersM, p_SS_Drb_ConfigList, p_RLC_BearerConfigList, -, -, -, -, RRC_CONNECTED, -, true); // @sic R5s190800, R5s201010, R5s201514, R5-205671 sic@ } else { f_NR5GC_ReConfigAM_UM(p_NR_CellId, p_RLC_BearerConfigList); } }

After Change:

function f_NR_RLC_Reconfiguration(NR_CellId_Type p_NR_CellId, integer p_NoOfNR_AMBearersN, integer p_NoOfNR_UMBearersM, template (value) NR_RLC_BearerToAddModList_Type p_RLC_BearerConfigList, template (omit) NR_RadioBearerList_Type p_SS_Drb_ConfigList := omit) runs on NR_BASE_PTC { if (f_GetTestcaseAttrib_ENDC(testcasename())) { //ENDC tc f_NR_ENDC_ReConfigAM_UM(p_NR_CellId, p_NoOfNR_AMBearersN, p_NoOfNR_UMBearersM, p_SS_Drb_ConfigList, p_RLC_BearerConfigList, -, -, -, -, RRC_CONNECTED, -, true, noRACH_Check); // @sic R5s190800, R5s201010, R5s201514, R5-205671 sic@ } else { f_NR5GC_ReConfigAM_UM(p_NR_CellId, p_RLC_BearerConfigList); } }