3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210262

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-4 | CR | 0135 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to a NR5GC UE Positioning related function |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2020-12-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Function f_POS_GetListOfSupportedGNSS does not include “modernizedGPS” as GNSS technology when GPS L5 band is supported. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include “modernizedGPS” as a PositioningSystemType if pc_A_GPS_L5 is set to TRUE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TC implementation may FAIL a conformant UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.4.x.15s.NR5GC, 9.3.1.2.15s.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1: Correction to f_POS_GetListOfSupportedGNSS	3

# Overview

This document lists all the changes needed to correct issues in the <ATS iwd-TTCN3-B2020-09_D20wk50> related to the title of this CR.

Contact:	Holger Jauch
	<holger.jauch@rohde-schwarz.com>

# Corrections required

## Change 1: Correction to f_POS_GetListOfSupportedGNSS

| Function name | f_POS_GetListOfSupportedGNSS |
| --- | --- |
| Reason for change | Function f_POS_GetListOfSupportedGNSS does not include “modernizedGPS” as GNSS technology when GPS L5 band is supported. |
| Summary of change | Include “modernizedGPS” as a PositioningSystemType if pc_A_GPS_L5 is set to TRUE. |
| TTCN module | Common\POS\PosSystem_Control.ttcn |
| MCC160 Comment |  |

Before

<<SKIPPED CODE>>

function f_POS_GetListOfSupportedGNSS() return PositioningSystemList_Type

{

var PositioningSystemList_Type v_GNSS_List;

var integer i := 0;

if (pc_A_GPS_L1C_A or pc_A_GPS_L1C_A_MGPS) {

v_GNSS_List[i] := gps;

i := i + 1;

}

if (pc_A_GLONASS) {

v_GNSS_List[i] := glonass;

i := i + 1;

}

if (pc_A_Galileo) {

v_GNSS_List[i] := galileo;

i := i + 1;

}

if (pc_A_BDS) {

v_GNSS_List[i] := bds;

i := i + 1;

}

//At this point, v_GNSS_List contains all the supported GNSS.

if (i == 0) {

FatalError (__FILE__, __LINE__, "No GNSS declared as supported in the PICS");

}

return v_GNSS_List;

}

<<SKIPPED CODE>>

After

<<SKIPPED CODE>>

function f_POS_GetListOfSupportedGNSS() return PositioningSystemList_Type

{

var PositioningSystemList_Type v_GNSS_List;

var integer i := 0;

if (pc_A_GPS_L1C_A or pc_A_GPS_L1C_A_MGPS) {

v_GNSS_List[i] := gps;

i := i + 1;

}

if (pc_A_GLONASS) {

v_GNSS_List[i] := glonass;

i := i + 1;

}

if (pc_A_Galileo) {

v_GNSS_List[i] := galileo;

i := i + 1;

}

if (pc_A_BDS) {

v_GNSS_List[i] := bds;

i := i + 1;

}

if (pc_A_GPS_L5) {

v_GNSS_List[i] := modernizedGPS;

i := i + 1;

}

//At this point, v_GNSS_List contains all the supported GNSS.

if (i == 0) {

FatalError (__FILE__, __LINE__, "No GNSS declared as supported in the PICS");

}

return v_GNSS_List;

}

<<SKIPPED CODE>>